3GPP TSG-SA WG2 Meeting #168	S2-2503176

# Goteborg, Sweden, April 07 – 11, 2025               	       (revision of S2-220xxxx)

Source: 	OPPO

Title: 	23.700-14: Key issue X: Sensing area authorization

Document for: 	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: The contribution proposes the key issue for WT#2 on Sensing area authorization.

1. Discussion

The WT-2 study addresses the need to dynamically authorize and revoke sensing services based on operator policies, regulatory constraints, and service consumer requirements. As Integrated Sensing and Communication (ISAC) supports diverse applications (e.g., public safety monitoring, commercial collision avoidance), robust mechanisms are required to validate service requests, enforce access control, and terminate sensing operations when policies are violated. A critical subset of this study focuses on ​service area authorization, which restricts sensing activities to predefined geographical zones to comply with regulations or commercial agreements.

Service Authorization and Revocation Background
Sensing services must dynamically adapt to contextual changes, such as:

- ​Regulatory compliance: Restricting sensing operations in protected areas (e.g., military zones, privacy-sensitive regions).

- ​Commercial constraints: Enabling paid services within specific areas (e.g., premium real-time mapping in industrial campuses).

- ​Dynamic revocation: Terminating sensing activities upon policy violations (e.g., unauthorized data collection) or emergency events.

Service Area Authorization Requirements
To fulfill SA1-defined use cases (e.g., intruder detection in restricted UAV zones), this Key Issue investigates:

- ​Policy acquisition and distribution:

- How to acquire and store allowed/forbidden area definitions from operator policy systems, third-party regulators or AFs.

- How to distribute service area information to core network functions (e.g., AMF, SMF) and sensing devices (transmitters/receivers) via standardized interfaces.

- ​Request validation against service areas:

- How to validate sensing requests by verifying target/sensor locations (implicitly via 3GPP positioning) against authorized areas.

- ​Enforcement and revocation mechanisms:

- How to enforce restrictions on transmitters (e.g., disabling RF signals in forbidden areas) and receivers (e.g., filtering data outside allowed zones).

NOTE: Security aspects will be coordinated with SA3.

Thus, it is proposed to agree the following changes to 3GPP TR 23.700-14.

2. Proposal

Start of Changes (all new text below)

## 5.1.x	Key Issue #x: Sensing area authorization

### 5.1.x.1	Description

Integrated sensing and communication (ISAC) requires dynamic control over sensing service areas to ensure compliance with regulatory policies and application-specific constraints. This Key Issue focuses on mechanisms to authorize sensing service requests based on pre-configured ​allowed service areas and ​forbidden areas, which define geographical zones where sensing operations are permitted or restricted.

This Key Issue aims to study:

- ​Whether and how to acquire and store service area information (allowed/forbidden areas) from operator or third-party for regulatory or commercial use cases.

- Whether and how to retrieve and distribute service area information (allowed/forbidden areas) to network entities (e.g., AMF, SF) and sensing devices (transmitters/receivers)

- ​Whether and how to validate sensing service requests for service area authorization.

- ​Whether and how to enforce service area restrictions on sensing transmitters/receivers (e.g., disabling signal transmission in forbidden areas, limiting receiver data collection).

NOTE: Security aspects will be coordinated with SA3

End of Changes