3GPP TSG-SA WG2 Meeting #168	S2-2503390

Goteborg, Sweden, April 07 – 11, 2025

Source:	Samsung

Title:	Reference Points

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: The contribution proposes reference point names for AIoT Architecture.

1. Discussion

Reference pont names need to be finalized in TS 23.369. LS from RAN3 (R3-250905) also needs to be considered for the protocol between AIOTF and AIoT RAN.

As the interface between AIOTF and AIoT RAN when there is direct interface, it needs to be differentiated with the case when there is indirect interface.

- 	When there is Indirect connectivity between AIOTF and AIoT RAN via an AMF, AIoT RAN is connected to the CN via AMF through N2 interface.

-	When there is Direct connectivity between AIOTF and AIoT RAN, AIoT RAN is connected to the CN via AIOTF through Nx interface.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 23.369.

* * * Start of 1st Change * * * *

4.3	Reference points

The AIoT Architecture contains the following reference points:

Ny:	Reference point between the AIoT Device and the AIOTF.

Nx:	Reference point between the AIoT RAN and the AIOTF.

The following reference points show the interactions that exist between the NF services in the NFs. These reference points are realized by corresponding NF service-based interfaces and by specifying the identified consumer and producer NF service as well as their interaction in order to realize a particular system procedure.

Na:	Reference point between the AIOTF and the AMF.

Nb:	Reference point between the AIOTF and the NEF.

Nc:	Reference point between the AIOTF and the NRF.

Nd:	Reference point between the AIOTF and the ADM.

Ne:	Reference point between the ADM and the UDR.

Nf:	Reference point between the ADM and the NEF.

In addition to the relevant reference points defined in TS 23.501 [3], in the case of AIoT, these reference points are as follows:

N2:	Reference point between the AIoT RAN and the AMF.

N33:	Reference point between NEF and AF.

* * * Start of 2nd Change * * * *

4.6.2.2	Protocol Stack between AF and AIoT Device for AIoT RAN Direct Path

Legend:

-	AIoT Reader Control: It is between the AIOTF and AIoT RAN.

Figure 4.6.2.2-1: Protocol Stack between AF and AIoT Device for Direct Path option

The AIoT NAS messages between the AIoT device and the AIOTF are transferred via the AIoT RAN transparently.

4.6.2.3	Protocol Stack between AF and AIoT Device for AIoT RAN Indirect Path

Figure 4.6.3.1-2: Protocol Stack between AF and AIoT Device for Indirect Path via AMF option

In this Protocol Stack, the AMF is additionally involved and its functionality is as defined in clause 4.5.7. The AIoT NAS messages between the AIoT device and the AIOTF are transferred via the AIoT RAN and AMF transparently.

AIoT Reader Control included in N2 AIOTF information container is transferred between AIOTF and AIoT RAN via AMF.

Editor's note:	The reference to AIoT AS Layer protocol will be updated based on RAN WG1 or RAN WG2 decision.

* * * End of Change * * * *