3GPP TSG-WG SA2 Meeting #168	S2-2503527

7th – 11th April 2025, Goteborg, Sweden	(revision of 25XXXXX)

Source:	Nokia

Title:	Updates on AF authorization

Document for:	Approval

Agenda Item: 	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: This paper proposes to update the AF authorization section in the TS 23.369 for Ambient IoT.

# Introduction

This paper is prepared for the normative work of Ambient IoT to clarify how to authorize the AF.

It is proposed that the NEF serve as the sole entity responsible for AF authorization.

# 2. Text Proposal

It is proposed to capture the following changes to 3GPP TS 23.369 for Ambient IoT.

# * * * * First change * * * *

## 5.6	AF authorization to the AIoT Services

The information needed to support the authorization of the AF for performing the AIoT service is stored as locally configured authorization data for 3rd party AFs in the NEF.

Table 5.6-1 below describes items stored as AF authorization data for the AIoT.

Table 5.6-1: AF Authorization Data for AIoT

| AF Authorization Data | Description |
| --- | --- |
| AF ID | Identifier used to identify the AF. |
| Allowed area | Indicate the allowed area for the indicated AF to perform the AIoT services operations. |
| Allowed service operations | Indicate the allowed service operation (s) for the indicated AF, e.g. inventory, read, write, permanent disable. |
| Allowed target AIoT devices | Indicate the allowed AIoT Device(s) for the indicated AF. |

The authorization to an untrusted AF for the AIoT mainly includes two parts:

-	NEF performs AIoT AF request authorization based on the service level agreement (SLA) between the 3rd party AF and the 5GS of the mobile network operator, the operator policy and local configuration as in TS 33.501 [8].

-	NEF may additionally perform authorization of AIoT service requested by the AF using the AF authorization data locally stored in the NEF, as described in Table 5.6-1 above.

# * * * * Second change * * * *

### 7.5.2	Nadm_DM_Query service operation

Service operation name: Nadm_DM_Query

Description: NF service consumer may request the AIoT device profile data or the AF authorization data from the ADM.

Inputs, Required: AIoT Device Permanent ID, or AF ID.

Input, Optional: None.

Outputs, Required: the AIoT device profile data.

Output, Optional: None.

# * * * * End of changes * * * *