3GPP TSG-SA2 Meeting #168	S2-2503732

# Goteborg, Sweden, April 7 - April 11, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6255 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Available Bitrate Reporting when the Serving RAN Node Changes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When the serving NG-RAN node changes, it can not be assumed that the target NG-RAN node supports Available bitrate Reporting and it can not be assumed that the target NG-RAN node can provide the same Available Bitrate that was last reported by the source NG-RAN. The clause on Available bitrate monitoring should reference the reporting procedure in clause 4.15.12 of TS 23.502. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added text to say that the SMF can notify the UPF when the serving NG-RAN node changes so that the UPF can report that the last reported Available bitrate is no longer valid. Added a reference to clause 4.15.12 of TS 23.502. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AS might assume an Available bitrate that can not be supported. There would be no reference to clause 4.15.12 of TS 23.502 so the reporting procedure would be unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.45.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.45.6	Available bitrate monitoring

The QoS Monitoring for available bitrate allows the measurement and reporting of the UL and/or DL available bitrate for a GBR QoS Flow without Alternative QoS Profiles at the NG-RAN. The available bitrate refers to the bitrate that NG-RAN is able to provide for a QoS Flow within a duration. The duration is based on RAN implementation.

NOTE 1:	How the NG-RAN measures the available bitrate is NG-RAN implementation specific.

NOTE 2:	The available bitrate can be between the GFBR and the MFBR. The available bitrate can also be below the GFBR for a QoS Flow with QoS Notification Control enabled without Alternative QoS Profiles.

For the reporting of the available bitrate, periodic reporting is not applied and only threshold-based reporting applies. Based on the PCC rule containing available bitrate as QoS monitoring parameter and Reporting threshold(s) per direction in the QoS Monitoring policy (as described in clause 6.1.3.27.1 of TS 23.503 [45]), the SMF configures the NG-RAN to measure and report the available bitrate by sending QoS monitoring request including the threshold(s). Based on the QoS monitoring request, if the bitrate that NG-RAN is able to provide matches or exceeds indicated Reporting threshold(s), the NG-RAN reports the UL and/or DL available bitrate information of the highest fulfilled threshold to the PSA UPF via the GTP-U header of UL packets.

NOTE 3:	The NG-RAN is expected to immediately report the UL and/or DL available bitrate when it matches or exceeds a lower threshold value and report the UL and/or DL available bitrate when it matches or exceeds a higher threshold value with some hysteresis.

The SMF may configure the PSA UPF to perform QoS monitoring for available bitrate as described in clause 5.8.2.18. For the reporting of the available bitrate via the PSA UPF, only event-based reporting applies in the PSA UPF. Reporting by the UPF is performed as described in clause 4.15.12 of TS 23.502 [3]. The Reporting threshold(s) in the PCC rule are not sent to UPF. In the UPF, the event for the reporting is the reception of available bitrate information from NG-RAN or reception of a notification from the SMF that the serving NG-RAN has changed. According to the QoS Monitoring request for the available bitrate from the SMF, the PSA UPF monitors and exposes the available bitrate information reported from NG-RAN to the target NF as instructed.

During handover, the QoS Monitoring configuration is provided to the Target NG-RAN per QoS Flow in the QoS Monitoring configuration. When the SMF notifies the UPF that the serving NG-RAN has changed, the UPF reports that the last reported available bitrate cannot be assumed to be valid.

# * * * * End of changes * * * *