3GPP TSG-WG SA2 Meeting #168	S2-2503833

Goteborg, SE, 7th April – 11th April, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	pCR for TS 23.369: Clarification on Assistance Information

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: Clause 5.4 is updated to solve the two Editor’s Notes which are related to assistance information provided to AIOT RAN node.

1. Introduction/Discussion

There are two Editor’s Notes in clause 5.4.

Editor's note:	Other assistance information may be added later if necessary.

Editor's note:	How the AIOTF determines the assistance information (e.g. approximate number of AIoT Devices, approximate D2R message size) without input from the AF is FFS.

The first Editor’s note can be deleted directly since currently there is no other necessary assistance information foreseen and if there are any other requirements from RAN WG(s) to provide more assistance information further, this clause can be updated directly. Therefore, there is no need to keep this Editor’s note.

The second Editor’s note can be solved by adding clarification.

The optional approximate number of AIoT Devices needs some further clarification. If the AIOTF knows that if it is targeting a single device, it is not approximate, it is absolutely 1 device. If the AF does not provide an approximate number of devices, then the AIOTF can generate an approximate number of AIoT devices.

If AF does not provide any information which can be used to derive approximate D2R message size, the network be configured to provide such approximate D2R message size. For example, for Read service operation, if AF request does not provide any memory bank address information or data size corresponding to the data to be read, the network can be configured with a default value so that the Read service operation can be performed accordingly and the approximate D2R message can be derived based on this configuration.

Therefore, the second and third bullet in clause 5.4 can be updated to clarify that the approximate number of AIoT device and D2R message size can be provided based on AF request or network configuration (if not provided by AF request).

Moreover, in previous meeting, some argued that the AIoT service type is not used for radio resource allocation. Therefore, in the general description, the usage of such assistance information is reworded to perform corresponding service operations or radio resource allocation.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.369.

# * * * * First change * * * *

5.4	Assistance information provided to AIoT RAN node

The AIOTF provides the assistance information to the AIoT RAN together with the service operation requests. The AIOTF determines the assistance information provided to the AIoT RAN based on the information received from the AF. The Assistance information is used by the AIoT RAN e.g. for performing corresponding service operation and radio resource allocation.

The following assistance information may be provided:

-	AIoT service type (e.g. Inventory, Command);

-	Indication of a single AIoT device or an approximate number of AIoT devices based on AF request or network configuration (if not provided by AF request);

-	Approximate D2R message size based on AF request or network configuration (if not provided by AF request).

# * * * * End of changes * * * *