3GPP TSG-WG SA2 Meeting #168	S2-2504194

07 - 11 April, 2025, Goteborg, Sweden	(revision of S2-2503290)

Source:	vivo(rapporteur), Samsung(rapporteur)

Title:	New Key Issue for transfer of data over UP for UE data collection

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This paper proposed to include a new key issue for transfer of data over UP for UE data collection.

1. Introduction/Discussion

Agreed R20 FS_AIML_CN_Ph2 SID (SP-250413) contains the following.

WT#1: Study whether and how to support the standardized transfer of standardized data over UP for UE data collection to meet requirements for AI/ML for NR air interface operation with UE-side model training. Note 1: The WT#1 scope is based on UP Data Collection for Option2 documented in RAN LSs RP-243316 and RP-242389. Note 2: The requirements are elaborated in LSs RP-242389 and R2-2411152 which will be considered during the study phase. Note 3: WT#1 impact on user consent and privacy will be addressed in cooperation with SA3.

The following table are the Analysis of different data collection options for UE-side model training from RP-242389, which introduced the requirements for option 2 including that the first termination entity should inside the CN, the AI/ML-specific Data Transfer Path should be UE-> CN -> Server, etc.

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel (Note 7) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (Note 7) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer (Note 1) | No AI/ML specific controllability | FFS: level of controllability (Note 5) | Full controllability | Full controllability |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure or FFS other procedures | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS on level of visibility (Note 5) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) |
| Impacted WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, SA2 |
| - Note 1: Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer. - Note 2: Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA. - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content (e.g. UE proprietary information can be included transparently together with the standardized data message). - No standardized visibility (e.g. only UE proprietary information can be included transparently). - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: RAN2 cannot reach consensus on the level of MNO controllability and visibility possible via solution 1b without input from SA groups. - Note 6: RAN2 has not concluded on the need for partial and no visibility options. - Note 7: RAN2 could not reach consensus on the feasibility of UP solution in options 2 and 3. |  |  |  |  |

In this study, it is proposed to include a new key issue for transfer of data over UP for UE data collection.

2. Text Proposal

It is proposed to agree the following key issue(s) into TR 23.700-04.

# * * * * First change * * * *(All new text)

X.Y	Key Issue #X – Transfer of data over UP for UE data collection

X.Y.1	Description

This key issue aims to provide solutions for UE data collection to meet the requirements for AI/ML for NR air interface with UE-side model training. This process requires the transfer of training data from the UE to the 5G core and then to the OTT server.

This key issue will investigate the following aspects:

- How to support the standardized transfer of standardized data over UP for UE data collection including how to initiate, terminate and manage UE data transfer over the UP connection.

- How to support in 5GC the full visibility of standardized UE data contents.

- How 5GC can support the full controllability of standardized UE data transfer.

- How to differentiate the traffic for UE data collection from the UE regular traffic in the 5GC.

- Whether and how to inform the UE about what the standardized data to be transferred.

# * * * * End of changes * * * *