3GPP TSG-WG SA2 Meeting #168	S2-2504280

Goteborg, Sweden, April 7 - 11, 2025	(revision of S2-2504059, S2-2503196)

Source:	Ericsson, Huawei, HiSilicon

Title:	Clause 5.6 & 6.2.2 & 6.2.3: Resolve EN on AF Authorization

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC/ Rel-19

Abstract: This pCR updates AF authorization and resolve the relevant EN.

1. Introduction

Regarding AF authorization for the AIoT services, there two ENs about AF authorization about NEF and AIOTF left. One EN in clause 5.6:

Editor's note:	Whether to perform AF authorization only in NEF or in both NEF and AIOTF (including which parameter to use in Table 5.6-1), and whether a trusted AF requires authorization is FFS.

Another EN appears twice in clause 6.2.2 and clause 6.2.3:

Editor's note:	The authorisation split between the NEF and AIOTF needs alignment with the other clauses in this TS.

AFs in trusted domain

For AFs in trusted domain, there was a discussion on whether AF authorization for the AIoT services is needed or not in SA2#167.

In our view, AFs in trusted domain does not need the AF authorization which NEF performs. Those AFs can be trusted to interact with the NFs in 5GC. However, it does not mean they have the full authority to take any actions. For example, AIoT devices belongs to AF1 should not be allowed to be accessed by AF2, unless there are some agreements in place already. Another example, many AFs can be allowed to perform inventory and read operation, however, there should be limited AFs can perform write and disable operation.

[Proposal-1] AFs in trusted domain needs to be authorized to the AIoT services.

Regarding about how AF authorization for AIoT services can be performed, there are also some discussions on how much can be utilized in the SBI framework (the authorization framework specified in TS 33.501).

It is specified in clause 13.1.4.0 of TS 33.501: “The basic extent provided by the authorization token is at service level (i.e. the "scope" claim includes allowed services per NF type). Depending on the NF Service Producer configuration, higher level of granularity for the authorization token can be defined adding "additional scope" information within the token e.g. to authorize specific service operations and/or resources/data sets within service operations per NF Service Consumer type.”

[Observation-1] Utilizing the authorization framework, service level authorization can be performed natively

[Observation-2] Higher level of granularity of the authorization can be performed on service operation level and resources/date sets level.

Regarding AF authorization for AIoT services, checks need to be performed on whether the service operation can be performed, whether the targeting area is allowed, and whether the AIoT devices are allowed.

For the targeting area as well as the targeting devices, the authorization framework will not be helpful. For the service operation, the “addition scope” in authorization framework cannot be utilized as well, unless the read, write, disable are exposed as separate service operations, instead of including into one “command” service operation.

NRF is optional in some deployments where the expected NF and NF service information is locally configured on the requester NF. For such deployments, AIOTF needs to implement the AF authorization for AIoT services described above.

[Proposal-2] AIOTF needs to implement AF authorization for AIoT services for AFs in trusted domain.

In SA2#167, it was agreed that ADM can be utilized to store AF authorization data for AIoT. The utilizing of ADM is beneficial from data sharing point of view for multiple AIOTFs. However, considering the size of the AF authorization data for AIoT, it is much smaller than the AIoT device profile data which is also stored in the ADM. Especially in some localized deployment, the number of authorized AFs will be very limited. It can also be an alternative option to allow the AF authorization data for AIoT to be configured locally in the AIOTF.

[Proposal-3] Besides the ADM, it is proposed to allow the AF authorization data for AIoT to be configured locally in the AIOTF.

AFs outside trusted domain

There were some discussions on whether the NEF can perform the AF authorization for AIoT services, as the NEF is naturally an entity for AF authorization for AFs outside trusted domain.

In clause 5.9.2.3 of TS 33.501, it specifies: “The NEF shall be able to determine whether the Application Function is authorized to interact with the relevant Network Functions.”.

In clause 12.4 of TS 33.501, it specifies: “After the authentication, NEF determines whether the AF is authorized to send requests for the 3GPP Network Entity. The NEF shall authorize the requests from AF using OAuth-based authorization mechanism, the specific authorization mechanisms shall follow the provisions given in RFC 6749 [43].”

The AF authorization is to check whether AF is allowed to send requests to the relevant NFs.

[Observation-3] The NEF performs AF authorization on whether the AF is allowed to interact with the relevant Network Functions.

[Observation-4] AF authorization for the AIoT services is different from the AF authorization specified in TS 33.501.

NEF can perform AF authorization for the AIoT services based on the data configured in the NEF locally. It can stop the unauthorized AF request at the first place. However, considering the AIOTF needs to implement the function for AF in trusted domain. There are no needs to overload and complicate the NEF to implement the function again. NEF should perform the AF authorization as the first check to ensure the authorized AF to send request to the AIOTF. For the detailed check like the target devices, the work can be left to the AIOTF.

[Proposal-4] For AFs outside trusted domain, it is proposed to let NEF perform AF authorization as specified in TS 33.501, and AIOTF perform AF authorization for AIoT services as it does for AFs inside trusted domain.

[Proposal-5] Resolve the ENs and update based on the proposals above.

S2-2504059: Remove the procedure clauses which overlap with other contributions.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 23.369 v0.2.0:

* * * Start of Change * * * *

## 5.6	AF authorization for the AIoT Services

The information needed to support the authorization of the AF for performing the AIoT service is stored as the authorization data for 3rd party AF in the ADM, or locally configured in the AIOTF.

Table 5.6-1 below describes items stored as AF authorization data for the AIoT.

Table 5.6-1: AF Authorization Data for AIoT

| AF Authorization Data | Description |
| --- | --- |
| AF ID | Identifier used to identify the AF. |
| Allowed area | Indicate the allowed area for the indicated AF to perform the AIoT services operations. |
| Allowed service operations | Indicate the allowed service operation (s) for the indicated AF, e.g. inventory, read, write, permanent disable. |
| Allowed target AIoT devices | Indicate the allowed AIoT Device(s) for the indicated AF. |

The authorization of the AF for the AIoT includes two parts:

-	NEF performs AIoT AF request authorization based on the service level agreement (SLA) between the 3rd party AF and the 5GS of the mobile network operator, the operator policy and local configuration as in TS 33.501 [8].

-	AIOTF may perform authorization of AIoT service requested by the AF, using the AF authorization data retrieved from the ADM or configured locally as described in above Table 5.6-1. When ADM is used, the AIOTF also subscribes to changes of AF authorization data in the ADM for synchronization.

* * * End of Change * * * *