3GPP TSG-WG SA2 Meeting #168	S2-2504295

Goteborg, SE, 7th April – 11th April, 2025	(revision of S2-2504279, 4058, 3841)

Source:	Huawei, HiSilicon, Samsung

Title:	pCR for TS 23.369: Improve AIoT Device Profile Management

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: the paper address open issues for AIoT Device Profile Management, and add the support of permanent disable.

1. Introduction/Discussion

1.1	Issues related to SA3

Editor's note:	Whether ADM support ID security mechanism pending on SA WG3 decision.

Editor’s note:	SA WG2 will align security related materials with SA WG3 decision later.

SA3 concludes in TR33.713 that

-	1) AIOTF acts as network layer security termination point.

-	2) The credential (including device ID and authentication credential) for AIoT device authentication can be owned by an operator or by a third party.

-	3) The command request and response messages shall be confidentiality, integrity and replay protected based on the pre-shared key and freshness parameter or session keys derived from them.

Details of the authentication procedure are to be resolved during SA3 normative phase. Where to store the AIoT device credentials and related policy information will be based on SA2 and SA3 coordination.

Observation 1: AIoT authentication is supported as concluded by SA3, the entity to store the credentials and the role of ADM in AIOT authentication needs further to be resolved during normative phase

When the credential (including device ID and authentication credential) is owned by a third party, the device profile data (e.g., Device ID or credentials) is stored external to the network. This is documented as part of the AIoT Device Profile Management in clause 5.5 of TS 23.xyz. When the credential (including device ID and authentication credential) is owned by the operator, the authentication credentials of an AIoT Device shall be configured and security stored in the network side, and are seldomly refreshed during its lifecycle:

-	AIOTF is dynamically selected as the serving NF for a specific AIOT device during one service request, thus the AIOTF is not suitable to be used as the entity to store the authentication credentials of an AIoT Device.

-	Whereas, ADM that manages the profile data of an AIoT Device can be further used to store the authentication credentials of such AIoT Device. However, AIoT Device profile data can be exchanged among NFs (ADM, AIOTF, UDR), but the authentication credentials shall not expose outside the ADM.

SA3 WID scope objectives in SP-250359 contains that 1) Credentials are securely stored in the ADM on the network side

2) Security aspects of the storage of the credentials at the ADM. Thus, credentials in SA3 are now concluded to use ADM. However, like UE subscription data, no credentials are mentioned, similarly no need to explicitly mentioned the credential in device profile data.

Proposal 1: AIoT Device authentication credentials are securely stored in ADM. Details of the authentication procedure will be defined by SA3. Related ENs are removed.

1.2	Support of Permanent Disable operation

Permanent Disable operation is supported as one AIoT Service, this is to permanently disable the capability of the AIoT device(s) to transmit/receive RF signals.

If an AIoT device in service has been inventoried, the network considers this AIoT device authorized to use the network resources when its device profile data is retrieved.

If an AIoT device is assumed to be permanently disabled, as requested by the 3rd party AF, then the network is no longer required to store the device profile data for such AIoT device.

-	if the network can acknowledge the successful disabling of an AIoT device, the network can purge the profile data for such AIoT device at a later time or immediately.

-	if the network knows an AIoT device is to be permanently disabled, the network can trigger the "permanent disable" Command towards such AIoT device if this AIoT device is inventoried immediately in current permanent disable service request or is inventoried in future service requests (this case may occur if the AIoT device is not inventoried or is not disabled successfully during the permanent disable service request).

Observation 2: The network shall prevent an AIoT device that is assumed to be permanently disabled from becoming a target.

Question: If the network can acknowledge the successful disabling of an AIoT device, how to purge the profile data for such AIoT device in ADM? And if the network knows an AIoT device is to be permanently disabled, how to record the status?

Method A: Set the device status in AIoT device profile at ADM.

-	When AIOTF receives the permanent disable request from AF, it sets the device status to "to be disabled" in AIoT device profile data at ADM.

-	When AIOTF confirms the successful disabling of an AIoT device, the sets the device status to "disabled" in AIoT device profile data at ADM. The ADM, based on the "disabled" device status, deletes the corresponding profile data in ADM any time later based on operator implementation.

-	If the device status is "to be disabled", the AIOTF knows the device status from device profile data and triggers the "permanent disable" Command towards such AIoT device anytime the AIoT device is inventoried until the AIOT device is successful disabled.

Method B: Set the device status in AIoT device context at AIOTF + specify new ADM "Purge" AIoT service operation.

-	When AIOTF receives the permanent disable request from AF, it sets the device status to "to be disabled" in AIoT device context.

-	When AIOTF confirms the successful disabling of an AIoT device, the AIOTF immediately deletes the AIOT device context and invokes the new ADM "Purge" AIoT service operation to delete the corresponding profile data in ADM.

-	If the device status is "to be disabled", the AIOTF triggers the "permanent disable" Command towards such AIoT device anytime the AIoT device is inventoried until the AIOT device is successful disabled.

Method B needs a new ADM "Purge" AIoT service operation. Method B has problems to support device movement case and is limited to the lifecycle of AIoT device context.

Method C: Set the device status in the AF + specify new AIoT data exposure service.

-	When an AF sends the permanent disable request, it sets the device status to "to be disabled" in the AF.

-	When the AF confirms the successful disabling of an AIoT device, the sets the device status to "disabled" in the AF. The AF invokes new AIoT data exposure service (e.g., profile data delete) to purge the profile data in ADM.

-	If the device status is "to be disabled", the AF triggers the permanent disable request anytime the AIoT device is inventoried by the AF until the AIOT device is successful disabled.

Method C needs a new AIoT data exposure service. Method C will not purge the profile data in ADM timely once the AIoT device is disabled successfully, and it cannot stop an AIoT device that is assumed to be permanently disabled by an AF from becoming a target for another AF.

Among the three methods, method A introduces minimum enhancements and avoids the issues of method B&C. What is more, method A is future proof, e.g., temporary disable.

Proposal: Add "device status" in AIoT device profile data to complete the support of Permanent Disable operation.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.369.

# * * * * First change * * * *

5.5	AIoT Device Profile Management

The ADM may hold operator’s subscription data for the AIoT Device used in the network. If the AIoT Device is managed by the network, then the profile data for an AIoT Device is required in the network, otherwise the corresponding profile data (e.g. AIoT Device Permanent ID or credentials) is stored external to the network.

The AIoT Device Permanent ID is used by the AIOTF together with local configuration, 3rd party related context to locate the entity which stores the profile data of an AIoT Device.

In case the AIoT Device is managed by the network, the AIOTF checks whether the AIoT Device Permanent ID from AIoT Device has the profile data in the network and retrieves the profile data. The profile data for AIoT Device is different with UE subscription data as defined in TS 23.502 [4] clause 5.2.3, it is stored in the ADM network entity that exclusively supports management of AIoT Device’s profile data. The AIoT Device Permanent ID is the primary key for AIoT device profile data in the ADM.

Editor's note:	It is FFS how to stop AF request towards an AIoT device that has been permanently disabled.

The table 5.5-1 below describes information storage structures for AIoT device profile data.

Table 5.5-1: AIoT Device Profile Data

| Field | Description |
| --- | --- |
| AIoT Device Permanent ID | Uniquely identifies the AIoT Device. |
| Last known AIOTF information | Indicate the last known AIOTF that serves the AIoT device, or unknown |

NOTE:	Security materials and security mechanism involving ADM are specified in TS 33.369 [x].

# * * * * End of changes * * * *