SA WG2 Meeting #169	S2-2504499

19 - 23 May, 2025, Fukuoka, Japan

SA WG2 Meeting ##168	S2-2502811

07 - 11 April, 2025, Goteborg, Sweden

3GPP TSG-CT WG4 Meeting #127	C4-250554

# Athens, GREECE; 17th – 21st February 2025

# Title:	LS on Ethernet MA PDU session using MPQUIC-E steering

# Release:	Rel-19

# Work Item:	MASSS

Source:	CT4

To:	SA2

Cc:	CT1

Contact Person:

Name:	Bruno Landais

Tel. Number:

E-mail Address:	bruno.landais@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

Clause 5.32.6.2.2 of TS 23.501 specifies:

The MPQUIC-E functionality may be enabled when the type of the MA PDU Session is Ethernet. In case MPQUIC-E functionality is enabled for an Ethernet type PDU Session, the SMF indicates PDU Session type as IP in the N2 information to NG-RAN, as described in TS 23.502 [3].

Similar requirements have also been specified in CR 23.502 #5115 (agreed at SA2#166 Ad Hoc-e):

In case the PDU Session Type is “Ethernet”, then:

-	If the Steering functionality parameter in the MA PDU Session control information of the received PCC rule(s) is set only to MPQUIC-E, then SMF provides the PDU Session type as “IP” in the N2 SM information to NG-RAN,

NOTE Y:	When only MPQUIC-E functionality is enabled for an MA PDU Session, the packets exchanged between UE and UPF are IP packets of the "MPQUIC link-specific multipath" addresses/prefixes (i.e. the (R)AN transports IP packets between UE and UPF). PDU Session type IP is provided to the (R)AN so that the NG-RAN can apply ROHC.

-	If the Steering functionality parameter in the MA PDU Session control information of the received PCC rule(s) is set only to ATSSS-LL, then SMF behaviour is as per the clause 4.3.2.2.1 (i.e. PDU Session type “Ethernet” is provided in the N2 SM information).

NOTE Z:	Either ATSSS-LL or MPQUIC-E functionality is enabled for the same Ethernet MA PDU Session.

CT4 observes that SA2 has not defined any specific requirement about the handling of a MA PDU session using MPQUIC-E with a 3GPP access leg in EPS, e.g. upon the establishment of a 3GPP access leg in EPS (when a non-3GPP access leg has been established in 5GC) or upon a handover of the 3GPP access leg of a MA PDU session from 5GS to EPS. Note that TS 36.413 defines an Ethernet Type IE to enable the eNB to perform header compression appropriately.

Q1: Can SA2 clarify the handling of MA PDU sessions using MPQUIC-E with a 3GPP access leg in EPS with respect to the PDN connection type to be signalled to the eNB?

Besides, HR PDU sessions and PDU sessions with an I-SMF can support ATSSS. Per existing specifications, the V/I-SMF transfers ATSSS rules from the anchor SMF to the UE transparently (see ATSSS container in Table 6.1.6.4.4-2 of TS 29.502). Therefore, to support the quoted requirements for a HR PDU session or a PDU session with I-SMF, CT4 assumes that the (H)SMF shall newly signal to the V/I-SMF that the Ethernet PDU session uses an IPv4 or IPv6 transport (when using MPQUIC-E) and that using this new IE, the V/I-SMF shall signal the PDU session type as "IPv4" or "IPv6" in N2 signaling to the NG-RAN.

Q2: Can SA2 confirm CT4 assumption regarding the handling of MA PDU sessions using MPQUIC-E for HR PDU sessions and PDU sessions with an I-SMF.

2. Actions:

To SA2 group.

ACTION: 	CT4 kindly asks SA2 group to provide the requested clarifications and to consider updating stage 2 accordingly.

3. Date of Next CT4 Meetings:

CT4 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-C4.htm