SA WG2 Meeting ##169	S2-2504505

19 - 23 May, 2025, Fukuoka, Japan

3GPP TSG CT WG3 Meeting #140	C3-251653

Wuhan, China, 7th – 11th April 2025

# Title:	LS on AIML_CN issues

# Response to:	-

# Release:	Rel-19

# Work Item:	AIML_CN

Source:	CT3

To:	SA2

Cc:	CT4

Contact Person:

#### Name:	Apostolos Papageorgiou

#### E-mail Address:	apostolos.papageorgiou@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

CT3 noticed that the SCP, the NRF, and the UDM, which are analytics consumers of the Signalling Storm analytics according to 3GPP TS 23.288, do not appear in Table 8.1-1 and Table 9.1-1 as NF service consumers of the DCCF and the MFAF.

Question 1:

Can Signalling Storm analytics be subscribed (by the SCP, the NRF, the UDM) via the DCCF (potentially using the MFAF)?

Further, CT3 noticed that the DCCF is also not explicitly mentioned in 3GPP TS 23.273, which defines the Nlmf_DataExposure service. CT3 also noticed that 3GPP TS 23.288 clause 5A.1 says that "Data Collection Coordination and Delivery is applicable to NWDAFs that request data from a Data Source (e.g. for use in computing analytics)".

Question 2:

Can the DCCF be used to collect data using the Nlmf_DataExposure service?

Finally, CT3 noticed that information about the "used QoS Profile" are needed from the SMF in order to compute QoS and Policy Assistance analytics, according to 3GPP TS 23.288 Table 6.23.2-1. However, no further details about the exposed QoS Profile contents are provided, while the QoS Profile is not specified to be exposed by any event in the definition of the Nsmf_EventExposure service in 3GPP TS 23.502 clause 5.2.8.3.1.

Question 3:

Can information about the used QoS Profile be exposed by the QFI allocation, change, and deallocation events of the Nsmf_EventExposure service?

Question 4:

If the answer to Question 3 is "yes", can SA2 clarify which QoS Profile parameters are relevant to be exposed for QoS and Policy Assistance analytics?

2. Actions:

To SA2:

ACTION:

CT3 kindly asks SA2 to answer the questions and adjust its specifications accordingly, if needed.

3. Dates of Next CT3 Meetings:

CT3#141				May 19-23, 2025 				Bratislava, SK

CT3#142				August 25-29, 2025 				Gothenburg, Sweden