3GPP TSG-SA WG2#169	S2-2504593

Fukuoka, Japan, 19-23 May 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated

Title:	New Solution for KI#1: UP based UE data collection solution for AI/ML NR air interface operation

Document for:	Discussion/Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2

Abstract of the contribution: This contribution proposes the solution for KI#1 to support UE data Collection for option 2.

# 1.	Discussion

This contribution proposed a new solution based on newly defined entity UDCF in 5GC to support UP based UE data collection for AIML UE side model training.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-04:

# >>>>BEGINNING OF CHANGES (all new text) <<<<

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues and Use Cases

Table 6.0-1: Mapping of Solutions to Key Issues and use cases

|  | Key Issues |  | Use Cases (optional) |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Use Case #1> | <Use Case #2> |
| #x | x |  |  |  |

# >>>>Next Change (all new text) <<<<

## 6.X	Solution #X: Hybrid solution using UDCF to support UP based UE data collection for NR AIML air interface operation

### 6.X.1	High level principles

The following principles for this solution are proposed as below:

- New UDCF is proposed as a UP path termination entity in 5GC for “raw” UE data transfer

- When the 5GC-centric variant is used:

-	AF requests the data for specific UE from DCCF

-	DCCF checks if the requested UE collected data are already stored in ADRF or not. If the requested UE collected data are available in ADRF, DCCF exposes the data from ADRF to AF. If the requested UE collected data are not available in ADRF, DCCF sends data for the specific UE from UDCF.

-	UDCF informs the UDCF address to UE via NAS signalling. UE establishes the UP connection via UPF to UDCF.

-	UDCF sends data collection request to UE via application layer signaling, i.e. HTTP signalling. The security about the application layer signalling between UE and UDCF will be discussed in SA3.

-	UE collects data according to RAN1/RAN2’s decisions and sends the report to UDCF.

- 	UDCF sends the received “raw” data to DCCF, DCCF further sends to AF. DCCF may also store the collected UE collected data in ADRF.

- When the OAM-centric variant is used:

-	Details between the UDCF and OAM will be defined by SA5.

### 6.X.2	Description

This solution addresses KI#1.

Based on the LS in RP-24331, SA2 is required to study transfer of data over UP for option 2 and study if the solution fulfil the visibility and controllability requirements from RAN2.

In order to do that, a new NF named UDCF (UE Data Collection Function) is proposed as a new 5GC entity with the following functionality:

- The UP path termination entity in 5GC for UE data transfer from UE

- Receiving the UE data collection request from DCCF

- Requests data collection to UE

- Receiving “raw” data reporting from UE

- Expose UE “raw” data to DCCF or expose the UE “raw” data to OAM depending on the MNO choice for its AIML data management and processing architecture.

The interfaces for UDCF are described in Figure 6.x.2-1.

Figure 6.x.2-1: New Interfaces defined for UDCF

AF is the UE server for UE-side model training.

The same UP procedure about UE data collection between UE and UDCF can be used for both architecture option 2 or option 3. The reference of architecture Options 2 and 3 is referred to the data collection options described in RP-242389.

In Option 2, SA2 will further study the exposure procedure from UDCF to DCCF using new interface Nu1. The data exposure from DCCF to AF with NEF will reuse the existing procedures defined in TS 23.288. In option 3, the UDCF can expose the collected data to OAM, using Nu2 and SA5 can further study the rest of the functionality for an “OAM centric data collection architecture” and specifically how to expose the UE data to the UE server for UE-side model training.

In both cases it is proposed that the same UDCF NF can be used for both option 2 and option 3 as described in Figure 6.x.1-2.

Figure 6.x.2-2: UDCF based architecture to support option 2 and option 3

### 6.X.3	Procedures

#### 6.X.3.1	General call flow about data collection for the 5GC-centric variant and OAM-centric variant

Figure 6.x.3.1-1 describes the procedure about AF subscribes UE collected data via UDCF for the 5GC-centric variant:

Figure 6.x.3.1-1	AF subscribes UE collected data from UDCF (5GC-centric variant)

1. AF sends the Nnef_EventExposure_Subscribe request message to request the UE collected data. This message may include the following information:

- event ID, define a new event ID for different use cases that supported by RAN1, in this release the event ID may indicate beam management.

-	 Target of reporting (GPSI(s), or any UE), if the target of reporting indicates any UE, the AF may further provide the maximum number of UE,

-	location information

-	reporting info, e.g., reporting time period, number of reporting, reporting time window, reporting event condition .

2.	NEF discovers the DCCF from NRF for UE data collection request based on the following info:

-	the event ID: e.g., beam management

-	location information

If AF provides GPSI for specific UE, NEF maps the GPSI to SUPI and discovers the DCCF for the specific UE.

If AF provides any UE, NEF should discover the UE list from AMF based on S-NSSAI location info that provided by AF in step 1.

NEF discovers DCCF by using the requested UE collected data, location information and S-NSSAI.

3.	NEF sends Ndccf_DataManagement_Subscribe to the selected DCCF, the subscribe message includes the event ID, Target of reporting (GPSI(s), or any UE), location information and reporting info that received from AF.

If AF provides any UE, DCCF should further determine the UE list from AMF with the location info.

4.	For each UE, DCCF checks user consent about the UE collected data collection.

5.	If user consent is granted, DCCF determines if any available parameters that stored in ADRF, if yes, DCCF will retrieve data from ADRF and report to AF via NEF. The following steps are skipped.

If there is no available data stored in ADRF, DCCF will discover an UDCF for each for UE collected data.

The UDCF was discovered from NRF.

UDCF registers the supported event ID, e.g. beam management, S-NSSAI, DNN and location information as NF profile in NRF. DCCF will discover the UDCF from NRF for each UE with these information.

6.	DCCF sends the requested Nudcf_DataManagement_Subscribe signalling to UDCF for each UE, the signalling includesevent ID, e.g. beam management, UE ID, reporting info in the message..

7.	UDCF invokes the Namf_Communication_N1N2Transfer message to include the UDCF UP information to AMF. The UDCF UP information may be the UDCF IP address, or FQDN, or the connection capability. If the connection capability is used, a new connection capability set as “AIML” is required to be defined for this feature.

8.	AMF sends the NAS message to UE which includes the UDCF UP information.

9.	UE establishes the UP connection to UDCF by using the UDCF UP information and the URSP rule that received from PCF.

10.	UE sends the NAS response to AMF.

11.	AMF sends response to UDCF.

12.	UDCF sends the data request to UE, UE performs data collection as defined in RAN1 and RAN2, UE reports the collected data to UDCF if the reporting condition is fullfilled. The data collection and reporting between UE and UDCF is via a UP based signalling, it is proposed to use HTTP protocol in this release. The details of data collection and reporting procedure between UDCF and UE are described in clause 6.x.3.3.. The security about the interaction between UE and UDCF will be determined in SA3.

13.	UDCF reports the UE data received in step 12 to DCCF.

14.	DCCF reports data to NEF. DCCF may further store the UE data in ADRF as described in TS 23.288.

15. NEF forwards the data to AF.

Figure 6.x.2.1-2 describes the procedure about AF subscribes UE collected data via UDCF for the OAM-centric variant:

Figure 6.x.3.1-2	AF subscribes UE collected data from UDCF (OAM-centric variant)

- AF request the UE collected data to OAM. Details of the procedure will be defined by SA5.

2.	OAM sends the requested UE collected data, UE ID, reporting time info to the UDCF for each UE.

3- 8.	Same steps as steps 7-12 in Figure 6.x.3.1-1.

9.	UDCF reports the UE data received from UE in step 8 to OAM.

10. OAM reports data to AF. Details of the procedure will be defined by SA5.

#### 6.x.3.2		Traffic differentiation

This Traffic differentiation mechanism that proposed in this clause can be applied to both 5GC-centric variant and OAM-centric variant.

The traffic for UE data collection can be differentiate with UE regular traffic in either PDU session level or QoS flow level.

a) PDU session level

The URSP rule is provided by PCF to UE, in order to support the UE data collection, the operator may provide the UDCF UP information for UE data collection in traffic descriptors as described in table 6.x.3.2-1.

The structure of URSP rule is defined in Table 6.6.2.1-2 in TS 23.503. The parameters of traffic descriptor in table 6.x.3.2-1 is excerpted from Table 6.6.2.1-2 in TS 23.503, the non-relevant parameters are not shown here.

Table 6.x.3.2-1	Traffic descriptors for UE data collection

| Traffic descriptor | This part defines the Traffic descriptor components for the URSP rule. | Mandatory (NOTE 3) |
| --- | --- | --- |
| Application descriptors | It consists of OSId and OSAppId(s) (NOTE 2, NOTE 8). | Optional |
| IP descriptors (NOTE 6) | Destination IP 3 tuple(s) (IP address or IPv6 network prefix, port number, protocol ID of the protocol above IP) (NOTE 8, NOTE 12). | Optional |
| Domain descriptors | FQDN(s) or a regular expression which are used as a domain name matching criteria (NOTE 7, NOTE 8). | Optional |
| DNN | This is matched against the DNN information provided by the application (NOTE 8). | Optional |
| Connection Capabilities | This is matched against the information provided by a UE application when it requests a network connection with certain capabilities (NOTE 4, NOTE 8) or traffic categories (NOTE 5). | Optional |

In order to support the UE data collection, the UDCF IP address, or UDCF FQDN or a new defined AIML connection capability can be set in the traffic descriptor.

The structure of Route Selection Descriptor is defined in Table 6.6.2.1-3 in TS 23.503, the user plane UE data collection dedicated PDU session parameters (e.g., DNN and S-NSSAI) are set in Route Selection Descriptors to the UE as part of URSP rule.

After UE receives the UDCF UP information in step 8 in Figure 6.x.3.1-1, or step 4 in 6.x.3.1-2, the UE will bind the data collection traffic to the PDU session with specific DNN and S-NSSAI.

b) QoS flow level

The packet Filter set is used in the QoS rule and the PDR to identify one or more packet flow. Based on operator’s decision, SMF provides the QoS rule to UE and PDR to UPF during SM procedure. For the IP PDU session type, the packet filter set support packet filters based on IP 5-tuples as described in clause 5.7.6.2 in TS 23.501.

The UDCF IP address / IPv6 prefix or port number can be used as packet filter to differentiate the data collection traffic to other regular traffic.

#### 6.x.3.3		Format of data reporting parameters send to UDCF

The mechanism proposed in this clause can be applied to both 5GC-centric variant and OAM-centric variant.

It is proposed HTTP/2 is used for user plane signalling between UE and UDCF.

The details of HTTP protocol format used for UE data collection can be defined by stage-3.

SA2 defines a standard format of data reporting parameters for different use cases, see the example in table 6.x.3.3-1.

Table 6.x.3.3-1 Data reporting parameters format

| Information name | Presence | Description |
| --- | --- | --- |
| Event ID | M | The event ID is defined in spec for each use cases that supported by RAN WGs, e.g., beam management. |
| Reported parameters | M | The UE measurement parameters, the content of reported parameters are defined by RAN WGs. e.g., RSRP |
| Data exposure restriction | O | Indicate if any data processing in UE side, e.g., normalization within the reporting periodic. |

Considering the future enhancement, it is also supported to define some operator specific event ID and the corresponding UE measurement parameters. The detail can be defined by stage 3 based on operator’s requirement.

#### 6.x.3.4	Data collection reject and cancellation

It is supported by UE to reject the data collection requested from UDCF due to UE’s local decision, e.g., UE’s power condition, the radio connection or local configuration in UE side. Figure 6.x.3.4-1 is call flow of Data Collection reject procedure.

Figure 6.x.3.4-1 Data collection reject procedure

1. UDCF sends data collection request to UE, with the requested event ID and the reporting information. The reporting information may include the reporting time period, number of reporting, reporting time window, reporting event condition, .etc.

2. If UE can not support the reporting information, e.g., the number of reporting, reporting time period or reporting time window due to UE’s local decision, UE will reject the data collection to UDCF.

In order to reduce the UE data collection reject, the UDCF may provide some alternative reporting information to UE as proposed in Figure 6.x.3.4-2.

Figure 6.x.3.4-2	Data collection with alternative reporting information

1. UDCF sends data collection request with the requested event ID and the reporting information. For each reporting information, the UDCF may provide different allowable values with a precedence sequence to the UE. For example, the UDCF may include different number of reporting, different reporting time period, or different time window.

2. UE will consider the UE power condition, radio condition or local configuration and select the more suitable reporting information value. UE will include the selected value for reporting information in the response message send to UDCF.

After UE accept the data collection request, UE may decide to cancel the data collection procedure due to UE’s local decision as described in Figure 6.x.3.4-3.

Figure 6.x.3.4-3: Data collection cancellation

1. UE decides to cancel the data collection, the data collection cancellation message is sent to UDCF

2. UDCF sends ACK to UE.

All the procedure that proposed between UE and UDCF in this clause is based user plane signalling, and the HTTP protocol is proposed for the interaction between UE and UDCF. The detail of the HTTP enhancement to support all these procedures will be determined by stage-3.

### 6.X.4	Impacts on services, entities and interfaces

UDCF

- New 5GC entity to support UE “raw” data collection and exposure either to DCCF or OAM

- support the HTTP procedure for the interaction between UE and UDCF

DCCF

- Enhance to support data exposure to AF

- Enhance to support UDCF discovery

UE

- Receiving UDCF UP information via NAS signalling

- Establishing the UP connection to UDCF

- Receiving UE collected data collection request and data reporting  to UDCF  via UP signaling

- support the HTTP procedure for the interaction between UE and UDCF

- Support the Data reporting parameters format

OAM

- Can receive “raw” UE data collected by UDCF. Details on further procedures up to SA5.

# >>>>END OF CHANGES<<<<