3GPP TSG-SA WG2 Meeting #169	S2-2504603

Fukuoka, Japan, 19th May 2025 – 23rd May 2025

Source:	OPPO

Title:	New Solution for KI#4: Data collection from multiple sources

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: The contribution proposes a new solution on sensing data collection from multiple sources.

- Discussion

KI#4 have been defined in TR 23.700-14 as below:

Integration of sensing and communication requires 3GPP network to collect and transfer sensing data and the associated information to a Sensing Function. This Key Issue aims to study: - How to collect sensing data (and the associated information, if any) for sensing result generation? - Which Sensing Function performs sensing result generation? - Whether and what the above associated information is required for sensing result generation NOTE: Implications to RAN or RAN dependent aspects will be coordinated with RAN WGs.

Due to different purposes, sensing data may be collected by multiple network elements. The decentralized and multi-purpose nature of sensing data collection in networks introduces significant inefficiencies. When identical data is gathered by multiple network elements for distinct objectives, such as network management, analytics, or AI model training, it leads to redundant storage, increased overhead, and fragmented utilization. Storage resources are wasted as duplicated datasets accumulate across nodes, while interoperability barriers caused by inconsistent formats, granularities, or reporting cycles prevent cross-functional data reuse. Furthermore, repeated measurement triggers, each tied to a specific use case, impose unnecessary computational strain on UEs and network infrastructure, degrading performance for active services. For example, a UE might endure multiple measurement operations or signaling procedures to satisfy separate requests for mobility tracking, interference mapping, and QoS validation, even though a single dataset could suffice. This siloed approach not only escalates operational costs but also complicates synchronization and analysis, as data becomes dispersed and contextually decoupled. Streamlining collection mechanisms through centralized metadata orchestration, standardized interfaces, and purpose-agnostic data lakes could mitigate redundancy, reduce signaling overhead, and enhance cross-domain utility.

In the meantime, centralized data collection mechanism may lead to unnecessary delay and increase network load for data transferring, as the data can’t be transferred directly from the sensing entity to the entity where sensing data is processed.

Centralized data collection mechanisms often introduce inefficiencies, as sensing data cannot be transmitted directly from the sensing receiver (i.e. UE or gNB) to processing nodes. Instead, data must traverse intermediate aggregators or core network elements, increasing transfer latency and congesting backhaul links. This indirect routing also amplifies network load, particularly with high-frequency or voluminous data streams.

While centralization simplifies management, it creates a trade-off between data consolidation and real-time responsiveness. To mitigate this, a solution needs to be developed to consider both data consolidation and real-time responsiveness.

- Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-14.

* * * Start of Change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | Key Issue #1 | Key Issue #2 | Key Issue #3 | Key Issue #4 | Key Issue #5 | Key Issue #6 |
| #1 |  |  |  |  |  |  |
| #2 |  |  |  |  |  |  |
| #X | X |  |  | X |  |  |

* * * Next Change (All new text)* * * *

## 6.X	Solution #X: Data collection from multiple sources

### 6.X.1	Description

This solution is for Key Issue#1 and Key Issue#4 on Data collection from multiple sources.

Due to different purposes (e.g. network management, data analytics, AI model training, or sensing service), sensing data may be collected by multiple network elements, including both the NF for managing sensing service, i.e. SSMF (Sensing Service Management Function), and the data collection node specified by the NF managing non-sensing service. For data collection, Sensing Entity may be configured by the SSMF and/or the NF managing non-sensing service with some events or be immediately triggered to perform sensing for measurements.

After the SSMF successfully selects Sensing Receivers (based on different sensing modes, the SSMF may select one or multiple UEs and/or gNBs as sensing receivers), it first interacts with the NF managing non-sensing service to verify whether sensing data collection (e.g. the selected Sensing Receiver (s) is configerred with events for sensing data collection) meeting the current sensing service requirements is already being performed on these sensing receivers:

- If relevant data collection already exists, the SSMF collects data from the data collection node specified by the NF managing non-sensing service

- Otherwise, the SSMF collects data directly from the Sensing Receivers

In either case (collecting from the non-Sensing Service Management Function’s specified node or directly from sensing receivers), the SSMF first determines whether to collect data via UP. If UP-based collection is chosen, data is transmitted via an established PDU session/data connection or a newly established PDU session/data connection.

For any subsequent matching sensing service requests, the SSMF collects sensing data in the same manner, regardless of whether the request originates from the same or a different sensing requester.

### 6.X.2	Procedures

Figure 6.X.2-1: Sensing data collection service flow

- Non-sensing service is activated and the measurement data (which includes sensing data) is reported to the data collection entity.

- SSMF receives a sensing service request, and successfully selected transmitter(s)/receiver(s).

- SSMF sends a sensing data collection message to the NF managing Non-sensing service / data collection entity (specified by the NF managing Non-sensing service) to request/subscribe sensing data. The message includes the following information:

- data collection session ID

- Receiver ID(s) obtained at Step 2

- object sensing service area obtained from the sensing service request

- Time period obtained from the sensing service request

- QoS requirement obtained from the sensing service request

- Time interval obtained from the sensing service request if it a periodic request

- Event(s) obtained from the sensing service request if it is event-based request

- Sensing result type

- NF managing Non-sensing service / data collection entity checks if any data collection session matches with the sensing data service request. If any matched data collection session is available, the NF managing Non-sensing service / data collection entity responds to SSMF with the following information:

- data collection session ID

- data collection entity Address

- Sensing data format

- Sensing data volume

- Sensing data

- data collection entity capability to support UP transport

If SSMF receives information indicating available matched data collection session, e.g. data collection session ID, data collection entity Address, Sensing data format, Sensing data volume, Sensing data, SSMF collects data from the data collection entity for the sensing service request; otherwise SSMF collects data from sensing receiver, i.e. step 5-7 are skipped.

- SSMF determines whether the sensing data is transferred over CP or UP based on sensing data format, sensing data volume, data collection entity UP capability received from step 4, service type received from the sensing service request and local configuration.

- If SSMF decides to transfer data over UP, it checks if UP connection is already established with data collection entity, if yes, then this connection is used for sensing data collection; otherwise, SSMF establishes a UP connection to the data collection entity based on its address obtained from step 4 or preconfigured address.

- Sensing data collection is conducted between data collection entity and SSMF. For any further matching sensing service request, SSMF collects data over this connection.

- If SSMF receives information indicating no available matched data collection session, SSMF determines whether the sensing data is transferred over CP or UP between Sensing Receiver and SSMF based on sensing data format, sensing data volume, data collection entity UP capability received from step 4, service type received from the sensing service request and local configuration.

- If SSMF decides to transfer data over UP, it checks if UP connection is already established with the sensing receiver, if yes, then this connection is used for sensing data collection; otherwise, SSMF establishes a UP connection to the sensing receiver based on sensing receiver ID/Address.

- SSMF sends a sensing data collection request to the Sensing Transmitter and Sensing Receiver. Based on the sensing mode, the sensing data collection request may be sent to the Sensing Receiver directly or via Sensing Transmitter; the sensing data collection request may also be sent to the Sensing Transmitter directly or via Sensing Receiver. This message may be sent over CP or UP.

- If there’s no on-going sensing procedure matching the sensing data collection request, the Sensing Transmitter and Sensing Receiver perform sensing.

- Sensing data collection is conducted between the Sensing Receiver and SSMF. For any further matching sensing data collection request, SSMF collects data over this connection.

### 6.X.3	Impacts on services, entities and interfaces

SSMF:

  - Support sensing data collection from data collection entity

  - Support sensing data collection from Sensing Receiver

  - Support sensing data collection configuration to the Sensing Receiver

Receiver:

  - Support sensing data collection to SSMF

  - Support sensing data collection to data collection entity

  - Support sensing data collection configuration by SSMF

  - Support sensing data collection configuration by NF managing non sensing service

  - Support sensing measurement

NF managing non-sensing service:

  - Support request/subscription from SSMF for sensing data collection

  - Support sensing data collection from Sensing Receiver

  - Support sensing data collection configuration to the Sensing Receiver

Data collection entity:

  - Support request/subscription from SSMF for sensing data collection

  - Support sensing data collection from Sensing Receiver

* * * End of Change * * * *