3GPP TSG-WG Meeting #169	S2-2504609

# Fukuoka, Japan, May 19 - 23, 2025

Source:	Rakuten Mobile, NIST

Title:	Pseudo-CR on New Solution Key Issue #1: System Architecture to Support Sensing

Document for:	Approval

Agenda item:	20.2.1

Spec:	3GPP TR 23.700-14

Version:	0.1.0

Work Item:	FS_Sensing_ARC / Rel-20

# 1. Introduction/Discussion

This paper proposes a solution to Key Issue #1, System Architecture to Support Sensing. We propose a new Network Function “Sensing Function” for sensing services.

# Text Proposal

It is proposed to agree the following key issue(s) into TR 23.700-14.

* * * First Change * * * *

## 6.X 	Solution #1: Architecture enhancement to support Sensing          Services

### 6.X.0	High-level solution principles

A new NF called “Sensing Function” is introduced for service operations and procedures to support Integrated Sensing and Communication.

Integrated Sensing and Communications as a new feature requires 5GS to support authorization and revocation for a particular sensing service, Sensing entity(ies) discovery and (re-)selection, collect and transfer sensing data, sensing result exposure, parameter provisioning of sensing entities etc. To meet these requirements, the current 5G System (5GS) as specified in TS 23.501 [X] must be further enhanced.

### 6.x.1	Description

This solution addresses Key Issue # 1

This solution proposes:

- that a new NF is introduced in 5G core network to address the following functionalities:

- register itself to NRF

- authorize the sensing request from Consumer NF, AF

- sensing entities discovery and selection

- sensing measurement data collection and calculation

- sensing result exposure

- NRF is enhanced to register SF

- NEF is enhanced to receive sensing request and expose results to authorized 3rd parties.

- UDM is enhanced to store and manage sensing authorization data

Figure 6.x.1-1 describes the architecture of this solution.

Figure 6.x.1-1: Architecture assumption to support Sensing Services

### 6.x.2	Procedures

Figure 6.x.2-1: Procedure for Sensing related information collection and exposure

If AF triggers the exposure request of the energy consumption related information, then steps 1a, 1b, 1c, 1d, 1e are executed, otherwise step 1 is executed.

1a.	If AF triggers subscription of NEF on sensing function related information with Nnef_EventExposure_Subscribe Request, in the request, AF provides the subscription information including sensing related information such as purpose of sensing, target area granularity (e.g. TAI, PLMN etc), QoS requirement, reporting mode (defines whether the report is to be provided on event basis or periodically), reporting threshold (if the reporting mode is per event basis) and Time period (if the reporting mode is periodic).

1b. NEF send subscription request to SF with Nsf_Sensing_Exposure_Subscribe Request and interacts with the UDM to check the sensing service description related information

1c. SF confirms the request and send subscription confirmation to NEF with Nsf_Sensing_Exposure_Subscribe Response.

1d. NEF confirms the request to AF with Nnef_EventExposure_Subscribe Response.

1. SF is triggered to perform sensing related information collection and exposure based on the local operator policy (e.g., the granularity of sensing related information exposure).

2.	The SF performs the sensing entities discovery and selection according to the requirement in the sensing service description, optionally the AMF selection.

3.	SF exchanges any relevant configurations/adjustments and coordinates with the Sensing entities (e.g., parameters such as sensing area, sensing operation period and sensing operation time window etc.)

4.	Sensing entities initiates the sensing and measurements

5.	Sensing entities reports the sensing measurement data according to the configuration from SF

6.	The SF calculate the sensing measurement data and derives the sensing result.

7.	SF reports the sensing result to NEF.

8.	The NEF reports the sensing result from the SF to AF.

### 6.x.3	Impacts on services, entities and interfaces

SF: New NF. Functionalities are as description of clause 6.x.1.

NRF: Enhanced to support registration of SF

UDM: Enhanced to store and manage the authorization data for sensing service

NG-RAN: Expose the Sensing measurements to SF

NEF：There is an impact on existing Nnef_EventExposure_Subscribe service.

* * * End of Changes * * * *