3GPP TSG-SA WG2 Meeting #169	S2-2504630

19-23 May 2025 Fukuoka Japan

Agenda item:	20.4.1

Source:	NEC

Title:	KI#1 new solution: Consumption energy source exposure

WI/Release:	FS_EnergySys_Ph2 /Rel-20

Document for:	Approval

1. Introduction

This contribution proposes a new solution for KI#1 for the Rel-20 FS_EnergySys_Ph2 study.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-67.

***** First change  *****

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |
| --- | --- | --- | --- |
|  | 1 | 2 | 3 |
| X | X |  |  |

***** End of first change  *****

***** Second change  *****

## 6.x	Solution #X: Consumption energy source exposure.

### 6.x.1	Description

This solution applies to Key Issue #1.

Nowadays consumers are more sensitive towards the environment and the consumers may want to reduce their carbon footprint by choosing renewable energy sources over non-renewable energy sources for network services. The 3GPP system, subject to operator’s policy and agreement with 3rd party, may be required to expose information not only on the energy consumption as a whole, but also an additional information on the consumed energy mix, i.e. the energy sources of the consumed energy. This solution proposes that a service provider, e.g. AF, may request the EIF, in addition to energy consumption data, to provide the following additional information related to the source of the consumption energy:

- The share of the renewable energy as part of the full consumption energy;

- The energy mix of the consumption energy, e.g. the share of the nuclear power energy, thermal power energy, solar power energy, hydroelectric power energy and wind power energy of the full consumption energy, when available.

For this, the AF may indicate with a parameter in the event exposure subscription service operation to the EIF that an energy source information exposure is required per the requested granularity, e.g. UE or S-NSSAI of UE or Application, if available. The EIF may also indicate that an energy source information is required, when available, when the EIF collects the energy consumption information from the OAM and UPF, for example. When such information is received or available in the EIF, the EIF shall calculate the share of each energy source as percentage of the whole consumed energy per the exposed granularity, for example.

### 6.x.2	Procedure

Figure 6.x.2-1: Consumption energy source exposure

- The AF subscribes for energy consumption event by invoking the Nnef_EventExposure_Subscribe Request service operation which includes the energy source information required parameter.

- Based on the request from the AF, the NEF subscribes for energy consumption event by invoking the Neif_EventExposure_Subscribe Request service operation which carries the energy source information required parameter.

- The EIF acknowledges the subscription for the energy consumption event.

- The NEF acknowledges the subscription for the energy consumption event from AF.

- The EIF calculates the share per each type of source energy, e.g. the share of the renewable energy and also the shares of the nuclear power energy, thermal power energy, solar power energy, hydroelectric power energy and wind power energy of the full consumption energy, when available. The share of each type of source energy may be represented as a percentage of the full consumption energy, for example. The EIF may also indicate energy source information required parameter when collecting the energy consumption information from other NFs like OAM and UPF, for example.

Note: It is assumed that the 5G system can obtain energy supply mix information from the energy providers. How to obtain this information is out of scope. The adjustment is not assumed to be real-time, it’s up to the operator’s policy to define the time difference between obtaining the change on energy supply mix and providing adjustment on communication service.

- The EIF notifies the NEF with the energy consumption information by invoking Neif_EventExposure_Notify service operation. If the energy source information was available, the EIF provides the energy source information as well.

- If the NEF receives the notification from the EIF, the NEF notifies the AF with the energy consumption information by invoking Nnef_EventExposure_Notify service operation. If the energy source information was provided by the EIF, the NEF provides the energy source information as well.

### 6.x.3	Impacts on existing services, entities and interfaces

EIF:

-	source energy share of the full consumption energy calculation per requested granularity when available

NEF/AF:

-	energy source information required parameter in the EventExposure_(Un)Subscribe Request service operation

-	energy source information parameter in the EventExposure_Notify service operation

*****  End of second change  *****