3GPP TSG-SA WG2 Meeting #169	S2-2504631

19-23 May 2025 Fukuoka Japan

Agenda item:	20.4.1

Source:	NEC

Title:	KI#2 new solution: UE energy saving service in low UE battery

WI/Release:	FS_EnergySys_Ph2 /Rel-20

Document for:	Approval

1. Introduction

This contribution proposes a new solution for KI#2 for the Rel-20 FS_EnergySys_Ph2 study.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-67.

***** First change  *****

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |
| --- | --- | --- | --- |
|  | 1 | 2 | 3 |
| X |  | X |  |

***** End of first change  *****

***** Second change  *****

## 6.x	Solution #X: UE energy saving service in low UE battery

This solution applies to Key Issue #2.

### 6.x.1	Description

With the development of mobile networks the 3gpp network is expected to accommodate more services like Augmented Reality (AR), Extended Reality (XR) called immersive services which can cause higher energy consumption at the device side. To achieve providing immersive service experience, the 3GPP system is required to provide a long-time high data rate data transmission with low energy consumption in the device. To achieve energy efficiency, the operator will need to introduce the concept of energy as a service and to provide different levels of energy saving services for subscribers. This solution proposes two new UE energy saving services:

- UE energy saving service level A (high price subscription): the network provides high service experience in high UE battery and maintains high service experience in low UE battery (e.g., low latency, high throughput).

- UE energy saving service level B (low price subscription): the network provides high service experience in high UE battery and reduced service experience in low UE battery (e.g. performs QoS degrading).

It is assumed that a user is subscribed to one of the above UE energy saving services via the service provider AF or alternatively by the operator, based on the SLA agreement between the operator and the service provider. The AF monitors on the application level the UE battery when the UE is active using certain immersive service, for example. When the UE battery goes bellow certain threshold, the AF indicates that event to the 3gpp system. If the UE is subject to energy saving, the 3gpp system asks the PCF to activate the UE subscribed UE energy saving service policy.

### 6.x.2	Procedure

Figure 6.x.2-1: Energy saving service in low UE battery

- The AF subscribes for UE energy saving service by invoking Nnef_EnergySavingService_Subscribe Request service operation which includes the UE_Id and the UE energy saving service type, i.e. UE energy saving service level A or UE energy saving service level B. The UE energy saving service type defines the energy saving policy applicable to the UE when the UE battery is low. The following UE energy saving services types are proposed:

- UE energy saving service level A (high price subscription): the network provides high service experience in high UE battery and maintains high service experience in low UE battery (e.g., low latency, high throughput).

- UE energy saving service level B (low price subscription): the network provides high service experience in high UE battery and reduced service experience in low UE battery (e.g. performs QoS degrading).

Note: It is up to the operator’s policy what UE energy saving methods will be deployed by the operator as part of UE energy saving service level A/B.

- The NEF subscribes for UE energy saving service invoking the Nurd_EnergySavingService_Subscribe Request service operation which carries the UE_Id and the UE energy saving service parameters, e.g. UE energy saving service level A or B.

- The UDR stores the UE energy saving service parameter as UE subscription information. Alternatively, the UE energy saving service parameter can be provided by the operator as a result of SLA agreement with the service provider AF.

- The UDR acknowledges the subscription for the UE energy saving service.

- The NEF acknowledges the subscription for the UE energy saving service.

- When a certain UE battery goes bellow a threshold and the UE has a subscription for UE energy saving service, the AF asks for the UE energy saving service to be activated.

- The AF invokes the Nnef_EnergySavingService  Request service operation which includes the UE_Id and the UE low battery parameter.

- The NEF forwards the request from the AF to the UDR via the Nurd_EnergySavingService_Request service operation.

- If the UE is subject to energy saving and the UE has a subscription for a specific UE energy saving service, the UDR indicates the UE_Id and the UE subscribed UE energy saving service parameter, e.g. UE energy saving service level A or B to the PCF.

- The UDR invokes the Nudr_DM_Notify service operation in which the UDR includes the UE_Id and the UE subscribed UE energy saving service parameter.

- The PCF activates the UE subscribed UE energy saving service policy for a low UE battery situation.

### 6.x.3	Impacts on existing services, entities and interfaces

AF/NEF:

-	UE energy saving service parameter handling and provision to the UDR.

-	UE low battery parameter indication to the UDR.

UDR

-	UE energy saving service subscription information and UE low battery parameter handling.

PCR

- 	activates the requested UE energy saving service subscription policy.

*****  End of second change  *****