3GPP TSG-SA2 Meeting #166AH	S2-2504644

# emeeting, Elbonia,  20 – 24th January, 2025	  			       		          (revision of S2-250)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | DRAFT | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Documenting tools to support Fixed Wireless Access (FWA) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20_FWA |  |  |  |  |  | Date: |  |  | 2025-01-04 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Optimize networks support of FWA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of an Annex about Fixed Wireless Access (FWA) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Waste of signaling resources and of related Energy spending |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

Annex XX (informative):
Support of Fixed Wireless Access (FWA)

# XX.1	General

This Annex provides examples on how to optimize networks to support Fixed Wireless Access.

In order to lower the signaling load due state transitions e.g. between CM-IDLE and CM-CONNECTED states or to optimize RAN resources, the NG-RAN can use local configuration associated e.g. with allowed S-NSSAI(s) and/or with RFSP index referring to Fixed Wireline Access to determine for an UE using FWA proper RAN policies e.g. for setting UE inactivity timer(s) or for deciding the most optimal radio layer that can serve the UE.

In order to lower the signaling load due paging, the AMF can use local configuration associated with S-NSSAI(s) referring to Fixed Wireline Access to determine proper paging strategies.

NEXT CHANGE (2)

NEXT CHANGE (3)

END OF CHANGES