3GPP SA WG2 Meeting #169	S2-2504646

Fukuoka, Japan, May 19-23, 2025

# Title:	[Draft] Reply LS on paging ID length

# Release:	Release 19

# Work Item:	AmbientIoT, Ambient_IoT_solutions

Source:	Ofinno [Update to SA2]

To:	RAN2

CC :	CT4, RAN3

Contact Person:

Name:	SungDuck Chun

E-mail Address:	schun@ofinno.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

SA2 would like to thank RAN2 for LS on paging ID length, and sharing latest progress in RAN2.

Section 5.7.2 of TS 23.369 v0.3.0 describes AIoT permanent identifier:

- The AIoT permanent identifier includes domain information. Domain information can have none or at least one of three different fields (PLMN ID, NID ID, third party ID).

- The AIoT permanent identifier includes identification information. This can be either 96 bits or 128 bits.

Based on this, for AIoT permanent identifiers, SA2 currently see at least 10 different variations. But, this does not necessarily mean that all 10 different variations have different lengths. Some variations of the AIoT identifier construction may have the same length. It is FFS whether different variations of the same length need to be identified as early as possible.

In addition, SA2 would like to give following further feedback:

- SA3 study is ongoing regarding use of Temporary AIoT device identifier. It is not yet clearer whether both Temporary AIoT device identifier and Permanent AIoT device identifier can be used in AIoT paging, and whether different sizes of Permanent AIoT device identifier leads to the same size of Temporary AIoT device identifier.

- Discussion on REL-20 scope for AIoT is ongoing, and further enhancement of AIoT will be pursued. It is FFS whether additional AIoT paging identifier types are needed (e.g., for reader initiated paging case). In addition, it cannot be ruled out that additional domain information or addition lengths is defined for identifier information, in later releases. Thus, AIoT paging message needs to be designed to support flexibility and future extension.

2. Actions:

To RAN2, RAN3:

ACTION: 	SA2 requests RAN2 to take above feedback into consideration.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #170		25-29 Aug 2025	Goteborg, Sweden