SA WG2 Meeting #169	S2-2504651

# May 19 – 23, 2025, Fukuoka, Japan

Source: 	CSCN

Title: 	KI#X: New Solution: SDP negotiation with 100 trying for IMS NB-IoT GEO Access

Document for: 	Approval

Agenda Item: 	20.1.1 (FS_5GSAT_Ph4_ARC)

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution for KI#X.

1	Introduction

This contribution proposes a new solution for WT-1 on SDP negotiation with 100 trying for IMS voice service over GEO satellite, to setup voice call with less call setup time and better UE experience. The solution includes how to use 100 trying to prepare media resource efficiently, and is intended to be used together with other solutions to achieve such optimization. This solution allows the minimal impact on existing network function and strictly follow the RFC3261 and so on.

2	Proposal

It is proposed to add the following text to TR 23.700-19.

********** First Change (All new text) **********

6.x	Solution #X: SDP negotiation with 100 trying for IMS NB-IoT GEO Access

6.x.1	Key Issue mapping

This solution addresses Key Issue #Y: IMS enhancement for GEO NB-IoT NTN access

6.x.2	Description

High-level solution principles are as follows:

- Less impacts on IMS nodes in the network.

- Simplify the transaction over air interface to minimize the call setup time.

- Keep the media reservation mechanic.

- Follow the RFC 3261 with less SIP protocol impact

6.x.3	Procedures

6.x.3.1	General

This solution aims to reduce the call setup time by using SIP 100 Trying message to carry SDP parameters.

6.x.3.2	IMS call setup origination procedure

Figure 6.X.3-1: Call Setup Origination Procedure with 100 Trying message optimization

1.The MO UE using GEO NB-IoT NTN access initiates an IMS call and sends INVITE with a P-ANI header that indicates a NB-IoT access type.

2.Upon receipt of the INVITE message, the P-CSCF in the originating side shall firstly invoke the IMS-ALG function to allocate transport resources both for the access side and the core side. On receipt of the response from the IMS-AGW, P-CSCF shall initiate dedicated EPS bearer activation procedure.

3. After the dedicated EPS bearer is ready, the P-CSCF shall generate a special 100 Trying message with SDP answer to simplify the QoS pre-conditions process. The P-CSCF shall set the answerer’s destination address and port in the 100 Trying message body as the transport resource information for the access side allocated by IMS-AGW and deliver the information to the MO UE.

4~10. The P-CSCF shall use the transport resource information for the core side to modify the offerer’s destination address and port in the INVITE message body and forward the INVITE to IMS core. The following SIP interaction steps between P-CSCF, originating IMS core and the terminating side will be kept as the traditional QoS pre-conditions process.

11.	The MO UE will ring after receiving 180 Ringing without SDP.

12~14.MT UE answers the call, send 200 OK to MO UE and MO UE responds ACK. The media paths between MO UE and P-CSCF, between P-CSCF and terminating side have been established

6.x.3.3	IMS call setup termination procedure

Figure 6.X.3-2: Call Setup Termination Procedure with 100 Trying message optimization

1.A session is initiated by originating side and an INVITE message with SDP offer is received by terminating P-CSCF.

2. Upon receipt of the INVITE message, The P-CSCF shall check the access type of MT UE. If the MT UE is using GEO NB-IoT NTN access, the P-CSCF shall invoke the IMS-ALG function to allocate transport resources both for the access side and the core side.

3.The P-CSCF shall use the transport resource information for the access side to modify the offer destination address and port in the INVITE message body and forward the INVITE to MT UE.

4.Upon receipt of the INVITE message, the MT UE using GEO NB-IoT NTN access shall generate a special 100 Trying message with SDP answer to simplify the QoS pre-conditions process.

5. On receipt of the 100 Trying message, the P-CSCF shall invoke the IMS-ALG to delivery the MT UE transport resource information to the IMS-AGW. On receipt of the response from the IMS-AGW, the P-CSCF shall initiate dedicated EPS bearer activation procedure.

6~10. Once the EPS bearer is ready, the PCSCF shall use the transport resource information for the core side to generate a 183 Session Progress message body and forward it to IMS core. The following SIP interaction steps between P-CSCF, terminating IMS core and originating side will be kept as the traditional QoS pre-conditions process.

11.	The MT UE response 180 Ringing without SDP.

12~14.MT UE answers the call, send 200 OK to MO UE and MO UE responds ACK. The media paths between originating side and P-CSCF, between P-CSCF and MT-UE have been established.

6.x.4	Impacts on existing services, entities and interfaces

UE:

- Handling a  media indicator in SIP 100 trying message, skip 183 process.

- Due to 100 Trying message is sent back after media control and dedicated bearer establishment, to reduce INVITE message re-transmission times, T1 timer should be set to a larger value in NB-IoT UE.

P-CSCF:

- Working as B2BUA when UE is using IMS NB-IoT GEO Access.

- Response INVITE which PANI with RAT NB-IoT with 100 trying including SDP for MO UE.

- Handling a media indicator in SIP 100 trying message, skip 183 process for MT UE.

- Optimize the media control process between IMS-ALG and IMS-AGW.

********** End of Changes **********