3GPP TSG-SA2 Meeting #169 	S2-2504656

# 19- 23 May 2025, Fukuoka, Japan

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6284 | rev |  | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of NG connection interruption due to hard feeder link switch over |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In R3-252348(RAN3#127bis in April 2025), it is agreed to support an new indicator in RAN configuration update in case of hard feeder link switch over, to notify the 5GC to suspend or resume the DL signalling/data transmission(s) to the gNB. The detail is described in TP R3-252408 and R3-252409. To align with RAN3 areeement, it is necessary to support the handling of NG interruption at 5GC accordingly. The generic procedure was already described in clause 6.7 of TR 23.700-29 by using existing per UE session procedure. Alternatively, a per node procedure was also proposed in previous meeting to significantly reduce signalling load. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To align with RAN3 agreement, adding an new clause to describe 5GC handling for NG interruption due to hard feeder link switch over. To suspend or resume the DL signalling/data transmission to the gNB, it is proposed to use per UE session procedure, or per node procedure as an optimization. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NG connection interruption due to hard feeder link switch over is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.11.X (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# FIRST CHANGE(all new)

5.4.11.X	Support of NG connection interruption due to hard feeder link switch over

For hard feeder link switch over, the regenerative satellite payload connects to only one NTN Gateway at any given time, then a radio link interruption and/or NG link interruption may occur during the transition between the feeder links. To avoid the data and signalling pactet loss during NG link interruption, an indication to suspend data and signaling procedures over NG interface is sent to AMF from gNB onboard satellite in the NGAP Configuration Update message when the Hard Feeder Link switch occurs, as described in clause 16.14.4 of TS 38.300.

After receving the indication, the AMF shall suspends all the signalling  to the gNB. The AMF determines all the active PDU sessions impacted by the gNB, e.g. via searching in the contexts of all UEs based on gNB ID or satellite ID, then invokes Nsmf_PDUSession_UpdateSMContext Request towards SMF for each PDU session, with an indication to suspend the N3 DL data transmission due to Hard Feeder Link switch. After receving the indication from AMF, the SMF sends the N4 Session Modification Request to UPF, with an indication to suspend the N3 DL data transmission due to Hard Feeder Link switch. The UPF suspends the N3 DL data transmission for corresponding PDU session after receving the indication from SMF. Based on configuration, the UPF can buffer or discard the received DL data. If an indication to resume the data and signalling procedures is recevied from the gNB when the Hard Feeder Link switch is completed, the AMF resumes the signalling  to the gNB and send to SMF for each impacted PDU sessions an indication to resume the N3 DL data transmission. Then SMF sends to UPF the indication accordingly to resume the N3 DL data transmission for corresponding PDU session.

Above procedures are approach based on per PDU session, it causes much signalling within 5GC during Hard Feeder Link switch. As an alternative optimization, the AMF can notify the SMF via a per node approach. After receving the indication from the gNB which is performing Hard Feeder Link switch, the AMF determines all the SMFs impacted by the gNB, e.g. via searching in the contexts of all UEs based on gNB ID or satellite ID. Then the AMF can indicate each of the SMFs to suspend the N3 DL data transmission due to Hard Feeder Link switch, by using a per node service. After receving the indication from the AMF, the SMF determines all the impacted PDU sesions, e.g. via searching in all the PDU sessions contexts based on gNB ID or satellite ID, and sends the N4 Session Modification Request to UPF for each PDU session, with an indication to suspend the N3 DL data transmission due to Hard Feeder Link switch. The SMF can receive the gNB ID or satellite ID from AMF during PDU session create or update procedures. If an indication to resume the data and signalling procedures is recevied from the gNB when the Hard Feeder Link switch is completed, the AMF and SMF can use above same per node approach to resume the N3 DL data transmission.

Editor's note:	The per node service between AMF and SMF is FFS. If and how to support per node procedure in N4 is FFS.

# End of CHANGEs