3GPP TSG-SA2 Meeting #169	S2-2504659

19- 23 May 2025, Fukuoka, Japan

Source:	China Telecom

Title:	23.700-19: New solution of combined CP/UP in a PDN connection for KI#1

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This paper propose solution of combined CP+UP in a PDN connection for KI#1

1Discussion

Based on previous discussion, we can see that either CP or UP based solution has its pros and cons, e.g. CP by default has higher priority than UP data which means less call setup time, UP has less overhead than CP which means more efficient transport for voice data. In this paper we propose to utilize the pros of both CP and UP, by combining CP bearer and UP bearer in a IMS PDN connection.

2 Proposal

It is proposed to agree the solution of combined CP/UP bearers in a PDN connection for KI#1 into TR 23.700-19 on FS_5GSAT_Ph4_ARC as follows

* First change *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 |  |  |  |
| X | x |  |  |  |

* Second change (all New)*

6.X	Solution #X: Combined CP/UP bearers in a PDN connection

6.X.0	High-level solution Principles

This solution utilizes the benefits of both CP and UP for separate transmission of IMS signalling and vioce data, by combining a CP bearer and a UP bearer in a IMS PDN connection. The solution only uses one dynamically allocated radio UP bearer for GEO call, so the remaining radio UP bearer (s) can be used for other services during the voice call, if the NB-IoT UE is assumed to support up to 2 radion UP bearers.

6.X.1	Description

This solution focus on the establishment of CP and UP bearer for IMS PDN connection. Other aspects for KI#1, e.g. how the UE decides to request IMS PDN connection based on information from EPS, is not described and is assumed to be solved by other solutions.

The CP bearer is the default bearer and established when the IMS PDN connection is setup, by using exising Control Plane CIOT optimiztion procedure. The CP bearer is used for transmission of IMS signalling. The UP bearer for transmission of voice data is dedicated bearer dynamically established when the the voice path needs to be setup, e.g. during the SIP negotiatoin between two UEs. In this solution User Plane CIOT EPS Optimisation is optional but not prefered, considering it only benefits for occasional data transmission. The UP bearer can be either Non-GBR or GBR bearer which will be aligned with RAN decision.

To support combined CP and UP bearers in IMS PDN connection, the SGW needs to simultaneously maintain corresponding S11-U path and S1-U path during the IMS call.

6.X.2	Procedures

6.X.2.1 Combined CP/UP bearers establishment

Figure 6.X.2.1-1: Combined CP/UP bearers establishment

- The UE initiates the PDN Connectivity Request to MME to rqeuest IMS PDN connection setup. It may be an initial attach request with PDN connectivity or UE requested PDN connectivity. The UE can indicates its support of combined CP/UP bearers in Preferred Network Behaviour in initial attach request or TAU.

NOTE: How the UE make the decision to request IMS PDN connection for voice service, e.g. based on information from RAN or EPC, is assumed to be solved by other solutions.

- Based on UE support of combined CP/UP bearers, RAT type(NB-IoT access), UE subscription and network capability, the MME determines to setup the IMS PDU session and allocates S11-U resource for default bearer(CP bearer). The MME sends a Create Session Request to SGW. Before this, the MME selects the SGW supporting combined CP/UP bearers via DNS query.

The MME includs S11-U info(IP address and TEID) and indicator of combined CP/UP bearers in Create Session Request. MME can includes the QoS for default bearer applied for IMS signalling.The exact QoS value(QCI) for CP bearer can be aligned with RAN decision.

During the PDN connection setup procedures, the PGW employs an IP-CAN Session Establishment procedure with the PCRF as defined in TS 23.203 to get the default PCC rules for the UE. The PCRF may configure the QoS parameters (QCI) associated with the default bearer, based on RAT type.

- The SGW returns a Create Session Response to MME, including S11-U info of default bearer.

- If separation of S11-U from S1-U is required, the SGW shall additionally include the SGW IP address and TEID for S1-U, as described in TS 23.401. This is different from the S1-U info for dedicated UP bearer applied for subsequent voice data at step 6.

If indicator of combined CP/UP bearers is received in step 2 from MME, the SGW should not change the default bearer from S11-U to S1-U.

- The MME sends Activate Default EPS Bearer request to UE. This message is contained in S1-AP Downlink NAS transport message to eNB. In the message the MME includes combined CP/UP bearers indicator to the UE.

If indicator of combined CP/UP bearers is received, the UE should not change the default bearer from S11-U to S1-U.

Above steps 1-4 follows the existing procedures of PDN Connectivity establishment for Control Plane CIoT EPS Optimisation, with enhancement on the IMS PDN connection to support combined CP/UP bearers.

After the PDN Connectivity is established, the UE uses the CP bearer to transport the SIP signalling to IMS(P-CSCF)

- If a call is received in IMS, following the existing IMS procedures, or the enhanced IMS procedures for GEO NB-IoT NTN access described in other solutions, the IMS(P-CSCF) requests the PCRF to authorize the resources necessary to establish a dedicated bearer for voice data transmission. Then the PCRF follows the Dedicated Bearer Activation Procedure as described in clause 5.4.1 of TS 23.401 by sending to PGW a PCC decision provision (PCC rule with QoS policy) message to modify the IP-CAN session.

- The SGW shall allocate S1-U resource for the bearer if indicator of combined CP/UP bearers is received in step 2 from MME. The SGW should not request creation of additional CP bearer . The SGW sends the Create Bearer Request to MME including the S1-U info for the Dedicated Bearer.

- The MME sends Activate dedicated EPS bearer context request message to UE, if the number of currently activated UP bearers for the UE dose not reach the maximum number of user plane radio bearers(existing NB-IoT specification supports 2 bearers at most). In the message the MME includes QoS(QCI) and TFT for the dedicated UP bearer to the UE.

Activate dedicated EPS bearer context request message is contained in S1-AP Bearer Setup Request message to eNB. The eNB establishes the corresponsing UP bearer, based on QoS info in Bearer Setup Request message .

NOTE: The UP bearer can be either Non-GBR or GBR bearer which will be aligned with RAN decision. The exact QCI value will be aligned with RAN decision.

- The MME sends Create Bearer Response to SGW, including EPS Bearer ID and S1-U info for UP bearer from eNB

After the activation of dedicated UP bearer, the UE can transport the voice data over UP bearer based on TFT.

6.X.3	Impacts to Services, Entities and Interfaces

The following impacts are foreseen by this solution:

MME:

- Establishment of combined CP/UP bearers in IMS PDN session, with CP for default bearer and UP for dedicated bearer

SGW:

- Establishment of combined CP/UP bearers in a PDN session, with CP for default bearer and UP for dedicated bearer

PCRF:

- Decides QoS for default CP bearer and dedicated UP bearer in IMS PDN session

UE:

- Establishment of combined CP/UP bearers in a PDN session, with CP for default bearer and UP for dedicated bearer

eNB:

- Support of activating both CP bearer and UP bearer for a UE, with respective bearer QoS

* End of change *