3GPP SA WG2 Meeting #169	S2-2504660

# 19 - 23 May, 2025, Fukuoka, Japan												(revision of S2-25xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1462 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on User consent |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-05-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The LMF retrieves user consent for data collection from UDM with the indication, with providing a specific purpose. The user consent purpose about “model performance monitoring” is not clearly described in clause 6.2.9 User consent for analytics. Moreover, “model performance monitoring” is not clearly specified in the current version of TS 29.503 as follows; 6.1.6.3.20 Enumeration: UcPurpose Table 6.1.6.3.20-1: Enumeration UcPurpose Enumeration value; Description. "ANALYTICS"; User consent for analytics.. "MODEL_TRAINING"; User consent for model training.. "NW_CAP_EXPOSURE"; User consent for network capability exposure.. "EDGEAPP_UE_LOCATION"; User consent for the manipulation of UE information for the purpose of UE Location retrieval by the EDGEAPP EAS entity.. This CR proposes to describe the purpose on “model performance monitoring” in the stage 2 specification, and send a LS to CT4 to ask the alignment to stage 3 specification. There are two alternative proposals for new NOTE, and one of them is expected to be specified based on the discussion. NOTE -option1: The purposes of user consent for model training and for performance monitoring are not separately specified in this Release of the specification. NOTE -option2: The user consent on ML model training includes the purpose for ML model performance monitoring in this Release of the specification. |
| Summary of changes: |  | - Clause 6.2.9: clarifying clearly the purpose on “model performance monitoring” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.273 CR#0732 (submitted at SA2#169) |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Draft LS to CT4 (S2-2504655) is submitted at SA2#169. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of 1st Changes * * *

6.2.9	User consent for analytics

Depending on local policy or regulations, to protect the privacy of user data, the data collection, ML Model training and analytics generation for a SUPI or GPSI, Internal or External_Group_Id or "any UE" may be subject to user consent bound to a purpose, such as analytics or ML Model training. The user consent is subscription information stored in the UDM, which includes:

a)	whether the user authorizes the collection and usage of its data for a particular purpose;

b)	the purpose for data collection, e.g. analytics orML model training and performance monitoring.

NOTE -option1:	The purposes of user consent for model training and for performance monitoring are not separately specified in this Release of the specification.

NOTE -option2:	The user consent on ML model training includes the purpose for ML model performance monitoring in this Release of the specification.

The NWDAF retrieves the user consent to data collection and usage from UDM for a user, i.e. SUPI prior to collecting user data from an NF as described in clause 6.2.2 and from a DCCF as described in clause 6.2.6 and prior to retrieving data from NWDAF (either directly according to clause 6.2.6.2 or via DCCF according to clause 6.2.6.3) or ADRF (either directly or via DCCF according to clause 6.2.6.3).

In roaming scenario, the H-RE-NWDAF is the enforcement point to check user consent. The H-RE-NWDAF retrieves the roaming-related user consent for a user from the UDM.

NOTE 1:	The content of the roaming-related user consent is specified in clause X.7 and Annex V of TS 33.501 [49].

If a request for analytics is for "any UE", meaning that the consumer requests analytics for all UEs registered in an area, such as a S-NSSAI or DNN or AoI, then the NWDAF resolves "any UE" into a list of SUPIs using the Namf_EventExposure service with EventId "Number of UEs served by the AMF and located in an area of interest" and retrieves user consent for each SUPI. If a request for analytics is for an Internal or External Group Id, NWDAF resolves it into a list of SUPIs and retrieves user consent for each SUPI.

If user consent for a user is granted, then the NWDAF subscribes to user consent updates in UDM using Nudm_SDM_Subscribe service operation. Otherwise, the NWDAF excludes the corresponding SUPI from the request to collect data and generate analytics or ML Model on the other users for which user consent is granted if the request is for a group of UEs identified by an Internal-Group-Id or "any UE".

When data is collected from the UE Application, the ASP is responsible to obtain user consent to share data with the MNO.

If the UDM notifies that the user consent changed, then the NWDAF checks if the user consent is not granted for the purpose of analytics or model training. If user consent was revoked for a UE, the NWDAF stops data collection for that UE. For analytics subscriptions to UE related analytics with the Target of Analytics Reporting set to that UE, the NWDAF stops generation of new analytics and stops providing affected analytics to consumers. For ML Model subscriptions with Target of ML Model Reporting set to that UE, the NWDAF containing MTLF stops (re-)training of ML Model(s) using data from the UE and stops providing the ML Model(s) to consumers (NWDAF containing AnLF) for analytics. If the Target of Analytics Reporting or Target of ML Model Reporting is either an Internal or External Group Id or a list of SUPIs or "any UE", the NWDAF skips those SUPIs that do not grant user consent for the purpose of analytics or model training. The NWDAF may unsubscribe to be notified of user consent updates from UDM for users for which data consent has been revoked.

NOTE 2:	The NWDAF can provide analytics or ML Model to consumers that request analytics or ML Model for an Internal or External Group Id, or for "any UE", skipping those users for which consent is not granted or is revoked.

The Analytics ID that needs to check user consent before collecting input data are those that collect input data per user, i.e. per SUPI, GPSI, Internal or External Group Id, or those with the Target of Analytics Reporting or Target of ML Model Reporting set to a SUPI, GPSI or External or Internal Group Id and are described in clause 6.

* * * End of Changes * * *