SA WG2 Meeting #169	S2-2504669

19 - 23 May, 2025, Fukuoka , Japan

Source:	IPLOOK

Title:	New solution for KI#2: IMS procedure optimization for IMS voice over GEO satellite

Document for:	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC/Rel-20

Abstract of the contribution: The contribution proposes a new solution for KI#2.

1	Discussion

The S-CSCF supports user information caching functionality as defined in Clause 4.2.4 of 3GPP TS 23.228:

The S-CSCF shall decide whether an Application Server is required to receive information related to an incoming initial SIP request to ensure appropriate service handling. The decision at the S-CSCF is based on (filter) information received from the HSS. This filter information is stored and conveyed on a per Application Server basis for each user. It shall be possible to include a service indication in the filter information, which is used to identify services and the order that they are executed on an Application Server within a single SIP transaction. The name(s)/address(es) information of the Application Server (s) are received from the HSS.

Proposal: A session persistence mechanism supports session suspension/recovery, which quickly rebuilds the session after a brief disconnection.

2	Proposal

It is proposed to include the following changes in TR 23.700-19 V0.1.0.

* * * * Start of Changes (All text new) * * * *

6     Solutions

6.0   Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |
| x |  | x |  |

6.x	Solution #X: IMS procedure optimization for IMS voice over GEO satellite

6.X.1	General description

The S-CSCF supports user information caching functionality as defined in Clause 4.2.4 of 3GPP TS 23.228:

The S-CSCF shall decide whether an Application Server is required to receive information related to an incoming initial SIP request to ensure appropriate service handling. The decision at the S-CSCF is based on (filter) information received from the HSS. This filter information is stored and conveyed on a per Application Server basis for each user. It shall be possible to include a service indication in the filter information, which is used to identify services and the order that they are executed on an Application Server within a single SIP transaction. The name(s)/address(es) information of the Application Server (s) are received from the HSS.

GEO provides wide-area coverage with continuous service in theory. However, for NB-IoT devices, physical obstructions or power-saving modes can take them offline. Rapid network recovery can reduce the loss caused by network interruption.

This contribution proposes a session persistence mechanism. When the network is interrupted, S-CSCF uses cached information to support rapid network recovery. The function of session persistence mechanism is:

- ‌Session state maintenance‌: While the UE is temporarily offline (e.g., due to obstructions or power-saving mode), the S-CSCF caches session context (e.g., registration status, media negotiation parameters) to prevent complete session termination.

- ‌Fast recovery: Upon UE re-connection, the S-CSCF directly restores the session from cached data, eliminating the need to re-initiate full registration/authentication procedures, thereby reducing signaling overhead and latency.

6.X.2	Procedure

Figure 6.X.2-1:Terminal Recovery Trigger Procedure

- After the network is restored, the UE sends the Register information flow to the P-CSCF.

- The P-CSCF forwards the SIP REGISTER message towards S-CSCF.

- The S-CSCF queries the cache to verify whether it is within its validity period.

- If the cache is valid and has not expired, the S-CSCF skips authentication and sends a 200 OK to the UE. Otherwise, the UE needs to perform the complete registration process.

Figure 6.X.2-2: Session Triggered by AS

- AS sends a SUBSCRIBE request to S-CSCF to subscribe the registration events of the UE.

- After the network is restored, the UE sends a REGISTER request to re-register.

- S-CSCF sends NOTIFY to AS and marks the state of UE as active.

- The AS sends SIP INVITE request to S-CSCF.

- If the cache is valid and has not expired, the S-CSCF uses the cached IFC to determine whether AS needs to be triggered.

- The S-CSCF forwards the INVITE to ‌UE Contact address.

- After receiving the INVITE, the UE returns 200 OK.

6.X.2	Impacts

S-CSCF：

- Support S-CSCF to dynamically adjust the cache validity period.

- Support S-CSCF in querying the cache.

AS:

- Support AS subscribing to UE registration events from S-CSCF.

* * * * End of Changes * * * *