3GPP TSG SA WG2#169	S2-2504699

Fukuoka, Japan, May 19 – 23, 2025	(was S2-241xxxx)

Source:	Lenovo

Title:	New WT for AIML_CN_Ph2 work

Document for:	Discussion

Agenda Item:	30.2

Work Item / Release:	Release 20

Abstract of the contribution:

# 1	Discussion

## 1.1	Introduction

This DP proposes a new Work Task for the AIML_CN_Ph2 study based on discussions of ML Model Transfer from the RAN node to a UE-side server for the scenario where UE and RAN node use a “2-sided ML model” for an AI/ML-enabled functionality.

## 1.2	RAN progress on ML model transfer for 2-sided ML model.

SA2 has received the following LSs from RAN WGs related to 2-sided ML models.

S2-250041 LS on signalling feasibility of dataset and parameter sharing )(SA2 was on CC) indicating options related to inter-vendor training collaboration for 2-side ML model.

In particular, one scenario is for the NW-side (i.e. RAN mode) to provide to the UE/UE-side server training data (model parameters and/or training dataset associated to the model parameters) that the UE-side server uses to train a UE-side ML model as described below:

- NW-side shares model parameters and/or dataset to the UE or UE-side via standardized signaling, where the model parameters and/or dataset received at the UE or UE-side go through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., model (re-)training.

RAN1 has indicated that the method to convey standardized signalling corresponding to the transfer of model parameters and/or training data set from the NW-side to the UE-side server may be over-the-air (OTA) or by other approaches (non-OTA).

- If over-the-air solution is used the NW-side (i.e. RAN) conveys the standardized data over RRC signalling to the UE and the UE further forwards the data to the UE-side server via OTT mechanisms.

- RAN has not studied other approaches as it is outside the scope of RAN WGs.

R2-2503169 Reply LS on signalling feasibility of dataset and parameter sharing summarising OTA vs non-OTA approaches.

RAN2 has listed the following options of supporting non-OTA approaches where it is assumed that they can be supported within Rel-19 existing architecture framework, ,

Table 1. non-OTA candidate solutions

| Option | Impacted WG | Specification impact/Implementation impact |
| --- | --- | --- |
| Option 1: OAM -> UE-side training entity (a server inside MNO or an OTT server), where OAM is NW-side dataset/model parameter collection entity | SA5, SA3 | Up to SA5 (any intermediate node between OAM and UE-side OTT server is up to SA5; CN involvement if needed is up to SA2/SA5 discussion) |
| Option 2: CN -> UE-side training entity (a server inside MNO or an OTT server), where CN is NW-side dataset/model parameter collection entity | SA2, SA3 | Up to SA2 (any intermediate node between CN and UE-side OTT server is up to SA2) |
| Option 3: gNB -> OAM/CN -> UE-side training entity (a server inside MNO or an OTT server), where gNB is NW-side dataset/model parameter collection entity | RAN3, SA2, SA5, SA3 | Up to RAN3, SA2, SA5 (any intermediate node between gNB/OAM, OAM/UE-side OTT server, CN/UE-side OTT server is up to RAN3/SA2/SA5) |

For the options listed of SA2 merit the following observations are made:

Options 2 and 3 have a common assumption that the UE-side server may be a server inside the core network or an OTT server whereas the NW-side server is a dataset/model parameters collection entity in the core (Option 2) or in the gNB (Option 3). Both options 2 and 3 require a study in SA2 to determine their feasibility.

As far as the RAN assumption that all options can be supported within the existing Rel-19 architecture framework, SA2 can provide feedback on such assumption once the feasibility of the aforementioned options is completed.

Observation 1: For options of SA2 merit a study is needed in Release 20 (with potential coordination with other groups SA3/RAN3) to determine their feasibility to support the non-OTA approach.

In the LS RAN2 has also listed some challenges on supporting OTA solution as shown in the table below:

Table 2. OTA candidate solutions

| gNB -> UE via CP | Challenges | - UE is only required to support 45kB RRC buffer size, according to TS 38.306. - Significant specification impact: - Other segmentation beyond RRC layer requires a new SRB protocol stack to perform segmentation, including functions such as handling segmentation, retransmission, etc - UE selection - Challenges to support E2E reliability, considering dataset/model parameter transfer is shared by different gNB/vendors during UE mobility and different RRC state transition - Uu overhead for data collection from UE and dataset/model parameter transfer to UE - Overloading CP with large datasets would disrupt core control message transmission (e.g. service degradation, reliability, etc) |
| --- | --- | --- |
|  | Potential suitable scenario | - Small dataset/model parameter size. However, the maximum RRC segment needs to be further studied - Split large dataset/model parameter into small pieces, and potentially send to multiple UEs, then gather by UE training entity. RAN2 has not study the feasibility of split dataset/model parameter to multiple UEs. |
| CN -> UE via gNB | Challenges | - Same challenges as OTA solution ‘gNB->UE via CP’, if OTA solution ‘CN -> UE via gNB’ via CP - No benefit over non-OTA solution, as dataset/model parameter needs to transmit to CN, then transmit back to gNB. Relaying dataset/model parameter via gNB to UE then back to UE training entity is not desirable. - Unclear how to guarantee E2E reliability across multiple hops - A risk of proprietary information exposure if gNB and CN are not from the same NW vendor |
|  | Potential suitable scenario | - Feasibility analysis of OTA solution ‘CN -> UE via gNB’ via CP is the same as OTA solution ‘gNB -> UE via CP’ - OTA solution ‘CN -> UE via gNB’ and its feasibility is required to be evaluated by RAN3 and SA2. |
| OAM -> UE via gNB | Challenges | - Same challenges as OTA solution ‘gNB -> UE via CP’, if OTA solution ‘OAM -> UE via gNB’ via CP - No benefit over non-OTA solution, as dataset/model parameter needs to transmit to OAM, then transmit back to gNB. - Unclear how to guarantee E2E reliability across multiple hops - A risk of proprietary information exposure that OAM may share to a second NW vendor |
|  | Potential suitable scenario | - Feasibility analysis of OTA solution ‘OAM -> UE via gNB’ via CP is the same as OTA solution ‘gNB -> UE via CP’ - OTA solution ‘OAM -> UE via gNB’ and its feasibility is required to be evaluated by RAN3 and SA5. |

The following observations are made for options of SA2 merit:

gNB -> UE via CP Option: has considerable UE and RAN impacts as coordination is needed between the UE and the RAN node for setting up the radio resources for receiving the model parameters. In addition, the UE needs to additionally convey the data to the UE-side server.

CN -> UE via gNB Option: Requires the gNB to transfer NW-side dataset that are stored in an repository in the core network and when the UE requires the dataset triggering the transfer of the data from the repository to the UE via a control plane procedure. Such option has the same limitation as with the gNBUE via CP option (UE – RAN impacts).

Observation 2: OTA options have considerable UE and RAN impacts

# 2. Proposal

As the OTA solutions have considerable UE impacts it is proposed for SA2 to study alternatives that do not rely on the UE for conveying NW-side related training data set to the UE-side server. The non-OTA options of SA2 merit listed in R2-2503169 will be used as a basis for the study.

The following is proposed as a WT within the AIML_CN_Ph2 study:

New Proposed WT: 2-side ML model training collaboration:

- Study whether and how NW-side ML model training parameters (ML model parameters and/or training data set associated to the ML model parameters) are transferred to the UE-side server via the core network via standardized signalling that do not rely on the UE for conveying NW-side related training data set to the UE-side server.

NOTE:	The non-OTA options of SA2 merit listed in R2-2503169 will be used as a basis for the study.