3GPP TSG-SA2 Meeting #169	S2-2504703

# 19 - 23 May, 2025, Fukuoka, Japan

Source: 	vivo

Title: 	KI#2: New Solution –Collocation of UPF and NWDAF containing AnLF to support efficient performance of User Plane

Document for: 	Approval

Agenda Item: 	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes a solution for KI#2 to support efficient performance of User Plane by collocating UPF and AnLF.

1	Discussion

This paper proposes a solution to support efficient performance of User Plane by collocating UPF and AnLF.

Since UPFs in the network have plenty observed information and measurements data about UP traffic, they can be reported from UPFs to NWDAF for model training and model inference (i.e. to generates analytics). However, the data volume reported from UPFs to the NWDAF is big for the data reporting may be per UE or per PDU session or per service flow or even per packet, which may influence the UPF’s performance itself and may also cause the big load on Control Plane. Thus, it has been approved in the key issue description that the solution should consider to minimize the load on Control Plane NFs as much as possible. For model training, this issue can be avoided by offline data collection and reporting from the UPF to the NWDAF, however, for model inference this issue is still there. If the UPF is collocated with the NWDAF containing AnLF from architecture point, the issue can be avoided.

2. Proposal

It is proposed to add the following contents to TR 23.700-04.

Start of Change

6.0	Mapping of Solutions to Key Issues and Use Cases

Table 6.0-1: Mapping of Solutions to Key Issues and use cases

|  | Key Issues |  | Use Cases (optional) |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Use Case #1> | <Use Case #2> |
| #1 | x |  |  |  |
| #2 | x |  |  |  |
| #3 | x |  |  |  |
| #4 | x |  |  |  |
| #X |  | X |  |  |

Next Change (all new text)

6.X	Solution #X: Collocation of UPF and NWDAF containing AnLF to support efficient performance of User Plane

6.X.1	High level principles

The NWDAF containing MTLF is used to training ML model for user plane Traffic pattern.

The UPF is collocated with NWDAF containing AnLF, and it is used to generate inference result (i.e. analytics) of Traffic pattern based on the ML model from The NWDAF containing MTLF.

The UPF collocated with NWDAF containing AnLF notifies the consumer about the analytics of Traffic pattern, it will help the consumer perform corresponding mitigation actions e.g. avoid abnormal or malicious traffic.

6.X.2	Description

This solution is to address Key Issue#2: AI/ML-assisted user plane traffic pattern and behavior analysis to support efficient performance of User Plane.

Since UPFs in the network have plenty observed information and measurements data about UP traffic, the NWDAF containing MTLF can use these input data from UPFs to train ML model for user plane Traffic pattern. This ML model will be provided to NWDAF containing AnLF to generate analytics of Traffic pattern by performing ML model inference process.

Editor's Note: What input data should be collected to train ML model by the NWDAF containing MTLF is FFS.

However, considering latency and too much input data reporting from UPF to the NWDAF containing AnLF during online ML model inference process, this solution proposes to collocate the UPF and the NWDAF containing AnLF together. In this case, the ML model from the NWDAF containing MTLF will be offloaded to UPF collocated with AnLF, then the UPF can generate analytics result for Traffic Pattern itself, and this result can be notified to the consumer e.g. SMF to perform some corresponding mitigation actions, e.g. avoid abnormal or malicious traffic.

Editor's Note: What actions can be done by which consumers is FFS.

6.X.3	Procedures

6.x.3.1 Support of efficient performance of User Plane by collocating UPF and NWDAF containing AnLF

Figure 6.x.3.1-1:  Support of efficient performance of User Plane by collocating UPF and NWDAF containing AnLF

0. NWDAF containing MTLF trains ML model for Traffic pattern based on training data offline collected from data sources e.g., UPFs. And the NWDAF containing MTLF provides the trained ML model to the UPF which is collocated with NWDAF containing AnLF.

1. Consumer (e.g. SMF) may subscribe to the UPF collocated with AnLF about analytics of Traffic pattern.

2. Based on the ML model from the NWDAF containing MTLF, the UPF collocated with AnLF collects data e.g. UPF local data, N3 measurements, N6 measurements, etc. as input data for ML model inference.

3. Based on the ML model and input data, the UPF collocated with AnLF generates inference result (i.e. analytics) of Traffic pattern.

4. The UPF collocated with AnLF sends analytics of Traffic pattern to the consumer (e.g. SMF).

5. According to the Traffic pattern result, the consumer (e.g. SMF) performs specific action.

6.X.4	Impacts on services, entities and interfaces

UPF collocated with NWDAF containing AnLF

- Retrieves ML model for Traffic pattern from NWDAF containing MTLF;

- Generates analytics of Traffic pattern based on the ML model.

- Notifies the consumer about the analytics of Traffic pattern.

NWDAF containing MTLF:

-	Trains and provides ML model for Traffic pattern.

Consumer (SMF):

-	TBD

End of Changes