3GPP TSG-WG SA2 Meeting #169	S2-2504706

# Fukuoka, Japan, May 19 - 23, 2025

Source: 	China Mobile

Title: 	New solution for key issue #3: UPF (re)selection based on energy related information

Document for: 	Approval

Agenda Item: 	20.4

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: Propose a new solution for KI#3: NF (re)selection and/or UP path adjustment based on energy related information.

1	Discussion

This paper is to propose a new solution on KI#3.

According to key issue #3, NF selection enhancement to consider energy related information shall be studied. This solution proposes to include Renewable Energy Information and Energy Priority Information in the NF profile of UPF. The UPF registration, update and UPF (re)selection procedures are based on clause 4.17 of TS 23.502 [3]. SMF performs UPF (re)selection based on the enhanced UPF profile.

2	Proposal

It is proposed to include the below changes into TR 23.700-67.

FIRST CHANGE

6.0	Mapping of Solutions to Key Issues

Editor's note:	The number of columns of Key Issues will be updated according to clause 5.

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solution# | 1 | 2 | 3 | ... |
| Solution #X: UPF (re)selection based on energy related information |  |  | X |  |

NEXT CHANGE (all the text is new)

6.X	Solution #X: UPF (re)selection based on energy related information

6.X.0	High-level solution principles

This solution is proposed to enhance the UPF (re)selection, including

-	NF profile of UPF update with including energy related information, i.e., Energy Priority Information and Renewable Energy Information;

-	NRF provided the UPF energy related information to SMF;

-	SMF performs UPF (re)selection based on the energy related information.

6.X.1	Description

In order to support UPF (re)selection based on energy related information, this solution proposes to enhance the NF profile of UPF, including:

-	Energy Priority Information: This information describes the priority relative to other UPFs considering the energy characteristics.

-	Renewable Energy Information: This information describes the use of renewable energy by UPF, including one of the following:

- Energy Type: Describing whether the UPF uses renewable energy or non-renewable energy; or

- Renewable Energy Ratio: Percentage of Energy Consumption using renewable energy in the total Energy Consumption using all sources.

A UPF registers and updates the NF profile into the NRF. SMF performs UPF (re)selection considering the energy related information of the enhanced UPF profile.

NOTE:	The additional energy related information in the UPF's NF profile can alternatively be configured by OAM system.

The NF energy related information included in the NF profile can also be provisioned to SMF via N4 related procedures as defined in clause 4.4 of TS 23.502 [3].

6.X.2	Procedures

The NRF service framework procedures as defined in clause 4.17 of TS 23.502 [3] are reused for UPF (re)selection. UPF registration, update and (re)selection procedures are described as follows.

6.X.2.1	UPF registration procedure

Based on the NF Service Registration procedure in clause 4.17.1 of TS 23.502 [3], the Figure 6.X.2.1-1 describes the procedure of the UPF registration with energy related information to NRF.

UPF

NRF

1. Nnrf_NFManagement_NFRegister Request

2. Store UPF profile

3. Nnrf_NFManagement_NFRegister Response

Figure 6.X.2.1-1: Procedure for UPF registration to NRF

1.	The UPF sends Nnrf_NFManagement_NFRegister Request message to NRF with providing UPF profile. The profile includes energy related information as described in clause 6.X.1.

2.	The NRF stores the NF profile of the UPF and marks the UPF available.

3.	The NRF acknowledge NF Registration is accepted via Nnrf_NFManagement_NFRegister response.

6.X.2.2	UPF profile update procedure

UPF

NRF

1. Nnrf_NFManagement_NFUpdate Request

2. Update UPF profile

3. Nnrf_NFManagement_NFUpdate Response

0. Energy information for the UPF changed

Based on the NF Service Update procedure in clause 4.17.2 of TS 23.502 [3], the Figure 6.X.2.2-1 describes the enhanced procedure of the UPF profile update with energy related information to NRF.

Figure 6.X.2.2-1: Procedure for UPF profile update to NRF

0.	The UPF detects whether the energy related information (as described in clause 6.X.1) has changed.

1.	The UPF sends Nnrf_NFManagement_NFUpdate Request message (the updated NF profile of the UPF) to NRF to inform the NRF of its updated NF profile (e.g. with updated Renewable Energy Information) .

2.	The NRF updates the NF profile of the UPF.

3.	The NRF acknowledge NF Update is accepted via Nnrf_NFManagement_NFUpdate response.

6.X.2.3	UPF discovery and (re)selection procedure

Based on the NF Service discovery procedure in clause 4.17.4 of TS 23.502 [3], the Figure 6.X.2.3-1 describes the enhanced procedure of the UPF discovery and (re)selection based on energy related information with NRF.

SMF

NRF

1. Nnrf_NFDiscovery Request

2. Authorize UPF discovery

3. Nnrf_NFDiscovery Response

4. UPF (re)selection

Figure 6.X.2.3-1: Procedure for UPF discovery and (re)selection

1.	The SMF triggers of UPF (re)selection and requests NF discovery to NRF. When the SMF invokes Nnrf_NFDiscovery_Request, the message may include the requirements on energy related information (as described in clause 6.X.1).

2.	The NRF authorizes the Nnrf_NFDiscovery_Request.

3.	The NRF provides the NF profile(s) of the candidate UPFs, including the energy related information of each UPF (as described in clause 6.X.1).

4.	The SMF performs UPF (re)selection based on the received energy related information. For example, the SMF (re)selects a UPF using renewable energy or a UPF with required renewable energy ratio.

NOTE:	The detail of UPF (re)selection depends on operator’s policy.

6.x.3	Impacts on services, entities and interfaces

NRF:

-	Storing enhanced UPF profile including Renewable Energy Information and Energy Priority Information.

-	Providing the enhanced UPF profile to SMF.

UPF:

-	Performing NF registration with providing the enhanced UPF profile.

SMF:

-	Performing UPF (re)selection by considering the energy related information.

End of CHANGE