3GPP TSG-SA WG2 Meeting #169	S2-2504726

Fukuoka, Japan, May 19 – 23, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5491 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of DNAI Handling for UE-SAT-UE Communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In clause 5.2.8.3 of TS23.502: “Nsmf_EventExposure Service…… NOTE 2:The UP path change notification includes DNAI to represent the change of satellite identifier in case of UE-Satellite-UE (UE-SAT-UE) communication. NOTE 3:DNAI value can be derived from the satellite identifier as per operator policy and implementation.” This indicates that the input of Nsmf_EventExposure_Notify includes the DNAI, which is derived from the satellite identifier in accordance with operator policy and implementation. However, in clause 5.2.4.2.5 of TS23.502: “Npcf_PolicyAuthorization_Notify service operation Inputs, Optional: …… To support UE-Satellite-UE (UE-SAT-UE) communication, the UP path change notification includes DNAI to represent the change of satellite identifier. NOTE: DNAI value can be derived from the satellite identifier as per operator policy and implementation.” The note here suggests that the PCF may again derive the DNAI based on the satellite identifier, which is unnecessary. Given that the DNAI has already been derived by the SMF and included in the Nsmf_EventExposure_Notify, redundant derivation by the PCF should be avoided. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Avoided redundant derivation of DNAI by clarifying that the DNAI included in Npcf_PolicyAuthorization_Notify for UE-SAT-UE communication is already derived by the SMF and does not need to be re-derived by the PCF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PCF may redundantly derive the DNAI from the satellite identifier, leading to inconsistent interpretations across network functions and potential misalignment in UP path policy enforcement for UE-SAT-UE communication. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.5.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 5.2.5.3.5	Npcf_PolicyAuthorization_Notify service operation

Service operation name: Npcf_PolicyAuthorization_Notify

Description: provided by the PCF to notify NF consumers of the subscribed events.

Inputs, Required: Event ID.

The events that can be subscribed are defined in clause 6.1.3.18 of TS 23.503 [20].

Inputs, Optional: Event information (defined on a per Event ID basis) are defined in clause 6.1.3.18 of TS 23.503 [20], Notification Correlation Information (information to identify the application session), DNN, S-NSSAI.

Notification Correlation Information is mandatory except in the case of the new 5GS Bridge/Router information detected event if no AF session exists between the PCF and the AF.

DNN and S-NSSAI are required in the case of private IPv4 address being used for the IP type PDU Session that are potentially impacted by time sensitive communication and time synchronization service.

The PCF forwards UP path change notification to the AF in the case that UP path change notification is received at the PCF from SMF.

To support UE-Satellite-UE (UE-SAT-UE) communication, the UP path change notification includes DNAI to represent the change of satellite identifier.

The event notification may contain the type of notification ("EARLY" or "LATE").

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

# * * * * End of change * * * *