3GPP TSG-WG SA2 Meeting #169	S2-2504739

Fukuoka, Japan, 19 – 23 May 2025	  (was S2-250xxxx)

Source:	vivo

Title:	Update to inventory procedure

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: It proposes the update to AIoT Inventory procedure to fix ENs and align RAN agreement.

- Discussion

It is proposed the following updates to the AIoT Inventory procedure:

1. There are two ENs in clause “6.2.1 General”.

Editor's note: Additional information in the steps, parameters, and their naming throughout the procedures requires additional details, alignment with other clauses and references adding as required. Editor's note: Alignment is required for how to document and describe Direct Connectivity and Indirect Connectivity options in the procedures.

The LS S2-2504523 (R3-252481) provides Ambient IoT further progress in RAN3 which can be used to resolve these two ENs:

General Aspects: - Previous working assumption: “Including AIOTF information containers in NGAP, rather than introducing new protocol carried by NGAP” is changed to an agreement. - Existing NGAP Interface management procedures will be enhanced to support A-IoT. - RAN3 agreed to define a new “A-IoT Area”. The AIOTF may indicate the Requested Service Area as a list of “A-IoT Areas” and/or a list of readers in the Inventory Request message. The detailed coding of the new “A-IoT Area” is FFS. - The AIOTF is aware of the area within which the gNB supports the provision of A-IoT services by means of OAM. This area may be represented as “A-IoT Areas” and/or the readers supported by the gNB. On Inventory: - Within all Inventory related NGAP messages: - The Correlation Identifier is included inside the AIOTF information container. - The AIOTF Identifier is included outside the AIOTF information container. - Working assumption: The Correlation identifier is also included outside the AIOTF information container. …… - RAN3 agreed to introduce per-session per-device device context (“RAN NGAP Device ID”). - Upon receiving the “follow-on command indication”, the gNB allocates for each device a “RAN NGAP Device ID” and provides it in the AIOTF information container in the Inventory Report message. - The “RAN NGAP Device ID” is included in the AIOTF information container of each Command related NGAP message.

Proposal 1:

- The first EN can be removed after updates with additional information to the steps in the inventory procedure according to RAN3 progress.

- The second EN can be removed, there is a separate clause “6.2.4 procedures between AIOTF and NG-RAN for Indirect Connectivity” to address this issue.

2. There is an EN in step 1 in clause “6.2.2 Inventory Procedure”:

Editor's note: The parameters for the inventory service operation need further definition.

In RAN2 LS R2-2503199, RAN2 assumes that the inventory response size can always be provided by the core network to the reader.

Proposal 2: In clause “6.2.2 Inventory Procedure”, the approximate D2R inventory response message size is added and this EN can be removed.

3. There is an EN in step 7 in clause “6.2.2 Inventory Procedure”:

Editor's note: Whether the Inventory Request sent to NG-RAN includes indication about whether there will be a follow up command or not needs to be determined.

In the LS S2-2504523 (R3-252481), The RAN3 agreements are provided:

- “follow-on command indication” is introduced in the AIOTF information container in the Inventory Request message.

- The Correlation Identifier is included inside the AIOTF information container.

Proposal 3: This EN can be removed with the following updates:

- In step 7, a parameter “follow-on command indication” is added to the Inventory Request message;

- In step 8, a parameter “Correlation ID” is added to the Inventory Response message;

4. Some parameters are missing in step 10.

In RAN3 LS S2-2504523 (R3-252481)

- “RAN NGAP Device ID” is included in the AIOTF information container of each Command related NGAP message.

Proposal 4: The RAN NGAP Device ID is added to the Inventory report in step 10.

5. There is an EN in step 12 in clause “6.2.2 Inventory Procedure”.

Editor's note: The details how interactions between NG-RAN and AIOTF to indicate no further Inventory Reports will be sent and completion of the procedure need to be aligned with RAN.

Proposal 5: This EN can be removed according to SA2 agreements of the last report indication in the last meeting, see also step 13.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.369.

# * * * * First change (all new texts)***

6.2	AIoT Service Procedures

6.2.1	General

Clause 6.2.2 provides the procedure for AIoT Inventory. Clause 6.2.3 provides the procedure for AIoT Command.

6.2.2	Inventory Procedure

Figure 6.2.2-1 describes the inventory procedure.

The procedure focuses on the messages and parameters used for the communication between AIOTF and NG-RAN regardless of the path to access NG-RAN, see clause 4.2.2.1. The handling of the different communication paths is described in clause 6.2.4.

Figure 6.2.2-1: Inventory Procedure

1.	The AF invokes Nnef_AIoT_Inventory(AF ID, [External Target Area information], information about the target AIoT Device(s), [Approximate number of AIoT Devices], Approximate D2R inventory response message size) service operation request to the NEF.

The External Target Area information is specified in clause 5.3.

Information about the target AIoT Device(s) may include Filtering Information, as described in clause 5.8, or include complete AIoT Device Identifier(s).

The approximate number of AIoT Devices, if provided, is used to indicate the number of AIoT Devices expected to respond to this inventory service operation, which is sent by AIOTF to the NG-RAN in the assistance information for NG-RAN in step 7 for proper radio resource allocation.

The approximate D2R inventory response message size, if provided, is used to indicate the size of the inventory response from the AIoT device which is sent by AIOTF to the NG-RAN in the assistance information for NG-RAN in step 7.

2.	The NEF may further authorize the AF request as specified in clause 5.6.

The NEF determines the Target Area information, and selects one or multiple AIOTF(s) to handle the request as specified in clause 5.3.1.

3.	The NEF invokes the Naiotf_AIoT_Inventory(AF ID, [Target Area information], [information about the target AIoT Device(s)], [Approximate number of AIoT Devices]) service operation towards to the selected AIOTF(s).

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. The AIOTF may perform authorization as specified in clause 5.6. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT service operation request with an appropriate cause code, and step 7 onwards are skipped.

The AIOTF generates a Correlation ID corresponding to this AF service operation request.

The AIoT Identification Information to be provided to NG-RAN can include Filtering Information, as defined in clause 5.8, or a single AIoT Device Identifier.

AIOTF performs Reader Selection, see clause 5.3.3.

The AIOTF may also use the last serving Reader to assist with determining which Readers to use for an AFs request targeting for a specific AIoT Device.

The AIOTF determines assistance information as described in clause 5.4.

5.	AIOTF sends the AIoT Inventory Service Response to the NEF containing the accept or reject result for the AIoT Inventory service operation request based on step 4.

6.	NEF sends the AIoT service operation response to the AF, containing the accept or reject result for the AIoT Inventory service operation request as specified in clause 8.3.

7.	The AIOTF sends the Inventory Request message including the Correlation ID, the AIoT Identification Information to be included in the paging message, follow-on command indication, and assistance information to the selected NG-RAN.

8.	The NG-RAN sends an Inventory Response message (Correlation ID) to the AIOTF indicating that the Inventory Request is received successfully and will perform the service operation accordingly.

9.	Upon reception of the Inventory Request message from the AIOTF, the RAN Reader(s) will execute the inventory operation. The RAN Reader(s) broadcast the paging message that includes the AIoT Identification Information.

The AIoT Device determines whether it matches the AIoT Identification Information, as described in clause 5.8.

If an AIoT device matches the AIoT Identification Information in the paging message, the AIoT Device responds to the paging message and sends an AIOT NAS message that includes its AIoT Device ID.

Editor's note:	Whether and how the Device ID is concealed or encrypted will be determined and aligned with SA WG3.

10.	NG-RAN sends one or more Inventory Report messages to the AIOTF including the Correlation ID, Reader ID and a list of AIoT Device(s) related information, each AIoT Device related information include the AIOT NAS message(s) from the AIoT Device and the RAN NGAP Device ID. The RAN NGAP Device ID is allocated by the NG-RAN for the AIoT device and used to uniquely identify the AIoT device over NG interface between NG-RAN and AIOTF.

The AIOTF stores the mapping between the Reader ID and AIoT Device ID(s).

NOTE:	When to erase the stored mapping between the Reader ID and AIoT device ID(s) is up to implementation and local configuration.

11.	The AIOTF validates the results, using local stored device information or device profile data retrieved from the ADM. The AIOTF may aggregate the results.

12.	Optionally, if the NG-RAN detects that no more AIoT Devices will respond to the inventory procedure, the NG-RAN requests the AIOTF to release the operation with the cause no more AIoT device is detected  and report the inventory result if any). The AIOTF then confirm the NG-RAN the operation is release.

The AIOTF may request NG-RAN to release the operation without NG-RAN request. NG-RAN responses the operation is release to the AIOTF.

13.	The AIOTF reports the progress of the AIoT inventory request to the NEF by sending the Naiotf_AIoT_Notify message including a list of AIoT Device Permanent Identifier (s). The AIOTF may send multiple reports. The AIOTF in the final Naiotf_AIoT_Notify message indicates it is the last report for this operation. If multiple AIOTFs are involved in the procedure, the NEF may receive the AIoT_Notify from multiple AIOTFs.

14.	When receiving the Naiotf_AIoT_Notify message from AIOTF, the NEF informs the AF of the outcome of the AIoT_Inventory request by sending the Nnef_AIoT_Notify message(s) including the AIoT Device Permanent Identifier(s). The NEF in the final Nnef_AIoT_Notify message indicates that it is the last report for this operation.

# * * * * End of changes * * * *