SA WG2 Meeting #169	S2-2504768

19 – 23 May 2025, Fukuoka, Japan

Title:	New Solution on Sensing Request and Exposure Procedure

Source:	Apple

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution for FS_Sensing_ARC.

# 1.		Introduction

The call flow in clause 6.x.3 shows a high-level procedure where the names of the involved network entities are proposed to be generic but can be easily mapped to NFs in the 5GC. Details of the signalling in the solution are purposedly not provided to uphold the assumption of addressing only common aspects during Q2, but they are intended to be provided at later meetings once the SID scope update is clarified.

# 2.	Text Proposal

It is proposed to agree the following solution to be part of TR 23.700-14.

************ 1st change *************

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 | #6 |
| #1 |  |  |  |  |  |  |
| #2 |  |  |  |  |  |  |
| #x | X | X |  |  | X |  |

************ 2nd change (all new text) *************

## 6.x	Solution #X: Sensing Request and Exposure Procedure

This solution addresses KIs #1, #2, and #5.

### 6.x.1	High-level solution principles

The following high-level solution principles apply to this solution:

-	A high-level procedure for sensing request and exposure is proposed with a generic Sensing Entity (SE), i.e. with no assumptions on the supported sensing mode(s) by this solution.

-	The solution proposes a split of the sensing management and sensing processing functionality into two different Core Network entities, with the option of having the processing functionality at the SE.

-	The solution accounts for the Sensing Service Request (SSR) coming from a Sensing Service Consumer (SSC) that may or may not reside within a 3rd party, with involvement of the Exposure Function when the 3rd party is an untrusted AF.

-	The solution allows for the SE and SSC to be the same entity.

Editor's note:	The above high-level solution principles may be revised based on potential updates of the SID scope.

### 6.x.2	Procedures

Figure 6.x.2-1: Procedure for sensing request and exposure

Editor's note:	Details of each step in the procedure are FFS and will be provided once the SID scope update is clarified.

The following two alternatives are considered for the Sensing Service Request (SSR):

1.	[Alternative 1] The SSR is sent by a Sensing Service Consumer (SSC) to the Sensing Management Function (SeMF). The signaling path of the SSR from source to destination may involve additional entities (e.g. Mobility Function), which depend on the sensing mode.

2.	[Alternative 2] The SSR is sent by a SSC at a 3rd Party to an Exposure Function in the Core Network (if acting as untrusted AF), as shown in step 2. In that case, the Exposure Function forwards the request to the SeMF as shown in step 2.2.

NOTE 1:	If the 3rd Party SSC acts as trusted AF, the SSR is sent directly to the SeMF.

3.	The SeMF forwards the SSR for validation and authorization to a Sensing Policy and Authorization Function (SPAF). The validation functionality at SPAF checks if the SSR requirements can be fulfilled based on e.g. availability of resources, Sensing Entities, etc., and the authorization functionality at SPAF checks if the SSR is allowed to be performed based on the request parameters, e.g. sensing mode, sensing location, sensing time, etc.

NOTE 2:	The SPAF may be a standalone NF or co-located with another Policy Function.

4.	[Optional] In case the SSR requires the processing of Sensing Data in the Core Network, the SeMF may send a Sensing Processing configuration request to the Sensing Processing Function (SPF) including information relevant to the processing of Sensing Data for the received SSR.

5.	The SeMF sends a configuration request to the Sensing Entity (SE) to configure one or more SE’s. It shall be noted that the SSC and the SE might be the same entity.

The following two alternatives are considered for the SSR response:

6.	[Alternative 1] If step 1 was performed, an SSR response is sent by the SeMF to the SSC. The SSR response contains an ACK if steps 4 and 5 were successful, otherwise the SSR response contains a NACK, and the procedure stops at this step.

7.	[Alternative 2] If step 2 was performed, an SSR response is sent by the SeMF to the 3rd Party SSC via the Exposure Function if the 3rd Party SSC acts as untrusted AF, as shown in step 7.1. The SSR response contains an ACK if steps 4 and 5 were successful, otherwise the SSR response contains a NACK, and the procedure stops at step 7.2.

NOTE 3:	If the 3rd Party SSC acts as trusted AF, the SSR response is sent directly to the 3rd Party SSC.

The actual sensing by the SE is performed at this stage.

The following two alternatives are considered for processing of the Sensing Data:

8.	[Alternative 1] If Sensing Data is available at the SE and the Sensing Data is to be processed in the CN, the SE provides the Sensing Data to the SPF for processing.

9.	[Alternative 2] If the Sensing Data is not to be processed in the CN, the SE may itself process the Sensing Data. After processing, the Sensing Result will be provided by the SE to the SSC. Note that the SE and the SSC may also be the same entity.

10. [Optional]	 If step 8 is performed, the SPF has to process Sensing Data to Sensing Results. The term Processing is used as a general term which may include different kind of compute and inference such as Fusion, AIML, Rule Based Processing. Processing may include several (e.g. iterative, parallel) steps to compute the Sensing Result.

The following two alternatives are considered for exposing the sensing result:

11.	[Alternative 1] If step one was performed, the Sensing Result is provided from the SPF to the SSC. The signaling path to provide the Sensing Result from source to destination may involve additional entities (e.g. Mobility Function), which depend on the sensing mode.

12.	[Alternative 2] If step 2 was performed, the SPF provides the Sensing Result to the 3rd Party SSC via the Exposure Function if the 3rd Party SSC acts as untrusted AF, as shown in step 12.1. In that case, the Exposure Function forwards the Sensing Result to the 3rd Party SSC as shown in step 12.2.

NOTE 4:	If the 3rd Party SSC acts as trusted AF, the Sensing Result is provided directly to the 3rd Party SSC.

### 6.x.3	Impacts on services, entities and interfaces

SeMF:

-	Receives and responds to Sensing Service Request.

-	Provides sensing validation and authorization request to SPAF.

-	Provides sensing configuration request to the Sensing Entity.

SPF:

-	Supports the processing of Sensing Data and generation of Sensing Result upon request from the Sensing Entity.

-	Receives sensing processing configuration request from SeMF.

SPAF:

-	Supports validation of the Sensing Service Request.

-	Supports authorization of the Sensing Service Request.

Editor's note:	Impacts clause will be updated once the SID scope update is clarified.

************ End of changes **************