3GPP SA WG2 Meeting #169	S2-2504785

# Fukuoka, Japan, 19 – 23 May 2025										            (revision of S2-2503282)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6108 | rev | 2 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 23.501 AIML_CN SCP related alignment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SCP services has been introduced, however, there are some misalignment in TS 23.501 as below: - NOTE that SCP does not expose service is incorrect. - Service-based interface is not added. - Reference point is not added. - Reference to TS 23.502 is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Alignment with new defined SCP services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.3, 4.2.6, 4.2.7, 7.2.30 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S2-2501761 is the original verison which is not handled in the meeting. S2-2503282 is the r1 with changes on 4.2.7. S2-250xxxx is the r1 wihout any changes which is not handled in the last meeting. |  |  |  |  |  |  |  |  |

# * * * * 1st change* * * *

4.2.3	Non-roaming reference architecture

Figure 4.2.3-1 depicts the non-roaming reference architecture. Service-based interfaces are used within the Control Plane.

Figure 4.2.3-1: Non-Roaming 5G System Architecture

NOTE:	If an SCP is deployed it can be used for indirect communication between NFs and NF services as described in Annex E.

# * * * * 2nd change * * * *

### 4.2.6	Service-based interfaces

The 5G System Architecture contains the following service-based interfaces:

Namf:	Service-based interface exhibited by AMF.

Nsmf:	Service-based interface exhibited by SMF.

Nnef:	Service-based interface exhibited by NEF.

Npcf:	Service-based interface exhibited by PCF.

Nudm:	Service-based interface exhibited by UDM.

Naf:	Service-based interface exhibited by AF.

Nnrf:	Service-based interface exhibited by NRF.

Nnsacf:	Service-based interface exhibited by NSACF.

Nnssaaf:	Service-based interface exhibited by NSSAAF.

Nnssf:	Service-based interface exhibited by NSSF.

Nausf:	Service-based interface exhibited by AUSF.

Nudr:	Service-based interface exhibited by UDR.

Nudsf:	Service-based interface exhibited by UDSF.

N5g-eir:	Service-based interface exhibited by 5G-EIR.

Nnwdaf:	Service-based interface exhibited by NWDAF.

Nchf:	Service-based interface exhibited by CHF.

Nucmf:	Service-based interface exhibited by UCMF.

Ndccf:	Service based interface exhibited by DCCF.

Nmfaf:	Service based interface exhibited by MFAF.

Nadrf:	Service based interface exhibited by ADRF.

Naanf:	Service-based interface exhibited by AANF.

NOTE 1:	The Service-based interface exhibited by AANF is defined in TS 33.535 [124].

N5g-ddnmf:	Service-based interface exhibited by 5G DDNMF.

Nmbsmf:	Service-based interface exhibited by MB-SMF.

Nmbsf:	Service-based interface exhibited by MBSF.

NOTE 2:	The Service-based interfaces exhibited by MB-SMF and MBSF are defined in TS 23.247 [129].

Ntsctsf:	Service-based interface exhibited by TSCTSF.

Nbsp:	Service-based interface exhibited by an SBI capable Boostrapping Server Function in GBA.

NOTE 2:	The Service-based interfaces exhibited by an SBI capable Boostrapping Server Function are defined in TS 33.220 [140] and TS 33.223 [141].

Neasdf:	Service-based interface exhibited by EASDF.

NOTE 3:	The Service-based interfaces exhibited by EADSF is defined in TS 23.548 [130].

Nupf:	Service-based interface exhibited by UPF.

Neif:	Service-based interface exhibited by EIF.

Nscp:	Service-based interface exhibited by SCP.

# * * * * 3rd change * * * *

### 4.2.7	Reference points

The 5G System Architecture contains the following reference points:

N1:	Reference point between the UE and the AMF.

N2:	Reference point between the (R)AN and the AMF.

N3:	Reference point between the (R)AN and the UPF.

N4:	Reference point between the SMF and the UPF.

N6:	Reference point between the UPF and a Data Network.

N9:	Reference point between two UPFs.

The following reference points show the interactions that exist between the NF services in the NFs. These reference points are realized by corresponding NF service-based interfaces and by specifying the identified consumer and producer NF service as well as their interaction in order to realize a particular system procedure.

N5:	Reference point between the PCF and an AF or TSN AF.

N7:	Reference point between the SMF and the PCF.

N8:	Reference point between the UDM and the AMF.

N10:	Reference point between the UDM and the SMF.

N11:	Reference point between the AMF and the SMF.

N12:	Reference point between AMF and AUSF.

N13:	Reference point between the UDM and Authentication Server function the AUSF.

N14:	Reference point between two AMFs.

N15:	Reference point between the PCF and the AMF in the case of non-roaming scenario, PCF in the visited network and AMF in the case of roaming scenario.

N16:	Reference point between two SMFs, (in roaming case between SMF in the visited network and the SMF in the home network).

N16a:	Reference point between SMF and I-SMF.

N17:	Reference point between AMF and 5G-EIR.

N18:	Reference point between any NF and UDSF.

N19:	Reference point between two PSA UPFs for 5G LAN-type service.

N22:	Reference point between AMF and NSSF.

N23:	Reference point between PCF and NWDAF.

N24:	Reference point between the PCF in the visited network and the PCF in the home network.

N27:	Reference point between NRF in the visited network and the NRF in the home network.

N28:	Reference point between PCF and CHF.

N29:	Reference point between NEF and SMF.

N30:	Reference point between PCF and NEF.

NOTE 1:	The functionality of N28 and N29 and N30 reference points are defined in TS 23.503 [45].

N31:	Reference point between the NSSF in the visited network and the NSSF in the home network.

NOTE 2:	In some cases, a couple of NFs may need to be associated with each other to serve a UE.

N32:	Reference point between a SEPP in one PLMN or SNPN and a SEPP in another PLMN or SNPN; or between a SEPP in a SNPN and a SEPP in a CH/DCS, where the CH/DCS contains a UDM/AUSF.

NOTE 3:	The functionality of N32 reference point is defined in TS 33.501 [29].

N33:	Reference point between NEF and AF.

N34:	Reference point between NSSF and NWDAF.

N35:	Reference point between UDM and UDR.

N36:	Reference point between PCF and UDR.

N37:	Reference point between NEF and UDR.

N38:	Reference point between I-SMFs and between V-SMFs.

N40:	Reference point between SMF and the CHF.

N41:	Reference point between AMF and CHF in HPLMN.

N42:	Reference point between AMF and CHF in VPLMN.

NOTE 4:	The functionality of N40, N41 and N42 reference points are defined in TS 32.240 [41].

N43:	Reference point between PCFs.

NOTE 5:	The functionality of N43 reference point is defined in TS 23.503 [45].

NOTE 6:	The reference points from N44 up to and including N49 are reserved for allocation and definition in TS 32.240 [41].

N50:	Reference point between AMF and the CBCF.

N51:	Reference point between AMF and NEF.

N52:	Reference point between NEF and UDM.

N55:	Reference point between AMF and the UCMF.

N56:	Reference point between NEF and the UCMF.

N57:	Reference point between AF and the UCMF.

NOTE 7:	The Public Warning System functionality of N50 reference point is defined in TS 23.041 [46].

N58:	Reference point between AMF and the NSSAAF.

N59:	Reference point between UDM and the NSSAAF.

N60:	Reference point between AUSF and NSWOF.

NOTE 8:	The functionality of N60 reference point is defined in TS 33.501 [29].

N80:	Reference point between AMF and NSACF.

N81:	Reference point between SMF and NSACF.

N82:	Reference point between NSACF and NEF.

N83:	Reference point between AUSF and NSSAAF.

N84:	Reference point between TSCTSF and PCF.

N85:	Reference point between TSCTSF and NEF.

N86:	Reference point between TSCTSF and AF.

N87:	Reference point between TSCTSF and UDM.

N88:	Reference point between SMF and EASDF.

N88a:	Reference point between I-SMF and EASDF.

N89:	Reference point between TSCTSF and AMF.

N96:	Reference point between TSCTSF and NRF.

N97:	Reference point between two NSACFs in different PLMNs.

N99:	Reference point between two NSACFs within the same PLMN.

N110:	Reference point between EIF and AF.

N111:	Reference point between EIF and NEF.

N112:	Reference point between EIF and UDM.

N113:	Reference point between EIF and PCF.

N114:	Reference point between EIF and SMF.

N115:	Reference point between SCP and NWDAF.

N116:	Reference point between SCP and DCCF.

NOTE 9:	The reference points from N90 up to and including N95 are reserved for allocation and definition in TS 23.503 [45].

NOTE 10:	The reference points from N100 up to and including N109 are reserved for allocation and definition in TS 32.240 [41].

The reference points to support SMS over NAS are listed in clause 4.4.2.2.

The reference points to support Location Services are listed in TS 23.273 [87].

The reference points to support SBA in IMS (N5, N70 and N71) are described in TS 23.228 [15].

The reference points to support AKMA (N61, N62 and N63) are described in TS 33.535 [124].

The reference points to support 5G ProSe are described in TS 23.304 [128].

The reference points to support 5G multicast-broadcast services are described in TS 23.247 [129].

The reference points to Support Uncrewed Aerial Systems (UAS) connectivity, identification and tracking are described in TS 23.256 [136].

The reference points to support SBA in GBA and GBA push (N65, N66, N67 and N68) are described in TS 33.220 [140] and TS 33.223 [141].

The reference points to support SMS delivery using SBA are described in TS 23.540 [142].

The reference points to support Ranging based services and Sidelink Positioning are described in TS 23.586 [180].

# * * * * 4th change * * * *

### 7.2.30	SCP Services

The following NF service is specified for SCP.

Table 7.2.30-1: NF Services provided by SCP

| Service Name | Description | Reference in TS 23.502 [3] |
| --- | --- | --- |
| Nscp_EventExposure | This service exposes SCP related information to the consumer NFs. | 5.2.29.2 |

# * * * * End of changes * * * *