3GPP TSG-WG SA2 Meeting #169	S2-2504796

Fukuoka, Japan, 19th – 23rd May, 2025	(revision of S2-250xxxx)

Source:	Nokia

Title:	AIoT assistance information update

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-Arc / Rel-19

Abstract: Clause 5.4 is updated.

# 1. Introduction/Discussion

This pCR proposes to update the assistance information description for the following three aspects:

- It is clarified that the Assistance information is determined by the AIOTF.

- Based on S2-2504523, the “follow-on command indication” is included as part of the assistance information, replacing the AIoT service type information..

- Based on S2-2504517, the term “optionally” has been removed for the approximated D2R message size. However, the approximate number of AIoT devices remains optional, as it is not mandated by RAN2 and is sometimes not feasible to estimate—particularly when a large area is targeted.

# 2. Text Proposal

It is proposed to capture the following changes in TS 23.369.

# * * * * First change * * * *

## 5.4	Assistance information provided to NG-RAN node

The AIOTF determines and provides the following assistance information to the NG-RAN together with the service operation requests:

a)	Follow-on command indication, indicating whether an inventory will be followed by a command

b)	Optionally, approximate number of AIoT devices based on AF request.

c)	approximate D2R message size based on AF request.

d)	Optionally, time interval as AIoT aggregation assistance information.

Editor's note:	Other assistance information may be added later if necessary.

Editor's note:	It is FFS where detailed description on time interval as AIoT aggregation assistance information will be captured, i.e. in this clause or in another clause.

If not provided by the AF, bullets b) and c) in the above assistance information provided by the AIOTF may be based on local configuration based on SLA between the AIoT service provider represented by an AF and the operator.

If the AF has provided a time interval, then the AIOTF should set bullet d) to NG-RAN that is equal or shorter than the time interval received from the AF.

NOTE:	Based on local configuration, the AIOTF can reject the AF request, e.g. if the AF provided time interval is shorter than a locally configured minimum interval.

The assistance information is used by the NG-RAN for performing service operations, e.g. radio resource allocation by using bullets b) and c), AIoT Device responses aggregation by using bullet d).

# * * * * End of changes * * * *