3GPP TSG-WG2 Meeting #169	S2-2504862

# Fukuoka, Japan, May 19 – 23, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6194 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Indication of Rate-Adaptable QoS Flows to NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_Ph3-Core |  |  |  |  |  | Date: |  |  | 2024-05-08 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In LS R2-2501347, RAN2 requested that for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify that 5GC may provide an indication of rate-adapatable QoS flows to NG-RAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Indication from CN on which UL flows are rate adaptable is not available to RAN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.1, 5.37.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS23.503 CR1535, TS23.502 CR5419 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### 5.4.4b	UE 5GSM Core Network Capability handling

The UE 5GSM Core Network Capability is included in PDU Session Establishment/Modification Request.

The UE shall indicate in the UE 5GSM Core Network Capability whether the UE supports:

-	"Ethernet" PDU Session Type supported in EPC as PDN Type "Ethernet";

-	Reflective QoS;

-	Multi-homed IPv6 PDU Session (only if the Requested PDU Type was set to "IPv6" or "IPv4v6");

-	ATSSS capability (as referred to clause 5.32.2);

-	Transfer of Port Management Information containers;

-	Support for secondary DN authentication and authorization over EPC (as referred to clause 5.17.2.5);

-	(S)RTP Multiplexed Media Identification Information in IP Packet Filters.

-	Rate-adaptable QoS flows (as referred to clause 5.37.X).

The 5GSM Core Network Capability is transferred, if needed, from V-SMF to H-SMF during PDU Session Establishment/Modification procedure.

After the first inter-system change from EPS to 5GS for a PDU session established in EPS, the 5GSM Core Network Capability is also included in the PDU Session Modification if the Reflective QoS and/or Multi-homed IPv6 PDU Session is present.

**** Second Change ****

### 5.37.1	General

This clause provides an overview of 5GS functionalities for support of XR services (AR/VR applications) and interactive media services that require high data rate and low latency communication, e.g. cloud gaming and tactile/multi-modal communication services according to service requirements documented in TS 22.261 [2]. The standardized 5QI characteristics for such interactive services are provided in Table 5.7.4-1 and TSCAI is used to describe the related traffic characteristics as defined in clause 5.27.2. Further enhancements for these interactive media services are as follows:

-	The 5GS may support QoS policy control for multi-modal traffic, see clause 5.37.2.

-	The 5GS may support network information exposure which can be based on ECN markings for L4S, see clause 5.37.3 or 5GS exposure API, see clause 5.37.4.

-	The 5GS may support PDU Set based QoS handling including PDU Set identification and marking, see clause 5.37.5.

-	The 5GS may ensure that the UL and DL packets together meet the requested round trip delay and also update the delay for UL and DL considering QoS monitoring results, see clause 5.37.6.

-	The 5GS may perform per-flow Packet Delay Variation (PDV) monitoring and policy control according to AF provided requirements, see clause 5.37.7.

-	The 5GC may provide traffic assistance information to the NG-RAN to enable Connected mode DRX power saving, see clause 5.37.8.

-	The 5GS may consider dynamically changed traffic characteristics for better resource management, see clause 5.37.10.

-	The 5GS may support traffic identification for multiplexed media flows in the same transport layer connection, see clause 5.37.11.

-	The 5GC may perform PDU Set Importance based transport level packet marking, see clause 5.8.2.7.

-	The 5GC may provide the Multi-modal Service ID to NG-RAN, see clause 5.37.2.

-	The 5GC may provide an indication to the NG-RAN that a QoS flow may be subject to rate adaptation, see clause 5.37.X.

NOTE:	Home-routed roaming deployments cannot always support the low latency communication requirements of XR services or interactive media services.

**** Third Change ****

### 5.37.X	Support of rate-adaptable traffic

XR and interactive media services that require high data rate and low latency communication, can send data traffic that is subject to rate adaptation.

In order to enable rate adaptation based on RAN bit rate recommendation (see TS 38.300 [27]), the AF may provide, within the AF session with QoS procedure, an indication that the media flow(s) may be subject to rate adaptation. The PCF, based on the AF input or operator policies, may include an indication within the PCC rule that the service data flow(s) may be subject to rate adaptation; and the SMF may forward the indication to NG-RAN in N2 SM information when establishing and/or updating the corresponding QoS Flows.

The UE may indicate support of rate-adaptable QoS flows to the SMF at PDU session establishment and the SMF may provide this UE capability information to the PCF. The PCF considers such UE capability information when providing the PCC rules to the SMF.

When the AF request includes the indication of media flows subject to rate adaptation and the UE has not indicated support for rate-adaptable QoS flows, the PCF notifies the AF that rate adaptation based on RAN bit rate recommendation cannot be applied.

**** End of Changes ****