SA WG2 Meeting #169	S2-2504875

19 – 23 May, 2025, Fukuoka, Japan

Source:	Ericsson

Title:	(KI#2) Solution based on analysis of abnormal user plane traffic

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 /Rel-20

Abstract of the contribution: This contribution proposes a new solution for KI#2 for FS_AIML_CN_Ph2.

# 1. Discussion

This PCR proposes a new solution for KI#2 based on analysis of abnormal user plane traffic.

# 2. Text Proposal

It is proposed to capture the following changes into TR 23.700-04.

*** Start of changes ***

## 6.0	Mapping of Solutions to Key Issues and Use Cases

Table 6.0-1: Mapping of Solutions to Key Issues and use cases

|  | Key Issues |  | Use Cases (optional) |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Use Case #1> | <Use Case #2> |
| #1 | x |  |  |  |
| #2 | x |  |  |  |
| #3 | x |  |  |  |
| #4 | x |  |  |  |
| #X |  | x | x |  |

*** Next change (all new text) ***

## 6.X	Solution #X: Analysis of abnormal user plane traffic

### 6.X.1	High level principles

These are the high-level principles of the proposed solution:

- A new AnalyticsID “Abnormal user plane traffic” is proposed.

  - Input data retrieved from UPF on user plane traffic parameters as defined in Table 6.X.2.2-1.

  - Output, a list of exceptions and the list of SUPIs and/or Application ID(s) affected per exception in the list, as defined in Table 6.X.2.3-1 and Table 6.X.2.3-2. The list of exceptions are: too much data sent/received and/or too high or low rate, abnormal traffic bursts, too many malformed packets, too many duplicate packets, too many unknown packets, too many retransmitted packets and/or too many fragmented packets.

- The analytics consumer may be SMF or PCF.

- The consumer may request the type of exception is interested in when subscribing to this new AnalyticsID. The consumer takes actions based on the type of exception reported by NWDAF, such as blocking traffic reported as abnormal.

- Assumption: To identify abnormal user plane traffic, NWDAF may provide a threshold to the UPF when retrieving the input data, then any traffic that do not reach the threshold is not considered “abnormal” traffic by NWDAF and by UPF.

### 6.X.2	Description

This solution proposes a 5GC enhancement to support analysis of abnormal user plane traffic, specifically a new AnalyticsID related to abnormal user plane traffic.

#### 6.X.2.1	 Abnormal user plane traffic AnalyticsID

The consumer of this AnalyticsID shall indicate in the request:

-	Analytics ID = "Abnormal user plane traffic";

-	Target of Analytics Reporting can be a list of SUPIs, a group of SUPIs or any UE;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	Analytics Filter Information optionally including:

-	list of Exception ID, where each exception ID refers to a traffic data anomaly of interest (too much data and/or rate, abnormal traffic bursts, too many malformed packets, too many duplicate packets, too many unknown packets, too many retransmitted packets and/or too many fragmented packets);

- 	Optionally, when analytics type refers to abnormal traffic bursts, information relative to the definition of a traffic burst;

-	Optionally, associated thresholds (for each Exception Level);

-	Area of interest;

-	Application ID;

-	DNN;

-	S-NSSAI.

#### 6.X.2.2 	Input Data

NWDAF triggers data collection from UPF to retrieve user plane traffic parameters. The detailed input data is described in Table 6.X.2.2-1.

Table 6.X.2.2-1: Input data

| Information | Source | Description |
| --- | --- | --- |
| Application ID | UPF | Identifies the application. |
| Application Traffic Flow Information (1..max) | UPF | Per Application Traffic flow related information for the application and optionally, for a DNN, S-NSSAI combination. |
| > IP 5-tuple | UPF | Identifies a service flow of the UE that uses the application. |
| > Start time | UPF | Start time of traffic detection for the flow. |
| > End time | UPF | End time of traffic detection for the flow. |
| > UL Data volume | UPF | Measured UL data traffic volume for the flow. |
| > DL Data volume | UPF | Measured DL data traffic volume for the flow. |
| > UL Data Rate | UPF | Measured UL data rate for the flow. |
| > DL Data Rate | UPF | Measured DL data rate for the flow. |
| > URL list | UPF | List of URLs extracted from the inspected user plane packets in the flow. |
| > Domain Name list | UPF | List of domain names extracted from the inspected user plane packets in the flow. |
| > UL Traffic Bursts | UPF | Number of detected UL traffic bursts in the flow. |
| > DL Traffic Bursts | UPF | Number of detected DL traffic bursts in the flow. |
| > UL Malformed packets | UPF | Number of UL malformed packets in the flow. |
| > DL Malformed packets | UPF | Number of DL malformed packets in the flow. |
| > UL Duplicate packets | UPF | Number of UL duplicate packets in the flow. |
| > DL Duplicate packets | UPF | Number of DL duplicate packets in the flow. |
| > UL Unknown packets | UPF | Number of UL unknown packets in the flow. |
| > DL Unknown packets | UPF | Number of DL unknown packets in the flow. |
| > UL Retransmitted packets | UPF | Number of UL retransmitted packets in the flow. |
| > DL Retransmitted packets | UPF | Number of DL retransmitted packets in the flow. |
| > UL Fragmented packets | UPF | Number of UL fragmented packets in the flow. |
| > DL Fragmented packets | UPF | Number of DL fragmented packets in the flow. |

NWDAF also triggers data collection (e.g. from OAM, NRF, UPF) to retrieve information relative to UPF load via existing mechanisms.

#### 6.X.2.3	 Output Analytics

Corresponding to the "Abnormal user plane traffic" Analytics ID, the analytics result provided by the NWDAF is defined in Table 6.X.2.3-1 and Table 6.X.2.3-2. When the level of an exception passes above or below the threshold, the NWDAF shall notify the consumer with the exception ID associated with the exception if the exception ID is within the list of exception IDs indicated by the consumer or matches the expected analytics type indicated by the consumer. The NWDAF shall provide the Exception Level and determine which of the other information elements to provide, depending on the observed exception.

Abnormal user plane traffic statistics information is defined in Table 6.X.2.3-1.

Table 6.X.2.3-1: Abnormal user plane traffic statistics

| Information | Description |
| --- | --- |
| Exceptions (1..max) | List of observed exceptions |
| > Exception ID | The risk detected by NWDAF |
| > Exception Level | Scalar value indicating the severity of the abnormal UP traffic |
| > Exception trend | Measured trend (up/down/unknown/stable) |
| > SUPI list (1..SUPImax) | SUPI(s) of the UE(s) affected with the Exception. When the consumer requests the analytics for “any UE”, this information is not present. |
| > Application ID list (1..AppIDmax) | Application ID(s) affected with the Exception |
| > Ratio | Estimated percentage of UEs affected by the Exception within the Target of Analytics Reporting |
| > Amount | Estimated number of UEs affected by the Exception (applicable when the Target of Analytics Reporting = "any UE") |
| > Additional information | Specific information for each risk |

Abnormal user plane traffic predictions information is defined in Table 6.X.2.3-2.

Table 6.X.2.3-2: Abnormal user plane traffic predictions

| Information | Description |
| --- | --- |
| Exceptions (1..max) | List of predicted exceptions |
| > Exception ID | The risk detected by NWDAF |
| > Exception Level | Scalar value indicating the severity of the abnormal behaviour |
| > Exception trend | Measured trend (up/down/unknown/stable) |
| > SUPI list (1..SUPImax) | SUPI(s) of the UE(s) affected with the Exception. When the consumer requests the analytics for “any UE”, this information is not present. |
| > Application ID list (1..AppIDmax) | Application ID(s) affected with the Exception |
| > Ratio | Estimated percentage of UEs affected by the Exception within the Target of Analytics Reporting |
| > Amount | Estimated number of UEs affected by the Exception (applicable when the Target of Analytics Reporting = "any UE") |
| > Additional information | Specific information for each risk |
| > Confidence | Confidence of this prediction |

Exception ID can be: too much data and/or rate, abnormal traffic bursts, too many malformed packets, too many duplicate packets, too many unknown packets, too many retransmitted packets or too many fragmented packets.

Regarding Additional information, it includes specific information for each risk, e.g. to indicate, for the Exception ID, for which server IP address(es) it applies to. It might also indicate if the exception ID has any significant impact on the UPF load.

#### 6.X.2.4		 Procedure for AnalyticsID on abnormal user plane traffic

Figure 6.X.2.4-1 depicts the procedure for the proposed solution.

Figure 6.X.4-1: Procedure for analytics on abnormal user plane traffic

- Consumer (e.g. PCF, SMF) subscribes to NWDAF analytics on abnormal user plane traffic, by triggering Nnwdaf_AnalyticsSubscription_Subscribe request message including as analytics identifier (“Abnormal user plane traffic”), including as input the user plane traffic data anomalies of interest (too much data and/or rate, abnormal traffic bursts, too many malformed packets, too many duplicate packets, too many unknown packets, too many retransmitted packets and/or too many fragmented packets), and for each, optionally, a threshold (e.g. in case the number and/or percentage of malformed packets is above the threshold, this needs to be provided as analytic output to the consumer), as analytic filter the DNN, S-NSSAI and/or Area of Interest, as analytic target a list of SUPIs, a group of SUPIs or any UE, and this either for the traffic of the whole PDU session or for certain application identifier(s).

- NWDAF triggers data collection (e.g. from OAM, NRF, UPF) to retrieve information relative to UPF load via existing mechanisms.

- NWDAF triggers data collection from UPF to retrieve user plane traffic parameters, by triggering a Nupf_EventExposure_Subscribe request message, including as eventType="USER_DATA_USAGE_MEASURES", a new measurementType="TRAFFIC_DATA _ANOMALIES" and a list of traffic data anomalies of interest (volume and/or rate, traffic bursts, malformed packets, duplicate packets, unknown packets, retransmitted packets and/or fragmented packets), and for each a threshold, e.g. to report only when the percentage of fragmented packets goes over a certain threshold (when included in step 1 above or locally provisioned at NWDAF; when not included, it is assumed the threshold is locally provisioned at UPF), and the target SUPI(s) and application identifier(s). Optionally, when traffic data of interest refers to traffic bursts, information relative to the definition of a traffic burst.

NOTE1: It is assumed both UPF event subscription and reporting applies per SUPI (and application identifier), but optimizations (to minimize signaling) as discussed by CT4 can also be applied.

- UPF collects data for the above event, specifically the UP traffic parameters of interest: metrics or KPIs like volume, data rate, etc and/or packet parameters like 5-tuple(s), SNI(s), domain name(s), etc and/or classification related information, if available, like application identifier(s), traffic burst(s) and/or malformed, duplicate, unknown, retransmitted and/or fragmented packets.

- UPF notifies NWDAF by triggering a Nupf_EventExposure_Notify request message, including the eventType="USER_DATA_USAGE_MEASURES" and the event information, specifically the UP traffic parameters indicated above (when the corresponding thresholds are surpassed).

- NWDAF runs analytics based on collected data and derives the analytics result, specifically a list of Exception IDs corresponding to abnormal UP traffic detected, e.g. when the number and/or percentage of fragmented packets is over the threshold, indicating for which SUPI(s), application identifier(s) and/or server IP addresses it applies to. NWDAF can also determine and indicate to the consumer if and how the abnormal UP traffic has any impact on the UPF load.

- NWDAF notifies the analytic consumer (e.g. PCF, SMF) by triggering Nnwdaf_AnalyticsSubscription_Notify request message including the analytics identifier (“Abnormal user plane Traffic”) and the analytics result. Based on the analytics result, the consumer can apply actions to improve user plane performance, e.g. to avoid abnormal traffic, by PCF setting policies (via SMF) based on existing policy framework, e.g. for UPF to block traffic from/to servers (server IP addresses) from which abnormal behavior has been detected (e.g. too much packet fragmentation) or SMF to trigger UPF re-selection for PDU Sessions for the SUPIs from which abnormal behavior has been detected.

### 6.X.3	Impacts on services, entities and interfaces

NWDAF:

- New analyticsID relative to abnormal user plane traffic.

UPF:

- Extension of the existing USER_DATA_USAGE_MEASURES event with a new measurement type (TRAFFIC_DATA_ANOMALIES) which allows UPF to report information related with traffic data anomalies, specifically traffic bursts, malformed packets, duplicate packets, unknown packets, retransmitted packets and/or fragmented packets.

SMF, PCF:

- Act as consumer of the new analyticsID relative to abnormal user plane traffic, and based on the received analytics result, to apply actions, e.g. based on existing policy framework, PCF setting policies (via SMF) for UPF to block traffic from/to servers (server IP addresses) from which abnormal behavior has been detected (e.g. too much packet fragmentation) or SMF to trigger UPF re-selection for PDU Sessions for the SUPIs from which abnormal behavior has been detected.

*** End of changes ***