3GPP TSG-WG SA2 Meeting #169	S2-2504886

Fukuoka, Japan, May 19 - 23, 2025	(revision of S2-250xxxx)

Source:	Ericsson

Title:	Clause 6.2.3: Update on Command Procedure

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: This pCR updates the command procedure to align with RAN3 on the NGAP updates, i.e., introducing Follow-on Command Indication and RAN NGAP Device ID.

1. Introduction

In LS from RAN3 (R3-252481), RAN3 has introduced “follow-on command indication” and “RAN NGAP Device ID”:

On Command after Inventory:

- Introduce a “follow-on command indication” in the AIOTF information container in the Inventory Request message.

- Working assumption: The NGAP Command Request procedure concerns a single device only.

- Within all Command related NGAP messages:

  - The AIOTF Identifier is included outside the AIOTF information container.

  - Working assumption: The Correlation identifier is included inside and outside the AIOTF information container.

- RAN3 agreed to introduce per-session per-device device context (“RAN NGAP Device ID”).

  - Upon receiving the “follow-on command indication”, the gNB allocates for each device a “RAN NGAP Device ID” and provides it in the AIOTF information container in the Inventory Report message.

  - The “RAN NGAP Device ID” is included in the AIOTF information container of each Command related NGAP message.

In Inventory Request towards NG-RAN, the “follow-on command indication” is introduced:

- When the AIOTF receives an Inventory Request from the AF (possibly via the NEF), the indication is not set.

- When the AIOTF receives a Command Request from the AF (possibly via the NEF), the indication is set to indicate there is a following command round after the inventory round.

In the Inventory Report from the NG-RAN, if there is a following command round after the inventory round, there is a “RAN NGAP Device ID” per AIoT device, which is allocated by the NG-RAN. The AIOTF includes the “RAN NGAP Device ID” in the following Command Request. The NG-RAN includes it in the Command Response as well.

[Proposal-1] Update the Command Procedure for the introduction of “follow-on command indication” and “RAN NGAP Device ID”.

In the command procedure, there is an EN about the parameter of identifying the target AIoT Device in NG-RAN:

Editor's note:	What parameter(s) are used in the Command Request to enable NG-RAN node to target a specific AIoT Device requires coordination with RAN3.

Based on the LS, the RAN NGAP Device ID is the parameter in Command Request to enable NG-RAN to identify the target AIoT Device. It is also used by the AIOTF to identify the AIoT device context in the Command Response.

[Proposal-2] The EN can be resolved.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 23.369 v0.3.0:

* * * Start of Change * * * *

### 6.2.3	Command Procedure

Editor's note:	Additional information in the steps, parameters, and their naming throughout the procedures requires alignment with other clauses and references adding as required.

Editor's note:	Alignment is required for how to document and describe Direct Connectivity and Indirect Connectivity options in the procedures.

Figure 6.2.3-1 depicts the command procedure.

The procedure focuses on the messages and parameters used for the communication between AIOTF and NG-RAN regardless of the path to access NG-RAN, see clause 4.2.2.1. The handling of the different communication paths is described in clause 6.2.4.

Figure 6.2.3-1: Command Procedure

1.	The AF sends the Nnef_AIoT_Command (in case of untrusted AF) Request (AF ID, Command Type, AIoT Device identification information, [External Target Area information], [Approximate number of AIoT Devices], [Approximate D2R message size], AIoT data) message to NEF.

Editor's note:	It is FFS whether and how to structure the AIoT data if the Command Type is Read, Write or Disable.

The AIoT Device identification information may include one or more AIoT Device ID(s) or the filtering information which is used to associate with multiple AIoT devices.

2.	The NEF selects the AIOTF(s) as described in clause 5.3.1. If no AIOTF can be selected, the NEF rejects the AIoT Command request with an appropriate cause code and step 6 is performed before ending the procedure.

3.	The NEF sends Naiotf_AIoT_Command Request message (AF ID, Command Type, AIoT Device identification information, [Target area information], , [Approximate number of AIoT Devices], [Approximate D2R message size], AIoT data) message to the selected AIOTF.

4.	The AIOTF receives the AIoT command operation request and checks the parameters included in the request. The AIOTF performs NG- RAN and optionally RAN Reader selection as specified in clause 5.3. If no NG-RAN or RAN Reader can be selected, the AIOTF rejects the AIoT Command request with an appropriate cause code.

The AIOTF generates a Correlation ID corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request.

The AIOTF determines assistance information as described in clause 5.4.

The AIOTF performs AF authorization for AIoT command operation as described in clause 5.6.

The AIOTF performs AMF selection as described in clause 5.3.4.

5.	AIOTF sends the Naiotf_AIoT_Command Response message (accept or reject, [cause code]) to the NEF.

6.	NEF sends the Nnef_AIoT_Command Response message (accept or reject, [cause code]) to the AF. If the response was a reject the procedure stops here.

7.	Step 7 to step11 of procedure for Inventory specified in clause 6.2.2 are performed with the following clarifications:

-	In step 7, the AIOTF sets the "follow-on command indication" to inform the NG-RAN there is a following command delivery after the inventory.

-	In step 10, the AIOTF receives the "RAN NGAP Device ID" for each AIoT device in the Inventory Report.

-	In step 11, the AIOTF validates the results as specified in TS 33.369 [9], and determines whether the command should be sent to an AIoT Device, e.g., by checking the Target AIoT device information. The AIOTF associates the RAN NGAP Device ID with the AIoT device context in the AIOTF.

If none of successful Inventory response is received, Step 8 -11 is not performed and the AIOTF sends a failure report to the NEF in Step 12.

8.	For each successful Inventory response received, the AIOTF sends Command Request message (Correlation ID, [Reader ID], NAS Command Request, RAN NGAP Device ID, [Approximate D2R message size]) to the NG-RAN directly or as an AIoT Transfer Container via an AMF as specified in clause 6.2.4. The NAS Command Request message includes the AIoT data. The RAN NGAP Device ID is used by the NG-RAN to determine the AIoT device context in NG-RAN.

Editor's note:	Additional information included in the NAS Command Request for security will be determined and aligned with SA WG3.

9.	The NG-RAN sends the AS R2D message (NAS Command Request) to the AIoT Device.

10.	The AIoT Device sends the AS D2R message (NAS Command Response) to the NG-RAN. The NAS Command Response message may include the AIoT data.

Editor's note:	The AS R2D message and AS D2R message will be aligned with RAN WG's specification.

Editor's note:	Additional information included in the NAS Command Response for security will be determined and aligned with SA WG3.

11.	The NG-RAN responds with a Command Response message (Correlation ID, Reader ID, NAS Command Response, RAN NGAP Device ID) to the AIOTF directly or as an AIoT Transfer Container via an AMF as specified in clause 6.2.4. The AIOTF determines the AIoT device context by the RAN NGAP Device ID received.

12.	The AIOTF reports the result of the AIoT Command request to the NEF by sending the Naiotf_AIoT_Command Notify message (AIoT Device ID(s), AF ID, AIoT data).

13.	The NEF informs the AF of the result of the AIoT_Command request by sending the Nnef_AIoT_Command Notify message (AIoT Device ID(s), AF ID, AIoT data).

* * * End of Change * * * *