3GPP TSG-WG SA2 Meeting #169	S2-2504897

Fukuoka, Japan, May 19 - 23, 2025	(revision of S2-250xxxx)

Source:	Ericsson

Title:	New Solution for Differentiated Charging when Network Performs Service Adjustment

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_ph2/Rel-20

Abstract of the contribution: An additional IE (“reason for a policy change”) is added in PCC rules from the PCF to the SMF. The reason for a policy change is sent to the CHF from the PCF when service adjustment is performed due to energy saving and energy efficiency.

# 1. Introduction/Discussion

According to the WI#2, the PCF may make policy decisions related with energy efficiency and energy saving. Those policy decisions can have an impact in the traffic delivered to the UE. However, no specific solution has been defined to enable a differentiated charging for the UE when those energy efficiency-related actions are taken for this UE. Ideally, when the network applies such an action to enhance energy efficiency that affects service delivery, the charging applied to this traffic should be different (i.e., reduced) compared to the one in regular circumstances.

Relying on existing charging mechanism to have the PCF assigning a different service-specific Rating Group for IP flows where energy efficiency decisions have been applied might be overly simplistic and restrictive. This approach could lead to the need for multiple Rating Groups for the same service, becoming unmanageable in scenarios involving multiple services and energy efficiency decisions.​

This contribution proposes to extend the charging information provided in a PCC rule from the PCF to the SMF (N7 interface) including additional information about the “reason for a policy change”, where the energy efficiency could be one reason. Therefore, when the PCF applies policy decisions based on energy efficiency reasons, the PCF sends an update of the PCC rule to the SMF including the additional information with the reason indicating energy efficiency.

The solution enables that the network may apply a differentiated (reduced) charging for the UE when service adjustment actions such as ones triggered by energy efficiency and energy saving energy are performed by the network. It also enables 5GC network to keep track of the reason of the impact in the service delivery.

The solution is made general enough to cover scenarios for actions not only related with energy efficiency but also other reasons, e.g. in traffic congestion scenarios.

# 2. Text Proposal

It is proposed to agree the following solution to 3GPP TR 23.700-67

First change

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	The number of columns of Key Issues will be updated according to clause 5.

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solution# | 1 | 2 | 3 |  |
| X |  | X |  |  |

Second change (all new)

## 6.X	Solution #XX: Differentiated Charging when network performs service adjustment

## 6.X.0	High-level solution principles

## The solution is based on following principles:

-	PCF adds an IE of “reason for a policy change” in charging info of the associated PCC rule when service adjustment is applied for an SDF.

-    SMF sends the reason for a policy change together with existing charging info to CHF.

### 6.X.1	Functional Description

This solution addresses KI#2.

It is proposed to extend the charging information provided in a PCC rule from the PCF to the SMF over the N7 interface with an additional IE (“reason for a policy change”) which indicates the reason for a policy decision applied for a UE, where the energy efficiency could be one of the reasons. See the example IE, marked in blue, in the table 4.1.4.2.1-1 of TS 29.512.

Table 4.1.4.2.1-1: PCC rule information elements

| Information name | Description | Category |
| --- | --- | --- |
| Rule identifier | Uniquely identifies the PCC rule, within a PDU Session. It is used between PCF and SMF for referencing PCC rules. | Mandatory |
|  | Service data flow detection |  |
| Precedence | Determines the order, in which the service data flow templates are applied at service data flow detection, enforcement and charging. | Mandatory |
| Service Data Flow Template | For IP PDU traffic: Either a list of service data flow filters or an application identifier that references the corresponding application detection filter for the detection of the service data flow. For Ethernet PDU traffic: Combination of traffic patterns of the Ethernet PDU traffic. | Mandatory |
| Mute for notification | Defines whether application's start or stop notification is to be muted. | Optional |
|  | Charging |  |
| Charging key | The charging system (CHF) uses the charging key to determine the tariff to apply to the service data flow. | Optional |
| Reason for a Policy Change | It indicates the reason of a policy decision that can imply an impact in the service delivery of the UE, being energy efficiency (with different severity levels) one of the reasons. | Optional |
| Service identifier | The identity of the service or service component the service data flow in a rule relates to. | Optional |
| Sponsor Identifier | An identifier, provided from the AF, which identifies the Sponsor, used for sponsored flows to correlate measurements from different users for accounting purposes. | Optional |
| Application Service Provider Identifier | An identifier, provided from the AF, which identifies the Application Service Provider, used for sponsored flows to correlate measurements from different users for accounting purposes. | Optional |
| Charging method | Indicates the required charging method for the PCC rule. Values: online or offline or none. | Optional |
| Service Data flow handling while requesting credit | Indicates whether the service data flow is allowed to start while the SMF is waiting for the response to the credit request. Only applicable for charging method online. | Optional |
| Measurement method | Indicates whether the service data flow data volume, duration, combined volume/duration or event shall be measured. This is applicable to reporting, if the charging method is online or offline. Note: Event based charging is only applicable to predefined PCC rules and PCC rules used for application detection filter (i.e. with an application identifier). | Optional |
| Application Function Record Information | An identifier, provided from the AF, correlating the measurement for the Charging key/Service identifier values in this PCC rule with application level reports. | Optional |

This new IE information element together with the charging rate will be forwarded by the SMF to the CHF. The CHF can take it into account when applying the charging and billing for the UE. The new information can be also used by the CHF to maintain some KPIs including information about whether the impact in the service delivery of a UE has been caused by network applying actions.

The table below (from Table 6.2.1.3.1 of TS 32.255) shows how the example new IEs, marked in blue, fits within a charging request:

Table 6.2.1.3.1: Structure of PDU Container Information

| Information Element | Category | Description |
| --- | --- | --- |
| Time of First Usage | OC | This field holds the Timestamp when the first transmitted data packet of the service data flow matching the current used unit container |
| Time of Last Usage | OC | This field holds the Timestamp when the last transmitted data packet of the service data flow matching the current used unit container |
| QoS Information | OC | This field holds the QoS applied during the service data container interval |
| QoS Characteristics | OC | This field holds the QoS characteristics applied for QoS information. It is only be used when the non-standardized 5QI is present in QoS information. |
| AF Charging Identifier | OC | A Charging Identifier, provided from the AF, may be used to correlate the measurement for the Charging key/Service identifier values in this PCC rule with application level reports. |
| AF Charging Id String | OC | A string that, may be provided from the AF instead of AF Charging Identifier, depending on support. |
| Reason for Policy Change | OC | It indicates the reason of a policy decision impacting the service delivery, e.g. energy efficiency (with different severity levels). |
| User Location Information | OC | This field holds the user location during the used unit container interval |
| UE Time Zone | OC | This field holds the Time Zone of where the UE is located, during the used unit container interval. |
| Presence Reporting Area Information | OC | This field holds the Presence Reporting Area Information of UE during the used unit container interval. |
| Serving Network Function ID | OC | Serving Network Function identifier. |
| RAT Type | OC | This field holds the RAT type during the used unit container interval. For MA PDU session, this field holds the RAT type associated to the access which activated the rating group. |
| Sponsor Identity | OC | This field holds the identifier of the sponsor when sponsored data connectivity is used |
| Application Service Provider Identity | OC | This field holds the identifier of the application service provider that is delivering a service to the end user. |
| Charging Rule Base Name | OC | This field holds the reference to group of PCC rules predefined at the SMF |
| 3GPP PS Data Off Status | OC | This field holds the 3GPP Data off Status during the used unit container interval |
| MA PDU Steering functionality | OC | This field holds the Steering functionality used during the used unit container interval when MA PDU session |
| MA PDU Steering mode | OC | This field holds the Steering mode used during the used unit container interval when MA PDU session. |
| Traffic Forwarding Way | OC | This field holds the traffic forwarding way for the 5G VN group communication if present. If the SMF can distinguish the traffic forwarding way, the SMF reports the volume per traffic forwarding way. If the SMF cannot distinguish, the SMF reports the traffic and the corresponding traffic forwarding way (may be multiple). |
| QoS Monitoring Report | OC | This field holds the QoS monitoring result (i.e., average packet delay per QoS flow per UE) for the service data flow. |
| MBS Session ID | OC | This field holds the MBS session identifier for the service data flow referring to clause 6.5.1 of TS 23.247 [204] if present. |
| MBS Delivery Method | OC | This field holds the MBS traffic delivery method (i.e. 5GC Shared MBS traffic delivery method and 5GC Individual MBS traffic delivery method referring to clause 4.1 of TS 23.247 [204]) for the multicast communication if present. |

### 6.X.3	Procedures

The picture below shows the procedure for applying differentiated charging based on energy efficiency policy decisions as one reason example.

c

Figure 6.X.3.1: Differentiated Charging Based on Energy Efficiency Policy Decisions

Prerequisite: The PCF is retrieving the slice level energy consumption from OAM system.

- The UE triggers a PDU session establishment procedure according to 3GPP TS 23.502 clause 4.3.2.
During the establishment procedure the PCF makes policy decisions and determines regular QoS (QoS1) and Charging information (Rating Group 1) for the traffic associated with app1.

- The CHF applies charging based on Rating Group 1 to the traffic of the UE for app1

- At some point in time and based on the slice energy consumption measures, the PCF decides to apply some policy decisions (e.g. to change QoS although some other actions could be decided) to reduce the energy consumption of the UE and/or NW slice.
The PCF generates corresponding PCC rule including the new QoS (Qos_2) for the service, same Rating Group (RG_1) and the new  “reason for policy decision” IE including  “energy_efficiency”  as reason and “X” as severity level (for other use cases, other cause code and optionally severity level should be used).  The PCF delivers the PCC rule to the SMF and SMF forwards the charging information to the CHF. Note: the changing the QoS is just one policy example, any service adjustment related policy decision can be applied to.

- Upon the reception of the information from the SMF, the CHF uses the rating group plus additional “Reason for policy decision” (the reason + the severity level) to apply differentiated (reduced) charging to the traffic of the UE.

- The CHF updates also new KPIs with information about reason for policy decisions and impacts on the service delivery.

### 6.X.4	Impacts on existing services, entities and interfaces

PCF:

- Collect slice level EnC from OAM (using SA5 defined services). Compare the collected EnC to the thresholds in the slice policy data.

- Collect EnC of UE level, SDF, PDU session. Compare the collected EnC to the thresholds in the UE, SDF, Session policy data

-      Generate reason for pliocy decisions and include it as additional charging info of PCC rule to the SMF

SMF:

- Send the reason for pliocy decisions to the CHF.

CHF:

- Receive the reason for pliocy decisions from SMF.

- Use the reason for differentiated charging and KPI.

# * * * * End of changes * * * *