SA WG2 Meeting #169	S2-2504909

May 19 – 23, 2025, Fukuoka, Japan

Source:	Ericsson

Title:	TAI handling for regenerative access

Document for:	Discussion / Approval

Agenda Item:	19.1.2

Work Item / Release:	5GSAT_Ph3-ARC / Rel-19

Abstract of the contribution: This contribution discusses way forward for the TAI handling with regenerative satellite access.

# Background

At SA2#168, a reply-LS from RAN3 on configuring a Tracking Area ID timetable per gNB in the AMF was discussed without conclusions.

- 	Some companies preferred to reply positively to RAN3, i.e. confirming that configuring TAI timetables in AMF could be an (optional) optimization that could be used to reduce load on the feeder link by avoiding the need for RAN CONFIGURATION UPDATE procedures.

- 	Some other companies preferred to indicate to RAN3 that this solution should not be pursued in rel-19 due to marginal savings and other arguments.

A DP (S2-2503268) was presented at SA2#168 and submitted to RAN3 #127bis for information (R3-252073), providing some example numbers for how much feeder link load could be reduced. In summary, that paper showed that removing the need for RAN CONFIGURATION UPDATE procedures when the TAI is reconfigured could provide relative savings off the total NGAP load on the order of:

- ~0.5%, if a gNB onboard is connected to few (~2) AMFs and is serving a high number of CONNECTED mode UEs per TA (~400). In this case the total load on the feeder link is anyway high, due to the high number of CONNECTED mode UEs per TA.

- ~10% off the total NGAP load, if a gNB onboard is connected to more (~6) AMFs and is serving a low number of CONNECTED mode UEs per TA (~50). In this case the total load on the feeder link is however low, due to the lower number of CONNECTED mode UEs per TA.

The numbers are rough estimates and may differ depending on deployment, traffic scenarios etc.

# Discussion

Considering the fact that:

- 	It was not possible to reach a consensus at SA2#168, and it was anticipated that the situation would stay the same at SA2#169;

- 	The relative savings as mentioned above seem rather limited, especially in the case of a high number of CONNECTED mode UEs per TA, which is the case where optimizations would be most important;

- 	The use of TAI timetable configuration on AMF would not have impacts on the NGAP protocol definition. It would however require a coordinated configuration of onboard gNBs and AMFs, thus assuming a tight coordination of RAN and AMF;

- 	In case feeder link load turns out to be an issue in regenerative architecture deployments, a study may be initiated in a later release;

the authors of this paper suggest leaving the potential optimization to a discussion in a later release (as per regular working procedures). It is not precluded to achieve the solution proposed by RAN3, if desired, as a deployment/implementation specific optimization, without a description in the standard.

# Proposal

Based on the above, it is proposed to reply to RAN3 on the following lines:

SA2 thanks RAN3 and SA5 for their Reply Liaisons on OAM requirements to support regenerative payload transport links.

SA2 has further discussed the solution proposed by RAN3, and worked on by SA5, and would like to make the following observations:

- 	Configuring AMF with TAI timetable seems feasible and can allow some optimization in terms of NGAP signalling over the feeder link.

- 	However, the reduction in feeder link load may be small compared to UE-specific NGAP signalling: While a satellite is moving, there is a need to move/handover CONNECTED mode UEs from a gNB onboard a setting satellite to a gNB onboard a rising satellite, which will result in UE-associated NGAP signalling. By removing RAN CONFIGURATION UPDATE procedures due to TAI reconfiguration, SA2 estimates that it is possible to provide a relative reduction of the total NGAP load on the order of:

o 	~0.5%, if a gNB onboard is connected to few AMFs and is serving a high number of CONNECTED mode UEs per TA. In this case the total load on the feeder link is high, due to the higher number of CONNECTED mode UEs per TA, and avoiding a relatively small number or RAN CONFIGURATION UPDATE procedures does not make a substantial difference.

o 	~10% off the total NGAP load, if a gNB onboard is connected to more AMFs and is serving a low number of CONNECTED mode UEs per TA. However, in this case the total load on the feeder link is rather low, due to the low number of CONNECTED mode UEs per TA, and avoiding RAN CONFIGURATION UPDATE may be less important.

- 	SA2 could not reach a consensus on whether the TAI timetable configuration should be pursued in rel-19 or not.

Based on the above considerations, SA2 proposes that TAI timetable configuration per gNB in AMF via OAM is not captured in normative text in Release 19 and left for potential further discussion in a later release (as per regular working procedures).