3GPP TSG SA WG2 Meeting #169	S2-2504926

Fukuoka, JP, 19-23 May, 2025	(revision of S2-220xxxx)

Source:	China Mobile

Title:	New Solution for KI#1 on 5GC Support for UE Data Collection over the User Plane

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: Proposed a new solution to address KI#1 on 5GC Support for UE Data Collection over the User Plane.

1. Discussion

This proposal proposes a new solution to address KI#1 on 5GC Support for UE Data Collection over the User Plane.

5.2	Key Issue #1: Transfer of data over UP for UE data collection

5.2.1	Description

This key issue aims to provide solutions for UE data collection to meet the requirements for AI/ML for NR air interface with UE-side model training. This process requires the transfer of training data from the UE to the 5G core and then to the OTT server.

This key issue will investigate the following aspects:

-	How to support the standardized transfer of standardized data over UP for UE data collection including how to initiate, terminate and manage UE data transfer over the UP connection.

-	How to support in 5GC the full visibility of standardized UE data contents.

-	How 5GC can support the full controllability of standardized UE data transfer.

-	How to differentiate the traffic for UE data collection from the UE regular traffic in the 5GC.

-	Whether and how the UE knows about what the standardized data to be transferred are.

- Text Proposal

It is proposed to capture the following changes in TR 23.700-04.

# * * * * First change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues and Use Cases

Table 6.0-1: Mapping of Solutions to Key Issues and use cases

|  | Key Issues |  | Use Cases (optional) |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Use Case #1> | <Use Case #2> |
| #1 | x |  |  |  |
| #2 | x |  |  |  |
| #3 | x |  |  |  |
| #4 | x |  |  |  |
| #X | x |  |  |  |

# * * * * Second change (all new text) * * * *

6.X	Solution #X: UE data collection over the user plane

6.X.1	High level principles

The high level principles of the solution are:

-	The NWDAF-C directs AMF to initiate the UE data collection procedure and provides the IP address of NWDAF-U or ADRF to UE.

-  The NWDAF-U or ADRF receives the collected UE related data from UPF via N6 or N9 interface.

-	To report the uplink data to NWDAF-U or ADRF received from the NWDAF-C, the UE may request to initiate a new PDU session.

In this solution, the NWDAF splits into NWDAF-C and NWDAF-U. NWDAF-C is responsible for control plane, while NWDAF-U is responsible for data collection and processing, etc.

6.X.2	Description

This solution is to address Key Issue#1 for UE data collection over UP.

- X.3	Procedures

Figure 6.X.3-1: Procedure for UE data collection over user plane

- The AF sends the UE data collection request including UE ID(s), area of interest and time duration to 5GC functions, e.g. NWDAF-C via the NEF.

2.	The NWDAF-C stores the UE ID(s), area of interest and time duration and determines the AMF(s) based on the UE ID or the UE ID list.

3.	The NWDAF-C sends the UE data collection request to UE via the AMF including UE data collection indicator and NWDAF-U or ADRF IP Address.

4.	The UE may initiate a PDU session establishment procedure based on UE data collection indicator.

5.	The UE sends the UE data collection response to NWDAF-C, and NWDAF-C response to AF via the NEF.

6.	The UE performs UE data collection and sends the uplink data to the NWDAF-U or ADRF, whose IP Address is received in step 3, which stores the UE data.

7.	The NWDAF-U or ADRF exposes the data to AF via the NEF.

6.X.4	Impacts on services, entities and interfaces

NWDAF-C:

-	Sends the UE data collection request to UE via the AMF including UE data collection indicator and NWDAF-U or ADRF IP Address.

NWDAF-U or ADRF:

-	Receives the UE data from UPF via N6 or N9 interface and stores the data.

-	Exposes the data to AF via the NEF.

UE:

-	Initiate a PDU session establishment procedure based on UE data collection indicator.

-	Performs UE data collection and sends the uplink data to the NWDAF-U or ADRF

# * * * * End of Changes* * * *