3GPP TSG-WG SA2 Meeting #169	S2-2504935

19th – 23th May 2025, Fukuoka, Japan

Source:	China Mobile

Title:	NG-RAN Node and RAN Reader Selection—the NEF performs the location conversion

Document for:	Approval

Agenda Item: 	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: This paper proposes to address the NEF performs the location conversion in NG-RAN Node and RAN Reader Selection in 5.3.3 clause in TS 23.369 for AIoT.

- Introduction

During the SA2#167 meeting,  an description about the how NEF obtain target area info from external target area info clause 5.3 in 23.369-020:

The AF may provide the expected target area information in the AIoT service request. The NEF may convert the target area information received from the AF to internal area information. The NEF queries the NRF to obtain serving AIOTF(s) based on internal area information. Then, the NEF forwards the AIoT service request, including the internal area information, to the AIOTF.

However, in the SA2#168 meeting, these description are deleted in 5.3.3 clause and are not included in 5.3.1	AIOTF Discovery and Selection too.

Controversial Issue 1: There is no clear description that how NEF obtains the target area information based on external target area information.

Proposal 1:

Adding the description about the how NEF perform the conversion between external target area information and target area information back into 6.2.2 clause.

Controversial Issue 2: Multiple AIOTF may be involved in an AIoT service request. In this case, NEF needs to

sends the AIoT service request to each selected AIOTF along with its corresponding target area information.

Proposal 2:

Adding description in 5.3.3 clause:

If multiple AIOTFs are selected, the NEF sends the AIoT service request to each selected AIOTF along with its corresponding target area information.

2. Text Proposal

It is proposed to capture the following changes to 3GPP TS 23.369 for Ambient IoT.

# * * * * First change * * * *

## 5.3	Discovery and Selection of AIoT node(s)

### 5.3.1	AIOTF Discovery and Selection

The AIOTF discovery and selection functionality is supported by the NEF and AF to determine an AIOTF to handle AIoT Services.

When the NEF or AF performs AIOTF discovery and selection for an AIoT service operation request, the following applies:

-	The NEF or AF shall utilize the NRF to discover AIOTF instance(s) unless AIOTF information is available by other means, e.g. locally configured in the NEF or AF. The NEF or AF provides to the NRF the Target Area information, when trying to discover AIOTF instance(s). The NRF provides AIOTF instance(s) to the NEF or AF.

NOTE:	The local configuration in the NEF can be used for example to select a specific AIOTF instance for the request from a given AF.

-	The NEF or the AF selects the AIOTF instance(s) based on the available AIOTF instance(s) (obtained from the NRF or locally configured)

When AIoT Device ID information indicates individual AIoT device(s), the NEF may select AIOTF(s) by taking into account the last known AIOTF instance(s) for those device(s) from ADM.

### 5.3.2	ADM Discovery and Selection

The ADM discovery and selection function is supported by the AIOTF to select an ADM instance to retrieve the device profile data or update the last known AIOTF for the AIoT device. The AIOTF may also discover and select an ADM to retrieve AF authorization data. Similarly, the NEF uses the ADM discovery and selection function to select an ADM to obtain the last known AIOTF for the AIoT device.

When the ADM discovery is performed, the AIOTF or the NEF utilizes the NRF to discover the ADM instance(s) unless the ADM information is available by other means, e.g., locally configured. The AIOTF or the NEF selects an ADM instance based on the available ADM instances (obtained from the NRF or locally configured).

One or more of the following factors may be considered for the ADM discovery and selection:

-	The AIoT device permanent ID or domain information.

-	The AF ID.

Editor's note:	The further investigation of factors for the ADM selection will be needed and other factors are FFS.

### 5.3.3	NG-RAN Node and RAN Reader Selection

The AIOTF selects NG-RAN node(s) and optionally RAN readers.

The AF may provide the external Target Area information (e.g., geo-location) in the AIoT service request. Based on local configuration via OAM, The NEF may convert the external Target Area information received from the AF to Target Area information. The NEF obtains the targeted AIOTF(s) based on the clause 5.3.1. Then, the NEF forwards the AIoT service request, including the Target Area information, to the AIOTF. If multiple AIOTFs are selected, the NEF sends the AIoT service request to each selected AIOTF along with its corresponding Target Area information.

The AIOTF obtains the NG-RAN information (supported Area, RAN reader ID list, and, optionally, the location of each served RAN reader) via OAM. The AIOTF receives an AIoT service request including the Target Area information from the NEF or trusted AF. Based on the received Target Area information and the NG-RAN information configured by OAM, the AIOTF selects the NG-RAN node(s) and optionally RAN reader(s).

NOTE 1:	The Target Area information can span the supported Area of multiple NG-RAN node(s) or can be a subset of the supported Area of a NG-RAN node.

The AIOTF sends the AIoT service request to the selected NG-RAN node(s), optionally including the RAN Area information derived from mapping the Target Area information to the supported Area and/or RAN reader ID list, if selected, to assist RAN reader selection by NG-RAN node, either directly or through the selected AMF. If multiple NG-RAN nodes are selected, the AIOTF sends the AIoT service request to each selected NG-RAN node along with its corresponding RAN Area information and/or the RAN reader ID list, if selected

If the AIOTF does not provide the RAN Area information or the RAN reader ID list in the AIoT service request to the NG-RAN node, then the NG-RAN node may use all available RAN readers.

Editor’s note:	NG-RAN and RAN reader information needs to coordinate with the RAN WG(s). Details are pending RAN WG feedback.

Editor’s note:	The AIOTF or AMF configuration of NG-RAN and RAN reader information over NGAP needs to coordinate with RAN WG(s).

Editor’s note:	It is FFS how NG-RAN transfers and updates its information to the AIOTF in indirect connectivity.

NOTE 2:	RAN reader ID is not exposed to the AF.

If an AIoT service request includes AIoT device Identifier(s), the AIOTF may consider the last known serving RAN reader(s) from the AIoT device context to determine targeted RAN reader(s) directly.

NOTE 3:	From the selected NG-RAN node, the AIOTF receives the Inventory Report including the RAN reader ID that represents the AIoT device’s location at reader ID granularity. The AIOTF uses the RAN reader ID to update the last known serving RAN reader information in the local AIoT device context.

### 5.3.4	AMF Discovery and Selection

For indirect Connectivity via AMF (see clause 4.2), AMF discovery and selection functionality is implemented in AIOTF.

When the AIOTF performs AMF discovery and selection in order to forward AIoT service operation messages the NG-RAN via AMF, the following applies:

-	After the NG-RAN selection for the AIoT service operation (see clause 5.3.3), the AIOTF selects the AMF that has association with the selected NG-RAN nodes in order to forward the AIoT service operation messages.

Editor’s note: 	It is FFS whether and how the procedure is performed between AMF and AIOTF in order to provide the NG-RAN ID of NG-RAN from the AMF to the AIOTF.

# * * * * second change * * * *

## 6.2	AIoT Service Procedures

### 6.2.1	General

Clause 6.2.2 provides the procedure for AIoT Inventory. Clause 6.2.3 provides the procedure for AIoT Command.

Editor's note:	Additional information in the steps, parameters, and their naming throughout the procedures requires additional details, alignment with other clauses and references adding as required.

Editor's note:	Alignment is required for how to document and describe Direct Connectivity and Indirect Connectivity options in the procedures.

### 6.2.2	Inventory Procedure

Figure 6.2.2-1 describes the inventory procedure.

The procedure focuses on the messages and parameters used for the communication between AIOTF and NG-RAN regardless of the path to access NG-RAN, see clause 4.2.2.1. The handling of the different communication paths is described in clause 6.2.4.

Figure 6.2.2-1: Inventory Procedure

1.	The AF invokes Nnef_AIoT_Inventory(AF ID, [External Target Area information], information about the target AIoT Device(s), [Approximate number of AIoT Devices]) service operation request to the NEF.

The External Target Area information is specified in clause 5.3.

Information about the target AIoT Device(s) may include Filtering Information, as described in clause 5.8, or include complete AIoT Device Identifier(s).

The approximate number of AIoT Devices, if provided, is used to determine the number of AIoT Devices expected to respond to this inventory service operation, which is sent by AIOTF to the NG-RAN in the assistance information for NG-RAN in step 7 for proper radio resource allocation.

Editor's note:	The parameters for the inventory service operation need further definition.

2.	The NEF may further authorize the AF request as specified in clause 5.6.

Based on external Target Area information from the AF and location configuration via OAM, the NEF determines the Target Area information, and selects one or multiple AIOTF(s) to handle the request as specified in clause 5.3.1.

3.	The NEF invokes the Naiotf_AIoT_Inventory(AF ID, [Target Area information], [information about the target AIoT Device(s)], [Approximate number of AIoT Devices]) service operation towards to the selected AIOTF(s).

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. The AIOTF may perform authorization as specified in clause 5.6. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT service operation request with an appropriate cause code, and step 7 onwards are skipped.

The AIOTF generates a Correlation ID corresponding to this AF service operation request.

The AIoT Identification Information to be provided to NG-RAN can include Filtering Information, as defined in clause 5.8, or a single AIoT Device Identifier.

AIOTF performs Reader Selection, see clause 5.3.3.

The AIOTF may also use the last serving Reader to assist with determining which Readers to use for an AFs request targeting for a specific AIoT Device.

The AIOTF determines assistance information as described in clause 5.4.

5.	AIOTF sends the AIoT Inventory Service Response to the NEF containing the accept or reject result for the AIoT Inventory service operation request based on step 4.

6.	NEF sends the AIoT service operation response to the AF, containing the accept or reject result for the AIoT Inventory service operation request as specified in clause 8.3.

7.	The AIOTF sends the Inventory Request message including the Correlation ID, the AIoT Identification Information to be included in the paging message, and assistance information to the selected NG-RAN.

Editor's note:	Whether the Inventory Request sent to NG-RAN includes indication about whether there will be a follow up command or not needs to be determined.

8.	The NG-RAN sends an Inventory Response to the AIOTF with the Correlation ID indicating that the Inventory Request is received successfully and will perform the service operation accordingly.

9.	Upon reception of the Inventory Request message from the AIOTF, the RAN Reader(s) will execute the inventory operation. The RAN Reader(s) broadcast the paging message that includes the AIoT Identification Information.

The AIoT Device determines whether it matches the AIoT Identification Information, as described in clause 5.8.

If an AIoT device matches the AIoT Identification Information in the paging message, the AIoT Device responds to the paging message and sends an AIOT NAS message that includes its AIoT Device ID.

Editor's note:	Whether and how the Device ID is concealed or encrypted will be determined and aligned with SA WG3.

10.	NG-RAN sends one or more Inventory Report messages to the AIOTF including the Correlation ID, Reader ID and the AIOT NAS message(s) from the AIoT Device(s). The AIOTF stores the mapping between the Reader ID and AIoT Device ID(s).

NOTE:	When to erase the stored mapping between the Reader ID and AIoT device ID(s) is up to implementation and local configuration.

11.	The AIOTF validates the results, using local stored device information or device profile data retrieved from the ADM. The AIOTF may aggregate the results.

12.	Optionally, if the NG-RAN detects that no more AIoT Devices will respond to the inventory procedure, the NG-RAN informs the AIOTF that the procedure is complete.

Editor's note:	The details how interactions between NG-RAN and AIOTF to indicate no further Inventory Reports will be sent and completion of the procedure need to be aligned with RAN.

13.	The AIOTF reports the progress of the AIoT inventory request to the NEF by sending the Naiotf_AIoT_Notify message including a list of AIoT Device Permanent Identifier (s). The AIOTF may send multiple reports. The AIOTF in the final Naiotf_AIoT_Notify message indicates it is the last report for this operation. If multiple AIOTFs are involved in the procedure, the NEF may receive the AIoT_Notify from multiple AIOTFs.

14.	When receiving the Naiotf_AIoT_Notify message from AIOTF, the NEF informs the AF of the outcome of the AIoT_Inventory request by sending the Nnef_AIoT_Notify message(s) including the AIoT Device Permanent Identifier(s). The NEF in the final Nnef_AIoT_Notify message indicates that it is the last report for this operation.

# * * * * End of changes * * * *