SA WG2 Meeting #S2-169	S2-2504936

19 - 23 May, 2025, Fukuoka, Japan	(revision of S2-250xxxx)

# Title:	New solution for KI#3 and #6

# Source:	Sony

# Document for:	Approval

Agenda Item:	20.2.1 (FS_Sensing_ARC)

Abstract:	This contribution proposes a for KI#6 and KI#3

# 1. Discussion

This pCR proposes a new solution for KI#3 and KI#6.

# 2. Proposal

It is proposed to add the following KI to TR 23.700-14.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| #X |  |  | X |  |  | X |

# * * * * Next change (all new) * * * *

## 6.X	Solution #X: Sensing Function selection and Sensing Entity selection based on Sensing Entity Capabilities and Conditions.

#### 6.X.1	Description

This solution addresses Key Issue #3 and Key Issue #6.

This solution proposes a new network node Sensing Function (SF). The SF register/deregister/update itself to the NRF as defined in clause 4.17.1.1, 4.17.2, and 4.17.3 of TS 23.502 [x1], respectively. In case of registration or update the Sensing Function include its supported sensing capabilities to the NRF to be stored in the SF profile at the NRF. The NRF is used for the discovery of SF’s and may be used by any NF including Sensing Entities, trusted AF or by the NEF for any AF’s.

The capabilities in the SF profile may include:

- Supported area

- Supported sensing related capabilities: (object detection and tracking, environment monitoring, gesture/motion monitoring)

- Sensing object types (e.g., UAV, vehicle, object creating hazard, human)

Editor’s Note:	This list may not be complete and needs to be aligned with RAN.

The Sensing Entities register/deregister to the SF and the SF keeps an inventory of available Sensing Entities and their capabilities. The Sensing Entities discover the SF via local configuration or use the NRF for that.

For every new or changed sensing operation and after authorization and revocation that will be defined in Key Issue#2 the SF decides if any activation, deactivation or configuration changes is needed to any Sensing Entities. The decision logic is up to SF implementation but a set of capability parameters from the sensing entities will be specified and how the SF activate, deactivate or reconfigure the Sensing Entity.

The profile for each Sensing Entity may include the following capabilities:

- Common for Sensing Transmitter and/or Sensing Receiver

  - Type of supported sensing waveform/reference signal

    - The same/reuse the reference signal for communications/positioning (e.g., SSB, CSI-RS, TRS, PRS)

    - Dedicated reference signal for sensing, such as radar like signal LFM-OFDM (Linear Frequency Modulation-Orthogonal frequency division multiplexing).

  - Supported frequency band (FR1, FR2 & FR3)

  - Supported antenna configuration (Omnidirectional, multiple beams (x number of beams)

- Specific for Sensing Receiver:

  - Sensing measurement types, for example

    - Timing based measurements (e.g., Time Different of Arrival (TDOA))

    - Power based measurements (e.g., received signal strength)

    - Angle-based measurement: AoA (Angle of Arrival) or AoD (Angle of Departure).

Editor’s Note:	This list may not be complete and needs to be aligned with RAN.

#### 6.X.2	Procedures

The procedure for the discover and selection mechanism:

Figure 6.X.2-1: Procedure to support sensing service selection

- SF sends Nnrf_NFManagement_NFRegister and include the supported capabilities as defined in 6.X.1 that will be stored in the SF profile at the NRF.

- Sensing Entities sends Nsf_SERegister and include supported capabilities to the SF as defined in 6.X.1.

Editor’s Note:	It is FFS if additional configuration or local conditions at the Sensing Entity is included with the capabilities.

- NEF sends the Nnrf_NFDiscovery_Request to the NRF.

- NRF respond with supported SF(s) and their capabilities.

- NEF select a SF based on supported SF capabilities and the consumer’s request

- NEF sends Nsf_Activate_Subscribe to the SF to initiate a sensing session with the based on the sensing consumer’s request.

- SF verify that the AF is authorized for the sensing operation as part of solutions of KI#2

- SF decide from its inventory of available Sensing Entities which one to activate/deactivate or reconfigure based on the decided sensing operation.

- SF sends Nse_SE_Activate to activate all needed Sensing Entities that will be included for the sensing operation. This may also include deactivation or reconfiguration of already active Sensing Entities.

Editor’s Note:	It is FFS if additional configuration or local conditions at the Sensing Entity is included with the capabilities.

- Sensing Entity perform the requested configuration and starts the requested sensing operation

- Sensing Entity deliver the requested sensing data according to solutions for KI4 & 5.

#### 6.X.3	Impacts on Services, Entities and Interfaces

SF:

- Inventory of available Sensing entities and corresponding capabilities.

- Selection logic to select appropriate Sensing Entities and methods based on consumer request.

- Authorization of sensing operation.

- Activation/deactivation of Sensing Entities including configuration of Sensing entities.

NRF:

- Support for storing a profile of the SF capabilities.

NEF:

- Discovery and selection of SF based on SF profile.

UDM:

- UDM enhancements to store or manage the authorization data for sensing service according to solutions for KI#2.

# * * * * End of changes * * * *