SA WG2 Meeting #S2-169	S2-2504937

19 - 23 May, 2025, Fukuoka, Japan

Title:	Draft Reply LS on N6-Unmarked PDUs

Response to:	S2-250465/S4-250738

Release:	Rel-19

Work Item:	5G_RTP_Ph2 / XRM_Ph2

Source:	SA2(CATT)

To:	SA4, CT4

Cc:	-

Contact Person: 	Chunshan Xiong (chunshan.xiong@cictmobile.com)

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

SA2 thanks for the LS from the SA4, SA2 has the following observation and answer for the LS:

For a WebRTC application, the audio, video, and RTCP data flows are normally transported in the same IP-5-tuple connection. The audio, video and RTCP data flows can be bound into the same QoS Flow in R18 or can be bound into 3 different QoS Flows in R19.

If the audio, video and RTCP data flows are bound into the same QoS Flow, the different RTCP messages may have different importance and have different PSI values. E.g., the RTCP message for audio may be more important than the RTCP message for video. Even for the different RTCP messages for the same audio, the SR and RR RTCP messages may be more important than other RTCP messages. For the RTCP, if a dedicated PSI mechanism is used, SA2 thinks that a single value is not suitable for the different RTCP messages and multiple different PSIs are better to be used for the different RTCP messages.

In R19, if the RTCP is bound to a dedicated QoS Flow, there is no need to use PSI for the RTCP packets.

Based on the above observation, SA2 agrees that providing a single PSI value for an unmarked PDU Set does not provide better performance of the PDU Set QoS handling, and SA2 does not support any PSI signaling for N6-unmarked PDUs in R19.

2. Actions:

To SA4

ACTION: 	SA2 kindly asks SA4 to take the above information into account.

3. Dates of Next SA2 Meetings:

SA2#170	25th – 29th August 2025 		 Goteborg, SE

SA2#171	13th – 17th October 2025 		 TBD, CN