SA WG2 Meeting #S2-169	S2-2504938

19 - 23 May, 2025, Fukuoka, Japan

Title:	Draft Reply LS on RTP retransmission

Response to:	S2-2504566/S4-250739

Release:	Rel-19

Work Item:	5G_RTP_Ph2 /XRM_Ph2

Source:	SA2 (CATT)

To:	SA4, RAN2

Cc:	-

Contact Person: 	Chunshan Xiong (chunshan.xiong@cictmobile.com)

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

SA2 thanks for the LS from SA4, and SA2 provides the following answer for the question:

Question to SA2: Considering the options identified above, are there any benefits to providing application-layer retransmission information to the 5GC when PDU Set based handling is enabled? If so, SA4 would like to receive feedback on what type of application-layer retransmission information would be beneficial for PDU Set based handling in the 5GC.

SA2 Answer: The 5GC performs the QoS Flow binding for a service data flow based on the service data flow information, including e.g., the QoS requirements and packet filters of the service data flow. The QoS requirements include the data rate, the packet error rate, and packet delay requirements. If no such information is available, the service data flow normally is bound to the QoS Flow with the default QoS rules. In the LS, the information, to help determine the QoS requirements, on the frequency of the RTP retransmission packets (i.e., the number of the RTP retransmission packets per second) and how much percentage of the RTP retransmission packets compared to the source RTP packets is not provided. Based on this, SA2 wants SA4 to provide the further information for the RTP retransmission with the following question:

Question-1 to SA4: What are the QoS requirements (e.g., bitrate, error rate, latency) and packet filter information of the RTP retransmission service data flow? And how is these information provided to the 5GC?

If a service data flow is bound to a QoS Flow with PDU Set based handling, the PDU Set information also needs to be provided to the 5GC as defined in TS23.501 clause 5.37.5.2.

Question-2 to SA4: How the PDU Set information for the RTP retransmission is provided to the 5GC?

2. Actions:

To SA4

ACTION: 	SA2 kindly asks SA4 to provide answers to the above questions and welcome further information on RTP retransmission.

3. Dates of Next SA2 Meetings:

SA2#170	25th – 29th August 2025 		 Goteborg, SE

SA2#171	13th – 17th October 2025 		 TBD, CN