SA WG2 Meeting #S2-169	S2-2504942

19 - 23 May, 2025, Fukuoka, Japan

Source:	CATT

Title:	How to support Ethernet MA PDU Session using MPQUIC-E with one EPS leg and V-SMF

Document for:	Approval

Agenda Item:	19.13.2

Work Item / Release:	MASSS / Rel-19

Abstract: This paper proposes to change back from the fake "IP" type to the RAN to the real "ethernet" to the RAN to support the Ethernet MA PDU Session using MPQUIC-E in EPS and V-SMF cases

1. Introduction

In TS23.501, it is documented that "The MPQUIC-E functionality may be enabled when the type of the MA PDU Session is Ethernet. In case MPQUIC-E functionality is enabled for an Ethernet type PDU Session, the SMF indicates PDU Session type as IP in the N2 information to NG-RAN. ", i.e., a fake "IP" is provided to the NG-RAN.

However, CT4 WG has identified issues in two cases with this "fake" IP for the Ethernet MA PDU Session using MPQUIC-E.

The first case is described in Q1:

Q1: Can SA2 clarify the handling of MA PDU sessions using MPQUIC-E with a 3GPP access leg in EPS with respect to the PDN connection type to be signalled to the eNB?

For the Ethernet MA PDU Session using MPQUIC-E with a leg in EPS, the MME normally indicates PDN Connection type as Ethernet to the eNB, and now the traffic is IP type, the MME needs to be changed to indicate IP to the eNB, i.e., all MMEs in EPS need to be changed to support Ethernet MA PDU Session using MPQUIC-E.

Observation 1: For the Ethernet MA PDU Session using MPQUIC-E with a leg in EPS, all MMEs need to be enhanced and changed to indicate IP to eNB.

The Second case is described in Q2:

Q2: Can SA2 confirm CT4 assumption regarding the handling of MA PDU sessions using MPQUIC-E for HR PDU sessions and PDU sessions with an I-SMF.

For the HR Ethernet MA PDU Session using MPQUIC-E, the V-SMF normally indicates PDU Session type as Ethernet to the NG-RAN, and now the traffic is IP type, the V-SMF needs to be changed to indicate IP to the NG-RAN, i.e., all the V-SMFs in the VPLMN need to be changed to support Ethernet MA PDU Session using MPQUIC-E.

Observation 2: For the HR Ethernet MA PDU Session using MPQUIC-E, all V-SMFs in the VPLMN need to be enhanced and changed to indicate IP to NG-RAN.

MA PDU Session is not support in I-SMF

Based on the description of clause 5.32.1 and clause 4.23.5.1, the MA PDU Session is not supported in the I-SMF case. So the I-SMF case are not considered in the following solutions.

-----------------------------Start of excerpt from clause 5.32.1 --------------------

In this Release of the specification, support for ATSSS assumes SMF Service Areas covering the whole PLMN or that a MA PDU Session is released over both accesses when the UE moves out of the SMF Service Area.

-----------------------------End of excerpt from clause 5.32.1 --------------------

-----------------------------Start of excerpt from clause 4.23.5.1 --------------------

“If the service area of the selected SMF does not include the current UE location and the UE does not request for a MA PDU Session, the AMF selects an I-SMF that serves the area where UE” camps.

“If the service area of the selected SMF does not include the current UE location and the UE requests a MA PDU Session, then the AMF rejects the MA PDU Session Establishment procedure.”

-----------------------------End of excerpt from clause 4.23.5.1 --------------------

2. Solutions

Based on the above observations, 4 solutions are proposed as below, and their main cons for the above two cases are also provided.

2.1 The 4 solutions for the Ethernet MA PDU Session using MPQUIC-E

S1:  No support with EPS leg, and no support in the V-SMF.

S2:  Enhancements of the MME and V-SMF to provide the "IP" to RAN.

S3:  Change of providing "unstructured" instead of "IP" to RAN.

S4:  Change of providing "ethernet" instead of "IP" to RAN.

2.2 The 4 solutions for case 1 of EPS leg

S1: No support of the MA PDU Session using MPQUIC-E with EPS legs without any enhancement in MME;

Cons: MA PDU Session using MPQUIC-E cannot be supported with one leg in EPS, and cannot be supported in future releases because of the technical limitation of fake "IP" type to the RAN.

S2: Enhancements of all MMEs to provide the fake "IP" to the RAN;

Cons: Needs to enhance all the MMEs in the EPS.

S3: Change of the MA PDU Session type from "IP" to "unstructured" to the RAN.

Cons: All MMEs need to be enhanced to provide the fake "unstructured" information to the eNB, and there is NO ROHC in the RAN.

S4: Change of the MA PDU Session type from "IP" to "ethernet" to the RAN

Cons: A dummy Ethernet header is used in the user plane between the UE and UPF, and no changes to any MMEs.

To support Ethernet MA PDU Session using MPQUIC-E and case with EPS leg, it is allowed to change the functions of UE, UPF and SMF in specification, and is not allowed to change the functions of MME, V-SMF and RAN (in EPS/5G) in specification in the WID. Based on these requirements in the WID, only the S4 can fulfil the requirements.

Observation 3: To support Ethernet MA PDU Session using MPQUIC-E with one EPS leg and without changing the MME, only solution 4 changes the MA PDU Session type from "IP" to "ethernet" to the RAN can fulfil the function change requirements.

2.3 The 4 solutions for case 2 of V-SMF

S1: No support of the MA PDU Session using MPQUIC-E without any enhancement in V-SMF;

Cons: MA PDU Session using MPQUIC-E cannot be supported with HR and I-SMF cases, and cannot be supported in future releases because of the technical limitation of fake "IP" type to the RAN.

S2: Enhancements of V-SMF to provide the "IP" to RAN.

Cons: Needs to enhance all V-SMFs in all V-PLMNs.

S3: Change of PDU Session type from "IP" to "unstructured" to the RAN.

Cons: All V-SMFs need to be enhanced to provide the fake "unstructured" information to the NG-RAN, and there is NO ROHC in the RAN.

S4: Change of PDU Session type from "IP" to "ethernet" to the RAN.

Cons: A dummy Ethernet header is used in the user plane between the UE and H-UPF, and no changes on any V-SMFs.

To support Ethernet MA PDU Session using MPQUIC-E in HR and I-SMF cases, it is allowed to change the functions of UE, UPF and SMF in specification, and is not to change the functions of MME, V-SMF and RAN (in EPS/5G) in specification in the WID. Based on these requirements in the WID, only the S4 can fulfil the requirements.

Observation 4: To support Ethernet MA PDU Session using MPQUIC-E in V-SMF cases and without changing the V-SMF, only solution 4 changes the MA PDU Session type from "IP" to "ethernet" to the RAN can fulfil the function change requirements.

2.5 Dummy Ethernet Header

There are 5 cases using dummy IP/PDU packets in different clauses in TS23.501.The Dummy Ethernet Header can be defined by the stage 3, there are a lot of methods to generate the dummy Ethernet header as below, e.g. to Use the same MAC addresses of MA PDU Session user Ethernet traffic or using the MPQUIC link layer IP to generate the Ethernet MAC address for the UE and the UPF. The dummy Ethernet header are used only to carry the IP packet of the MPQUIC-E between the UE and the UPF. The MAC address in the dummy Ethernet header are not used in application and in N6. The MAC addresses of the MA PDU Session Ethernet frame can be used by the UE and the UPF to generate the MAC addresses of the dummy Ethernet header and the SMF does not allocate the MAC address of the dummy Ethernet header.

After the Ethernet MA PDU Session using MPQUIC-E is established, the UE adds the dummy Ethernet Header to the UL IP packet of MPQUIC-E traffic, and the UPF removes the dummy Ethernet Header from the UL Ethernet frame and provides the UL IP packet of MPQUIC-E to the MPQUIC-E proxy. The UPF adds the dummy Ethernet Header to the DL IP packet of MPQUIC-E traffic and the UE removes the dummy Ethernet Header from the DL Ethernet frame and provides the IP packet of MPQUIC-E traffic to the upper MPQUIC-E application.

Observation 5: There are a lots of methods to generating the dummy Ethernet header and the Dummy Ethernet Header can be defined by the stage 3.

2.6 EHC and ROHC Compression

Adding a dummy Ethernet Header does not block the ROHC for the above IP/UDP packets for the MPQUIC-E. As defined in clause 5.12.7 in TS38.323, the EHC (Ethernet Header Compression) and ROHC can be performed simultaneously.

Adding a dummy Ethernet Header also does not increase much traffic in the radio interface since the EHC can be performed.

---------------------------------------- Start of excerpt from clause 5.12.7 of TS38.323 ------------------------------------

5.12.7	Simultaneous configuration of ROHC and EHC

If both ROHC and EHC are configured for a DRB/MRB, the ROHC header shall be located after the EHC header. Figure 5.12.7-1 shows the location of the ROHC header and the EHC header in a PDCP Data PDU.

Figure 5.12.7-1: Location of ROHC header and EHC header in a PDCP Data PDU

If a PDCP SDU including non-IP Ethernet packet is received from upper layers, the EHC compressor shall bypass the ROHC compressor and submit the EHC compressed non-IP Ethernet packet to lower layers according to clause 5.2.1.

If a PDCP Data PDU including non-IP Ethernet packet is received from lower layers, the EHC decompressor shall bypass the ROHC decompressor and deliver the EHC decompressed non-IP Ethernet packet to upper layers according to clause 5.2.2.

---------------------------------------- End of excerpt from clause 5.12.7 of TS38.323 ------------------------------------

Observation 6: The EHC and ROHC can be configured simultaneously and no much traffic is added in the radio interface.

3. Proposal

Based on the above observations, Solution 4 is best solution and is proposed to be selected, and the following proposals are proposed to support Ethernet MA PDU Session using MPQUIC-E:

Proposal 1: Change the PDU Session type from the fake "IP" to "ethernet" in TS23.501 and TS23.502.

Proposal 2: Support Ethernet MA PDU Session using MPQUIC-E with one EPS leg and HR roaming cases.

Proposal 3: Provide Reply LS to CT4 and CT1 with the above conclusion and approved CRs in TS23.501 and TS23.502. and inform CT4 that the MA PDU Session is not support in I-SMF.