3GPP TSG-WG2 Meeting #169	S2-2504945

May 19 – 23, 2025, Fukuoka, Japan		(revision of S2-250xxxx)

Source:	Ericsson

Title:	KI#5, a new solution: Baseline for exposing sensing results to an AF

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution:

This contribution proposes a new solution for KI#5 that aims at providing a baseline for exposing sensing results to an AF.

# 1. Discussion

This document proposes a new solution for KI#5 that aims at providing a baseline for exposing sensing results to an AF.

# 2. Text Proposal

It is proposed to agree the following solution to 3GPP TR 23.700-14.

>>>> Start of Changes <<<<

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 | #6 |
| #X | X |  |  |  | X |  |
| #2 |  |  |  |  |  |  |

>>>> Next Change (all new text)<<<<

## 6.X	Solution #XY: Exposing Sensing Results to an AF

### 6.X.1	High-level solution principles

The solution is based on following principles:

-	AF invokes a sensing service request to a Sensing Exposure Function (SEF) – a new entity in the core network. The SEF authorizes the AF request and, if authorized, discovers and selects a Sensing Core Function (SCF). The SCF is a new CN NF that handles the sensing service request in the core network.

-	The SCF discovers and selects NG-RAN node for the relevant Target Sensing Area of the authorized sensing service. The SCF instructs the selected NG-RAN to obtain the sensing data.

-	The NG-RAN provides the obtained sensing data to the SCF for sensing result determination and/or for further exposure to the AF via the SEF.

### 6.X.2	Impacts on the Key Issues

This solution addresses KI#5 aspect related to how the core network exposes sensing results to the sensing service consumer. However, the solution impacts also other key issue, and the impact could be the following:

-	KI#1: New core network functions are being considered – Sensing Core Function (SCF) that handles sensing request in the core network and Sensing Exposure Function (SEF) that can be used to expose the obtained sensing result to the sensing service consumer (i.e., AF that invoked the sensing service request)

-	KI#2: AF authorization is performed by the SEF.

-	KI#3: SCF selection is performed by the SEF; the SCF selects an NG-RAN node to perform sensing data collection.

-	KI#4: NG-RAN obtained sensing data are transferred to the SCF for sensing result determination and, if applicable, to the SEF for the exposure to the AF.

-	KI#6: SCF includes the sensing operation specific parameters as inputs inside a sensing request to the selected NG-RAN node.

### 6.X.3.	Functional Description

The following scenario is considered for this solution. An application function (AF) invokes a sensing service to request the network to perform either (i) environment monitoring, (ii) motion monitoring, (iii) object detection, or (iv) object detection and tracking. The AF also may request a sensing result detail level to be exposed, i.e. sensing outcome (e.g., indication whether the object has been detected or not) or a more detailed description of what has been sensed by the 5GS (e.g. sensing data is a certain form). In this case, the AF acts as a sensing service consumer and a sensing service requester. The AF request is sent to a new NF – Sensing Exposure Function (SEF) that performs authorization of the AF requested sensing service.

If the AF request is authorized, the SEF selects a Sensing Core Function (SCF) that handles the sensing service request and the invokes the corresponding sensing service operation to the selected SCF. The SCF discovers and selects a NG-RAN node(s) to handle the core network-requested sensing service (e.g., object tracking in a certain area).

The NG-RAN node performs sensing and obtain sensing data. The NG-RAN node provides the obtained sensing data to the SCF for sensing result determination and/or sensing data exposure, if applicable, to the AF via the SEF.

NOTE X1:	Roles and functionalities of the involved NFs (e.g. SEF, SCF and NG-RAN) can change depending on the resolution of the ENs in the procedure, see clause 6.X.4.

NOTE X2:	The solution might need further updates and alignment with TSG RAN after TSGs#108.

### 6.X.4	Procedures

Procedure for sensing result exposure in cases when an AF invokes the sensing service towards a core network is shown in Figure 6.X.4-1.

Figure 6.X.4-1: Procedure of sensing result exposure to an AF.

1.	The Sensing service consumer (e.g. AF) invokes a sensing service request (e.g., Nsef_SensingEvent_Subscribe) to the SEF. Inside the request the AF includes the following mandatory and optional parameters:

Editor's note:	Whether the SEF can be a part of NEF is FFS and outcome of KI#1.

-	mandatory, AF Identifier;

-	mandatory, External Target Sensing Area information (e.g., geographical area information and, optionally, altitude information);

-	mandatory, Type of sensing to perform (e.g., SensingEventType = “Environment Monitoring”, “Motion Monitoring”, “Object Detection”, “Object Tracking”);

-	mandatory, sensing result detail level to report (e.g., notifications with, for example, an indication about the determined sensing result (e.g., “Object detected”, “Parameter estimated”) or a more detailed description of sensing data that can be used for the sensing result determination);

-	optionally, sensing periodicity (e.g., one-time reporting, periodic or event-triggered sensing);

-	optionally, sensing data exposure parameters (e.g., time and/or spatial resolution of the sensing data, type of streaming and transport mechanism to be applied in case the streaming of the obtained sensing data is being requested, etc.)

NOTE X3:	Other parameters that AF can include in a sensing service request can be added during the study phase.

NOTE X4:	The exact names of service operations and semantics can be determined in the later phase.

2.	The SEF performs the AF-requested sensing service authorization.

NOTE X5:	Data used for the AF-requested sensing service authorization and other details on how SEF authorizes the AF request is outcome of KI#2.

Editor's note:	Whether the SEF performs the consent check per AF identity is FFS and needs coordination with SA WG3.

If the AF request for sensing service is authorized, the SEF maps the External Target Sensing Area information in the AF request to a 3GPP-internal Target Sensing Area information and then discovers and selects a SCF to handle the sensing service request.

NOTE X6:	How a SCF is discovered and selected is outcome of KI#3.

Editor's note:	 How Target Sensing Area information needs to be described is FFS and requires coordination with RAN WGs.

3.	The SEF invokes the corresponding sensing service request (e.g. Nscf_SensingEvent_Subscribe) to the selected SCF and provides the relevant sensing service parameters from the AF request in Step 1.

4.	The SCF discovers and selects NG-RAN node(s) for the relevant Target Sensing Area of the authorized sensing service.

NOTE X7:	How the SCF discovers and selects NG-RAN node(s) to handle the AF-requested sensing service is covered by solutions for KI#3 of this study.

NOTE X8:	Whether the SCF needs to perform authorization of sensing operations is FFS and depends on type of sensing operations, sensing entities involved, and outcome of KI#2 of this study.

5.	The SCF sends the sensing service request to the selected NG-RAN node. Inside the request, the SCF includes, for instance, Target Sensing Area information, type of sensing to perform, relevant parameters from the AF request in Step 1, and other parameters derived by the SCF for the authorized sensing service.

6.	Based on the received sensing service request from the SCF, the NG-RAN starts obtaining sensing data.

Editor's note:	How sensing measurements are performed and how the NG-RAN obtains the sensing data are RAN WGs decisions and are FFS.

Editor's note:	Whether the NG-RAN can also perform determination of a sensing result depends on RAN WGs decision together with the study.

7.	The NG-RAN provides the sensing data to the SCF for sensing result determination, sensing result and/or sensing data exposure if applicable.

NOTE X9:	How the NG-RAN provides the obtained sensing data to the SCF is covered by KI#4 of this study.

8a.	The SCF may determine the sensing result considering the sensing result detail level included in the AF request in Step 1.

NOTE X10:	How the SCF uses the sensing data to determine a sensing result is up to implementation.

9a.	The SCF may notify the SEF using the Nscf_SensingEvent_Notify service operation about the determined sensing result.

10a.	The SEF forwards the received notification from the SCF to the AF using the Nsef_SensingEvent_Notify service operation.

8b.	The SCF may process the received sensing data considering the sensing result detail level from the AF request in Step 1, and then the SCF may provide the processed sensing data to the SEF for the exposure.

NOTE X11:	How the SCF processes the sensing data considering the sensing result detail level is up to implementation.

NOTE X12:	How the SCF provides the sensing data to the SEF is covered by KI#4 of this study.

9b.	The SEF may cache the provided sensing data and prepare the sensing data for exposure using the parameters for sensing data exposure from the AF request in Step 1.

10b.	The SEF may expose the sensing data to the AF according to the sensing result detail level and sensing data exposure parameters authorized for the AF request.

### 6.X.5	Impacts on existing services, entities and interfaces

AF:

-	acts as a Sensing Service Consumer by sending a sensing service request to the core network and consuming the sensing result and sensing data (if applicable).

Sensing Exposure Function (SEF):

NOTE X13:	Whether SEF has any impact on the NEF depends on KI#1 outcome.

-	performs AF request authorization for the sensing service;

-	discovers and selects a Sensing Core Function (SCF) for handling the sensing service in the core network;

-	maps an External Target Sensing area information to an internal Target Area Sensing Area information;

-	may cache and expose the determined sensing result and/or sensing data, if applicable, to a sensing service consumer (i.e., AF).

Sensing Core Function (SCF):

-	discovers and selects an NG-RAN node capable of sensing for a specific service;

-	may collect the obtained sensing data from the NG-RAN for determination of a sensing result;

-	may determine a sensing result based on the collected sensing data;

-	may process and provide the received sensing data to the SEF for exposure to the AF;

-	may notify the AF (via SEF) about the sensing result.

NG-RAN:

-	may obtain sensing data based on the sensing service request from the SCF;

-	may provide the collected sensing data to the SCF.

>>>> End of Changes <<<<