SA WG2 Meeting #S2-169	S2-2504954

19 - 23 May, 2025, Fukuoka, Japan	(revision of S2-250xxxx)

Source:	ZTE

Title:	KI#1, a new solution for renewable energy information collection and exposure

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This paper proposes to add a new solution to Key issue#1, Enhancements of collection, determination and exposure of Energy-related information

1. Discussion

The Key issue#1 of this study is to enhance the collection, determination and exposure of Energy-related information.

This key issue will study whether and how to enhance the collection, determination and exposure of energy related information in the 5GS.

NOTE 1:	If applicable, whether and how information is collected from OAM requires coordination with SA WG5.

NOTE 2:		Possible accuracy enhancement of energy consumption calculations considering RAN reporting will be considered only after RAN WGs supports related enhancement.

In the last meeting, SA5 has send the LS (S2-2502843) on the progress on the renewable energy information in SA5.

SA2 Question 1: SA2 would like SA5 to provide feedback on whether and what the Node-level renewable energy information can be collected from OAM.

SA5 Answer: the 3GPP management system can already provide the following information to authorized consumers:

- Per gNB Data Volume and Energy Consumption;

- Per UPF Data Volume and Energy Consumption;

In its Release 19 work item Energy_OAM_Ph3, SA5 has amongst objectives to specify energy supply, carbon emission and renewable energy information relative to gNBs and 5G NFs.

SA2 has agreed that the renewable energy information collection and exposure is moved to R20. This paper proposes the solution according to the SA5 reply LS.

2. Proposal

It is proposed to accept the change to 23.700-67.

>>>> Start of Change <<<<

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	The number of columns of Key Issues will be updated according to clause 5.

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solution# | 1 | 2 | 3 |
| 1 | X |  |  |
| 2 | X | X |  |
| 3 |  | X |  |
| 4 |  | X |  |
| 5 |  | X |  |
| 6 |  | X |  |
| 7 |  | X |  |
| X | X |  |  |

>>>> Next Change <<<<

## 6.X	Solution #X: Renewable energy information collection and exposure.

### 6.X.0	High-level solution principles

The AF may request the energy consumption information including the renewable energy information.

The EIF also collects the per node/NF level Renewable energy information from OAM. Then the EIF calculate the renewable energy consumption and return to the service consumer.

### 6.X.1	Description

This solution addresses Key Issue #1.

AF/NEF or 5GC NF subscribes the EIF for Energy consumption information of required granularities (UE, S-NSSAI, PDU session and/or Service Data Flow) with Renewable energy consumption indication. It indicates the EIF service consumer wants to obtain the Renewable energy consumption information.

In the Rel-19, the EIF retrieve the Node-level energy consumption information, Node-level data volume from OAM with interval T.

In this solution, the OAM also provides the following Node-level renewable energy information within the interval T:

- Renewable energy type

- Renewable energy consumption or Renewable energy ratio

NOTE:	The details of renewable energy information from OAM will depend on SA5 R19 normative work progress.

The EIF retrieves the data volume of the required granularities (i.e. S-NSSAI, PDU Session and/or Service Data Flow) from SMF as defined the clause 5.51.2 of TS 23.501 [2].

The Renewable energy consumed at a UPF (ERenew, UPF) or a gNB (ERenew, gNB) is known over a time interval T.

The EIF calculates the overall Energy consumption information of required granularities (UE, S-NSSAI, PDU session and/or Service Data Flow) as defined in the Annex T of 23.501 [2].

Taking the PDU session level granularity as an example, EIF calculates renewable energy consumption as follows:

ERenew, Session,gNB=ERenew,gNBDVSession,gNBDVgNB

ERenew,Session, UPF=ERenew,UPFDVSession,UPFDVUPF

ERenew,Session=gNBi∈gNBSessionTERenew,Session, gNBi+ UPFi∈UPFSessionTERenew,Session, UPFi

### 6.X.2	Procedures

The procedure in the clause 4.29.2 of TS 23.502 [3] is re-used with following addition change:

- Step 1: The service consumer indicate the renewable energy information

- Step 6: The EIF retrieves the per node level renewable energy information from OAM.

- Step 7: The EIF exposes the renewable energy consumption to service consumer.

### 6.X.3	Impacts on existing services, entities and interfaces

AF/NEF:

-	Indicate the renewable energy information to EIF.

EIF:

-	retrieves the per node level renewable energy information from OAM

-	calculate and exposes the renewable energy consumption information to service consumer.

>>>> End of Change <<<<