SA WG2 Meeting #168	S2-2505002

19th – 23th May, 2025, Fukuoka, Japan

Source:	ETRI

Title:	New Solution to KI#1: General Framework for UE Data Collection over UP

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_ AIML_CN_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes a new solution for for KI#1 of FS_AIML_CN_Ph2.

1. Introduction

The current TR 23.700-04 outlines Key Issue #1, which addresses the transfer of data over the User Plane (UP) for UE data collection, as described below.

This key issue aims to provide solutions for UE data collection to meet the requirements for AI/ML for NR air interface with UE-side model training. This process requires the transfer of training data from the UE to the 5G core and then to the OTT server.

This key issue will investigate the following aspects:

-	How to support the standardized transfer of standardized data over UP for UE data collection including how to initiate, terminate and manage UE data transfer over the UP connection.

-	How to support in 5GC the full visibility of standardized UE data contents.

-	How 5GC can support the full controllability of standardized UE data transfer.

-	How to differentiate the traffic for UE data collection from the UE regular traffic in the 5GC.

-	Whether and how the UE knows about what the standardized data to be transferred are.

This solution addresses Key Issue #1, with reference to the User Plane connection establishment procedures specified in Clause 6.18.1 of TS 23.273 [6] and Clause 6.2.1 of TS 24.572 [7].

The following technical elements address the above key issue.

-	The use of 'UE Data Collection' request from the DCF can indicate the initiation, termination and modification of UE data transfer over the UP connection.

-	The use of 'UE Data Collection' messages from the DCF can support full 5GC controllability of standardized UE data transfer.

-	The DCF’s process of verifying the 'UE Data Collection' data can support full 5GC visibility into standardized UE data transfer.

-	UP information in the 'UE Data Collection' request and the destination address of the 'UE Data Collection' traffic packet can support traffic differentiation.

-	Data collection parameters in the 'UE Data Collection' request inform the UE of the standardized data to be transferred.

2. Proposal

It is proposed to agree the following updates to TR 23.700-04.

*** Start of Change ***

6.0	Mapping of Solutions to Key Issues and Use Cases

Table 6.0-1: Mapping of Solutions to Key Issues and use cases

|  | Key Issues |  | Use Cases (optional) |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Use Case #1> | <Use Case #2> |
| #1 | x |  |  |  |
| #2 | x |  |  |  |
| #3 | x |  |  |  |
| #4 | x |  |  |  |
| #x | x |  |  |  |

*** Next Change (all new text) ***

6.X	Solution #X: General Framework for UE Data Collection over UP

6.X.1	High level principles

High level principles of the solution are:

-	Data Collection Function (DCF) controls and manages standardized UE data collection for UE-side model training. It performs the following functions:

-	Provides the UE with 'UE Data Collection' request via NAS signaling. The 'UE Data Collection' request contains UP information and data collection parameters.

-	Collects standardized UE data over the user plane (UP).

-	Verifies and transfers the collected data to the AF, i.e. OTT server.

-	The URSP is used to configure how to select the route for standardized UE data collection, such as determining the associated DNN, S-NSSAI, and/or PDU Session type for 'UE Data Collection' traffic.

-	The UE registers with the 'UE Data Collection' capability, allowing the AMF to identify UEs with this capability in a specified Area of Interest (AoI).

-	The UE establishes a PDU Session to the specified DNN and S-NSSAI that can be used for standardized UE data collection

-	The destination address of the 'UE Data Collection' traffic is the DCF address.

-	The NRF is used for the discovery of the DCF that provides 'UE Data Collection' service.

6.X.2	Description

This solution is for Key Issue#1 for UE data collection over UP.

The following terminologies are used specifically in the context of this solution.

-	OTT server is a trusted AF or untrusted AF that performs UE-side model training using 'UE Data Collection' data.

-	'UE Data Collection' service is a 5G core network service provided to the AF, i.e., the OTT server.

-	'UE Data Collection' data refers to data collected at the UE and the data provided to the OTT server. This data may be verified and aggregated by the DCF before being delivered to the OTT server.

-	Data Collection Function (DCF) is a 5GC network function (NF) that controls and manages UE data collection for UE-side model training.

Editor's note:	Whether the DCF is a new NF or the existing one is FFS.

-	'UE Data Collection' messages are exchanged between the UE and the DCF to manage data collection over the User Plane. These include the 'UE Data Collection' request and 'UE Data Collection' response messages.

-	The 'UE Data Collection' request is a message sent by the DCF to the UE to initiate, terminate or modify data collection for UE-side model training. It includes UP information and data collection parameters.

Editor's note:	Whether to include initiation, termination, or modification operations in the 'UE Data Collection' request message, or to define separate messages for each operation, is FFS.

-	The 'UE Data Collection' response is a message sent by the UE to the DCF to acknowledge the corresponding request.

-	The Correlation ID is a unique identifier used to associate related 'UE Data Collection' messages across the UE, AMF, and DCF. It is used by the AMF to route these messages appropriately. The Correlation ID consists of a DCF ID and a connection ID, allowing support for multiple DCFs. The connection ID is allocated by the DCF.

Editor's note:	The format of the Correlation ID is FFS.

-	'UE Data Collection' traffic refers to one or more data packets transmitted from the UE, which are intended to be used for UE-side model training.

-	'UE Data Collection' capability indicates that the UE is capable of collecting and transmitting 'UE Data Collection' traffic. It may be included in the registration request message.

-	UP information is used by the UE to generate and transmit 'UE Data Collection' traffic packets. It includes the IP address or FQDN of the DCF.

Editor's note:	Whether UP information includes additional content is FFS.

-	Data collection parameters define the data to be collected and transmitted as 'UE Data Collection' traffic by the UE. The data collection parameters include types of data, number of data samples, and/or time window.

Editor's note:	Whether data collection parameters include additional content is FFS.

This solution refers to the user plane connection establishment described in clause 6.18.1 of TS 23.273 [6] and in clause 6.2.1 of TS 24.572 [7].

The following behaviors are described in this Solution.

-	The DCF controls and manages UE data collection for UE-side model training.

-	The DCF is provisioned by the AF with target UEs and the data collection parameters. The target UE may be a specific UE, a list of UEs, or any UEs located in one or more Areas of Interest (AoIs). The data collection parameters includes types of data, number of data samples, and/or time window.

-	The DCF may resolve the list of UEs by querying the AMF to obtain UEs with the 'UE Data Collection' capability located in the specified AoIs.

-	The DCF checks user consent with the assistance of the UDM.

-	The DCF provides the UE with UP information and data collection parameters over NAS signaling. The UP information includes the DCF's address or FQDN for receiving 'UE Data Collection' traffic, and the data collection parameters include types of data, number of data samples, and/or time window.

-	The UE registers with the 'UE Data Collection' capability, and the AMF manages UEs that support this capability.

-	The PCF provides the UE with URSP rules that include route selection descriptors for 'UE Data Collection' traffic towards the DCF.

-	The UE establishes a PDU Session that can be used for standardized UE data collection.

-	The destination address of the UE Data Collection packet corresponds to the DCF address as provided in the UP information.

-	The UPF forwards the UE Data Collection packet from the UE to the DCF.

-	The UDM stores the user's consent regarding UE data collection for UE-side model training.

-	The DCF verifies the data collected from the UE and sends a notification to the OTT server. If required, the data may be aggregated before transmission.

The 'UE Data Collection' traffic can be differentiated from the regular UE traffic by the UPF:

-	If the URSP specifies a dedicated DNN or S-NSSAI for 'UE Data Collection' traffic, the traffic is carried in a specific PDU Session designated for 'UE Data Collection'.

-	If the URSP does not specify a dedicated DNN or S-NSSAI, the 'UE Data Collection' traffic can be detected by the UPF using a Packet Detection Rule (PDR) that matches the destination address corresponding to the DCF.

6.X.3	Procedures

6.X.3.1	Initiation of UE data collection transfer

Figure 6.x.3.1-1: Initiation of UE Data Collection over the User Plane

1.	The DCF registers 'UE Data Collection' service with the NRF as described in clause 4.17.1 of TS 23.502 [3].

2a.	The UE follows the registration procedure defined in Clause 4.2.2.2 of TS 23.502 [3], indicating support for the 'UE Data Collection' capability.

2b.	The PCF provides the UE with URSP rules that include route selection descriptors, such as DNN, S-NSSAI, and/or PDU session type, for 'UE Data Collection' traffic as described in clause 4.2.4.3 of TS 23.502 [3].

3.	Trusted AF, i.e., an OTT server, discovers the DCF that provides 'UE Data Collection' service as described in clause 4.17.4 of TS 23.502 [3]. For an untrusted AF's request for 'UE Data Collection' service, the request is authorized by the NEF, which then discovers the DCF.

4.	The AF or the NEF requests subscription to the 'UE Data Collection' service, which may include input parameters such as a list of UEs or any UEs in one or more Areas of Interest (AoIs), types of data, number of data samples, and/or time window.

5.	The DCF may retrieve a list of UEs located in the specified AoI(s), if one or more AoIs are provided in step 4. If the AoIs span multiple AMFs, the DCF may retrieve the UE lists with 'UE Data Collection' capability from each corresponding AMF.

6.	The DCF may check user consent by retrieving subscription data from the UDM.

7. 	The DCF may subscribe to notifications about changes in user consent for consented UE.

8. 	The DCF sends a 'UE Data Collection' request with initiation to the UE along with a correlation ID. The request may include UP information and data collection parameters. The UP information may contain IP address or FQDN of the DCF to be used by the UE for data transfer over User Plane. Data colleciton parameters may include types of data, number of data samples, and/or time window. The AMF stores the correlation ID for subsequent routing of 'UE Data Collection' messages.

9. 	The AMF may trigger a Service Request procedure for a UE in CM-IDLE state.

10. The AMF forwards the 'UE Data Collection' request and the correlation ID via DL NAS Transport message.

11. The UE may establish a PDU Session if no UP connection is available for 'UE Data Collection'. The UE may use the URSP rules to determine the parameters for PDU Session establishment. The SMF configures Forwarding Action Rules (FARs) for the PDU Session.

12. The UE starts collecting data according to the request from the DCF.

13. The UE sends 'UE Data Collection' response and the correlation ID via UL NAS Transport message.

14. The AMF forwards the 'UE Data Collection' response to the DCF.

15. The UE sends the collected data over the UP connection to the target DCF address. The UPF forwards the data traffic from the UE to the DCF based on the configured FARs.

16. The DCF verifies that the data collected from the UE conforms to the parameters specified in the request. If required, the DCF may perform data aggregation.

17. The DCF sends a notification to the OTT server containing the collected data. In the case of an untrusted AF, i.e., an OTT server, the DCF provides the collected data to the OTT server (i.e., the untrusted AF) through the NEF.

6.X.3.2	Modificaiton and Termination of UE data collection transfer

Figure 6.x.3.2-1: Termination of UE Data Collection over the User Plane

0.	The DCF determines that data collection needs to be modified. This may be triggered by an AF request to subscribe or unsubscribe from the 'UE Data Collection' service.

1.	The DCF sends a 'UE Data Collection' request with a modification or termination instruction to the UE, along with a correlation ID. The request may include data collection parameters to be modified, such as types of data, number of data samples, and/or time window.

2. The AMF forwards the 'UE Data Collection' request and the correlation ID to the UE via a DL NAS Transport message. If the UE is in CM-IDLE state, the AMF may trigger a Service Request procedure before sending the DL NAS Transport message.

3. The UE may release the PDU Session if the UP connection is no longer required.

4. The UE sends a 'UE Data Collection' response and the correlation ID via an UL NAS Transport message.

5. The AMF forwards the 'UE Data Collection' response to the DCF.

6.X.4	Impacts on services, entities and interfaces

UE:

-	Handles the sending and receiving of a new payload type related to 'UE Data Collection' in NAS signaling.

-	Gathers the data specified in the 'UE Data Collection' request.

-	Transfers the gathered data to the DCF over the UP.

DCF:

-	Registers 'UE Data Collection' service to the NRF.

-	Handles the 'UE Data Collection' subscription request from the NEF or the OTT server.

-	Obtains a list of UEs with 'UE Data Collection' capability for the specified AoIs.

-	Checks the user consent of the UE.

-	Handles the sending and receiving of 'UE Data Collection' messages.

-	Allocates a correlation ID for the UP connection used for 'UE Data Collection'.

-	Handles the reception, verification, and processing of 'UE Data Collection' traffic.

-	Notifies the NEF or the OTT server with the collected 'UE Data Collection' data.

OTT server:

-	Subscribes to 'UE Data Collection' data from the NEF or the DCF for UE-side model training.

NEF:

-	Supports 'UE Data Collection' exposure of 5GC to the OTT server.

NRF:

-	Supports DCF registration and discovery service for 'UE Data Collection' service.

AMF:

-	Supports routing of 'UE Data Collection' messages using correlation ID.

-	Supports forwarding of 'UE Data Collection' messages between NAS signaling and SBI.

-	Introduces a new payload type used for 'UE Data Collection' messages in NAS signaling.

-	Exposes the UE list with UE data collection capability in the specified AoI(s)

PCF：

-	Provides the UE with URSP rules that include Route Selection Descriptors for 'UE Data Collection' traffic.

*** End of Changes ***