SA WG2 Meeting #169	S2-2505010

Fukuoka, Japan, 19 – 23 May 2025	(revision of S2-250xxxx)

Source:	LG Electronics

Title:	Clarification on interaction between NEF and AF

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: The paper clarifies interaction between NEF and AF.

1. Discussion

Regardless whether AF is a trusted AF or an untrusted AF, it is considered that the AF can interact with AIOTF via NEF for AIoT service operations depending on operator's deployment scenario regarding NEF service operation.

However, these parts read that deployment scenario which the trusted AF interacts with AIOTF via NEF is precluded.

4.5.4 NEF In addition to the functions defined in TS 23.501 [3], the NEF performs the following functions: - Providing a service exposure API to AFs of 3rd party for AIoT services. - Interacting with AF of 3rd party and AIOTF. - Selection of AIOTF for AIoT services. - Authorization of the untrusted AF’s AIoT operation request.

6.2.3 Command Procedure … 1. The AF sends the Nnef_AIoT_Command (in case of untrusted AF) Request (AF ID, Command Type, AIoT Device identification information, [External Target Area information], [Approximate number of AIoT Devices], [Approximate D2R message size], AIoT data) message to NEF. …

Therefore, it is proposed to clarify that not only the untrusted AF but also the trusted AF can interact with AIOTF via NEF.

2. Proposal

It is proposed to capture the following changes into TS 23.369v0.3.0.

* * * * Start of Changes * * * *

4.5.3	AIOTF

The AIOTF supports the following functions:

-	Termination of AIoT NAS protocol with AIoT Device.

-	Connectivity with an NG-RAN via a direct interface reference point or via an AMF.

-	Support of AIoT service operations towards the AIoT Device(s):

-	Providing an interface to the AF (or via NEF) for AIoT services.

-	Authorizing the trusted AF's AIoT service operation request.

-	Triggering the NG-RAN to perform AIoT service operations towards the AIoT Device(s), and optionally determining and providing assistance information to the NG-RAN.

-	Report the service operation results to AF (or via NEF) based on the local configuration or the AF request.

-	NG-RAN selection and optionally a list of RAN Reader selection for AIoT service operations.

-	AMF selection based on target area information when AIOTF connects the NG-RAN indirectly via an AMF.

-	Correlation ID allocation corresponding to the AF service operation request.

-	Retrieving AIoT device profile data from ADM.

-	Retrieving AF subscription data from ADM.

-	Optionally AIoT Device context management.

-	Perform aggregation of the AIoT service responses, determine and provide assistant for the AIoT aggregation in NG-RAN.

NOTE:	Authentication of AIoT Devices related to AIOTF is defined in TS 33.369 [9].

* * * * Start of Next Change * * * *

4.5.4	NEF

In addition to the functions defined in TS 23.501 [3], the NEF performs the following functions:

-	Providing a service exposure API to AFs of 3rd party for AIoT services.

-	Interacting with AF of 3rd party and AIOTF.

-	Selection of AIOTF for AIoT services.

-	Authorization of the AF’s AIoT operation request.

* * * * Start of Next Change * * * *

6.2.3	Command Procedure

Editor's note:	Additional information in the steps, parameters, and their naming throughout the procedures requires alignment with other clauses and references adding as required.

Editor's note:	Alignment is required for how to document and describe Direct Connectivity and Indirect Connectivity options in the procedures.

Figure 6.2.3-1 depicts the command procedure.

The procedure focuses on the messages and parameters used for the communication between AIOTF and NG-RAN regardless of the path to access NG-RAN, see clause 4.2.2.1. The handling of the different communication paths is described in clause 6.2.4.

Figure 6.2.3-1: Command Procedure

1.	The AF sends the Nnef_AIoT_Command Request (AF ID, Command Type, AIoT Device identification information, [External Target Area information],  [Approximate number of AIoT Devices], [Approximate D2R message size], AIoT data) message to NEF.

Editor's note:	It is FFS whether and how to structure the AIoT data if the Command Type is Read, Write or Disable.

The AIoT Device identification information may include one or more AIoT Device ID(s) or the filtering information which is used to associate with multiple AIoT devices.

2.	The NEF selects the AIOTF(s) as described in clause 5.3.1. If no AIOTF can be selected, the NEF rejects the AIoT Command request with an appropriate cause code and step 6 is performed before ending the procedure.

3.	The NEF sends Naiotf_AIoT_Command Request message (AF ID, Command Type, AIoT Device identification information, [Target area information], , [Approximate number of AIoT Devices], [Approximate D2R message size], AIoT data) message to the selected AIOTF.

4.	The AIOTF receives the AIoT command operation request and checks the parameters included in the request. The AIOTF performs NG- RAN and optionally RAN Reader selection as specified in clause 5.3. If no NG-RAN or RAN Reader can be selected, the AIOTF rejects the AIoT Command request with an appropriate cause code.

The AIOTF generates a Correlation ID corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request.

The AIOTF determines assistance information as described in clause 5.4.

The AIOTF performs AF authorization for AIoT command operation as described in clause 5.6.

The AIOTF performs AMF selection as described in clause 5.3.4.

5.	AIOTF sends the Naiotf_AIoT_Command Response message (accept or reject, [cause code]) to the NEF.

6.	NEF sends the Nnef_AIoT_Command Response message (accept or reject, [cause code]) to the AF. If the response was a reject the procedure stops here.

7.	Step 7 to step11 of procedure for Inventory specified in clause 6.2.2 are performed with the following clarifications:

-	In step 11, the AIOTF validates the results as specified in TS 33.369 [9], and determines whether the command should be sent to an AIoT Device, e.g., by checking the Target AIoT device information.

If none of successful Inventory response is received, Step 8 -11 is not performed and the AIOTF sends a failure report to the NEF in Step 12.

8.	For each successful Inventory response received, the AIOTF sends Command Request message (Correlation ID, [Reader ID],  NAS Command Request, [Approximate D2R message size]) to the NG-RAN directly or as an AIoT Transfer Container via an AMF as specified in clause 6.2.4. The NAS Command Request message includes the AIoT data.

Editor's note:	What parameter(s) are used in the Command Request to enable NG-RAN node to target a specific AIoT Device requires coordination with RAN3.

Editor's note:	Additional information included in the NAS Command Request for security will be determined and aligned with SA WG3.

9.	The NG-RAN sends the AS R2D message (NAS Command Request) to the AIoT Device.

10.	The AIoT Device sends the AS D2R message (NAS Command Response) to the NG-RAN. The NAS Command Response message may include the AIoT data.

Editor's note:	The AS R2D message and AS D2R message will be aligned with RAN WG's specification.

Editor's note:	Additional information included in the NAS Command Response for security will be determined and aligned with SA WG3.

11.	The NG-RAN responds with a Command Response message (Correlation ID, Reader ID, NAS Command Response) to the AIOTF directly or as an AIoT Transfer Container via an AMF as specified in clause 6.2.4.

12.	The AIOTF reports the result of the AIoT Command request to the NEF by sending the Naiotf_AIoT_Command Notify message (AIoT Device ID(s), AF ID, AIoT data).

13.	The NEF informs the AF of the result of the AIoT_Command request by sending the Nnef_AIoT_Command Notify message (AIoT Device ID(s), AF ID, AIoT data).

* * * * End of Changes * * * *