SA WG2 Meeting #169	S2-2505011

Fukuoka, Japan, 19 – 23 May 2025	(revision of S2-250xxxx)

Source:	LG Electronics

Title:	Removal of general sub-clause under clause 7

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: The paper proposes to remove clause 7.1.

1. Discussion

Clause 7.1 seems not needed, so it is proposed to remove clause 7.1, and to adjust clause numbering for clauses 7.2 to 7.5.

2. Proposal

It is proposed to capture the following changes into TS 23.369v0.3.0.

* * * * Start of Changes * * * *

7	Network Function Services

7.1	AIOTF services

7.1.1	General

The AIOTF supports to expose AIoT services towards the AF or the NEF as described in Table 7.1.1-1.

Table 7.1.1-1: NF services provided by the AIOTF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Naiotf_AIoT | Inventory | Request/Response | NEF, AF |
|  | Command | Request/Response | NEF, AF |
|  | Notify | Subscribe/Notify | NEF, AF |

Editor's note:	It is FFS how to support the service between AMF and AIOTF in order to report and update the information provided by the RAN for indirect connectivity.

Editor's note:	It is FFS whether to use separate service operation to respectively support read, write or disable command procedure.

7.1.2	Naiotf_AIoT_Inventory service operation

Service operation name: Naiotf_AIoT_Inventory.

Description: The NF consumer requests an inventory operation for one or multiple AIoT Device(s).

Inputs, Required:

1)	AF ID.

2)	At least one of the following parameters are included:

-	Target area information for the inventory operation.

-	Information about the target AIoT Device(s):

-	either the AIoT Device ID(s) or the filtering information for multiple AIoT Devices.

Inputs, Optional:

1)	Information to be used for resource allocation:

-	Approximate number of AIoT Devices.

Outputs, Required: Transaction ID, Result indication (Success or Failure), Failure Cause in case of Failure.

Outputs, Optional: None.

7.1.3	Naiotf_AIoT_Command service operation

Service operation name: Naiotf_AIoT_Command.

Description: The NF consumer requests a command operation for one or multiple AIoT Device(s).

Inputs, Required:

1)	AF ID.

2)	At least one of the following parameters are included:

-	Target area information for the command operation.

-	Information about the target AIoT Device(s):

-	either the AIoT Device ID(s) or the filtering information for multiple AIoT Devices.

4)	Command Type: Read, Write or Permenant Disable.

5)	Command specific parameters:

Editor’s note:	The AIoT Data specific for the Read/Write/Disable command is FFS.

Editor’s note:	The Command specific parameters for Read/Write/Disable command is FFS.

Inputs, Optional:

1)	Information to be used for resource allocation:

-	Approximate number of AIoT Devices.

-	Approximate message size from the AIoT Device for Read Operation.

Editor’s note:	It is FFS whether the Approximate message size from the AIoT Device can also be provided by the AF for the other command operations.

Outputs, Required: Transaction ID, Result indication (Success or Failure), Failure Cause in case of Failure.

Outputs, Optional: None.

7.1.4	Naiotf_AIoT_Notify service operation

Service operation name: Naiotf_AIoT_Notify

Description: The AIOTF uses this service operation to notify the results or status of the service operation towards the NF consumers. If the NF consumer invokes the Naiotf_AIoT_Inventory, or Naiotf_AIoT_Command service operation, the NF consumer implicitly subscribes to the results of the requested service operation.

Inputs, Required:

1)	Common report information: Transaction ID, List of AIoT Device ID or Failure Cause in case of Failure).

2)	Read command specific report information: Information obtained from each target AIoT Device.

Editor’s note:	It is FFS whether that fail to discovery any AIoT device is normal operation result or failure case.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

Editor’s note:	It is FFS how to make AF be aware of the report is finished in case there are multiple notify for the same AIoT operation.

7.2	AMF services

7.2.1	General

AMF supports to expose AIoT services towards the AIOTF as described in Table 7.2.1-1. The Namf_AIoT AMF service is used when NG-RAN and the AIOTF communicate indirectly via an AMF.

Table 7.2.1-1: NF services provided by the AMF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Namf_AIoT | MessageDelivery | Request/Response | AIOTF |
|  | Notify | Subscribe/Notify | AIOTF |

7.2.2	Namf_AIoT_MessageDelivery service operation

Service operation name: Namf_AIoT_MessageDelivery

Description: The NF consumer requests to send AIoT data (i.e. AIoT inventory/command messages) towards NG-RAN or AIoT devices.

Inputs, Required:

1)	AIoT messages to deliver to NG-RAN.

2)	NG-RAN ID.

Outputs, Required: AIoT messages received from NG-RAN, Result indication (Success or Failure), Failure Cause in case of Failure.

Editor’s note:	It is FFS whether the Correlation identifier or AIOTF ID is included in the Namf_AIoT_MessageDelivery.

7.2.3	Namf_AIoT_Notify service operation

Service operation name: Namf_AIoT_Notify

Description: The NF consumer requests to receive AIoT data from NG-RAN or AIoT devices. If the NF consumer invokes the Namf_AIoT_MessageDelivery, the NF consumer implicitly subscribes to receive the AIoT data from NG-RAN or AIoT devices.

Inputs, Required:

1)	AIoT messages received from NG-RAN.

Input, Optional: None.

Outputs, Required: Operation execution result indication.

Output, Optional: None.

Editor's note:	AIoT messages is FFS and coordination with RAN3 is needed.

Editor's note:	It is FFS how to make AIOT be aware of the report is finished in case there are multiple notify for the same operation.

Editor's note:	It is FFS whether the Correlation identifier is included in the Namf_AIoT_Notify.

7.3	NEF services

7.3.1	General

In addition to those defined in clause 7.2.8 of TS 23.501 [3] and clause 5.2.6 of TS 23.502 [4], table 7.3.1-1 illustrates additional NEF services to support AIoT.

Table 7.3.1-1: NF Services provided by NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_AIoT | Inventory | Request/Response | AF |
|  | Command | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |

Editor’s note:	It is FFS whether to use separate service operation to respectively support read, write or disable command procedure.

7.3.2	Nnef_AIoT_Inventory service operation

Service operation name: Nnef_AIoT_Inventory

Description: The consumer requests to perform an inventory operation for an AIoT Device or multiple AIoT Devices.

Input, Required: AF ID, Target area information or AIoT Device ID(s) or AIoT Device ID filter information for the inventory operation.

Input, Optional: Approximate number of AIoT Devices.

Output, Required: AF Transaction ID, Result indication, Failure cause in case of Failure.

Output, Optional: None.

7.3.3	Nnef_AIoT_Command service operation

Service operation name: Nnef_AIoT_Command

Description: The consumer requests to perform a command operation for an AIoT Device or multiple AIoT Devices.

Input, Required: AF ID, Target area information or AIoT Device ID(s) or AIoT Device ID filter information for the command operation, Command type (Read, Write, or Disable).

Editor’s note:	The AIoT Data specific for the Read/Write/Disable command is FFS.

Input, Optional: Approximate number of AIoT Devices, Approximate message size from the AIoT Device for Read command type.

Editor’s note:	It is FFS whether the Approximate message size from the AIoT Device can also apply for the other command types.

Output, Required: AF Transaction ID, Result indication, Failure cause in case of Failure.

Output, Optional: None.

7.3.4	Nnef_AIoT_Notify service operation

Service operation name: Nnef_AIoT_Notify

Description: The consumer receives notification of the status or results of the requested service operation. If the consumer invokes the Nnef_AIoT_Inventory, or Nnef_AIoT_Command service operation, the consumer implicitly subscribes to the results of the requested service operation.

Input, Required: AF Transaction ID, a list of AIoT Device ID(s) or Failure Cause in case of Failure, Information obtained from each target AIoT Device.

Input, Optional: None.

Output, Required: Result indication.

Output, Optional: None.

Editor’s note:	The NEF AIoT service operation definition needs to be revisited to align with AIOTF services.

7.4	ADM services

7.4.1	General

The following table shows the Nadm_DM Service and Service Operations.

Table 7.4.1-1: NF services provided by ADM

| NF service name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nadm_DM | Query | Request/Response | AIOTF, NEF |
|  | Update | Request/Response | AIOTF |

7.4.2	Nadm_DM_Query service operation

Service operation name: Nadm_DM_Query

Description: NF service consumer may request the AIoT device profile data or the AF authorization data from the ADM.

Inputs, Required: AIoT Device Permanent ID, or AF ID.

Input, Optional: None.

Outputs, Required: the AIoT device profile data or the AF authorization data.

Output, Optional: None.

Editor's note:	Whether AF authorization data is stored in ADM is FFS.

7.4.3	Nadm_DM_Update service operation

Service operation name: Nadm_DM_Update

Description: NF service consumer may update the AIoT device profile data in the ADM.

Inputs, Required: AIoT Device Permanent ID, updated AIoT device profile data.

Input, Optional: None.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure.

Output, Optional: None.

* * * * End of Changes * * * *