SA WG2 Meeting #169	S2-2505022

19 - 23 May 2025, Fukuoka, Japan

Source:	Tencent, Tencent Cloud

Title:	KI#1, New Sol #X: Data collections for UE-side model training via user plane

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: This contribution proposes a way for data collections between UE and core network.

# 1. Discussion

3GPP SA2 Release 20 has approved the study item "AI/ML Core Network Enhancements Phase 2", which includes the research task of "whether and how to support the standardized transfer of standardized data over user plane for UE data collection." To enable the establishment of user plane connections from the UE to an NF, this paper proposes a way for data collections between UE and core network.

One key issue is proposed in TR 23.700-04 to provide solutions for UE data collection to meet the requirements for AI/ML for NR air interface with UE-side model training. This process requires the transfer of training data from the UE to the 5G core and then to the OTT server. This key issue will investigate the following aspects:

-	How to support the standardized transfer of standardized data over UP for UE data collection including how to initiate, terminate and manage UE data transfer over the UP connection.

-	How to support in 5GC the full visibility of standardized UE data contents.

-	How 5GC can support the full controllability of standardized UE data transfer.

-	How to differentiate the traffic for UE data collection from the UE regular traffic in the 5GC.

-	Whether and how the UE knows about what the standardized data to be transferred are.

# 2. Proposal

This paper proposes to add a solution for Key Issue #1 to TR 23.700-04 as follows.

---------- Start of change ----------

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue 1> | <Key Issue #y> |
| #1 | X |  |
| #2 |  |  |

---------- Next change (all new text) ----------

6.x	Solution #X: Data collections for UE-side model training via uplink

6.x.1 	     Description

This solution addresses Key Issue #1.

This solution proposes a data connection procedure for UE-side model training. The OTT Server received the user plane training data from UE for model training.

This solution has the following assumptions:

-   AI training related traffic and UE regular traffic can use the same or different PDU session which is determined by the URSP rule.

-	Data collection related function entity which can be NDWAF/DCCF or other entity select UEs to carry out AI training data collection which also initiate the data collection process.

-	UE is aware of what the standardized data to be transferred to partipate AI training.

6.x.2	Procedures

The procedure is shown below:

Figure 6.x.2-1: Data collections for UE-side model training via user plane;.

- The UE may use URSP configured by 5GC NF to select one existing PDU session or establish a new PDU session to configure the user plane for UE side data reporting towards OTT server.

- The OTT-Server (AF) requests UE data collection to NWDAF/DCCF or other NFs in core network via NEF.

- The NWDAF/DCCF or other NFs towards the AMF by invoking Namf_Communication_N1N2Transfer service operation to request the transfer of user plane connection message. The  NWDAF/DCCF or other NFs may determine which UE to be selected to perform data collection from other NFs in core network.

- The AMF transmits the data collection request and user plane configuration details to the UE(s) through a downlink (DL) NAS message to enable data reporting.

- UE setup a new PDU session to setup user plane for UE side data report.  If we differentiate the traffic for UE data collection from the UE regular traffic in the 5GC, the UE may establish one PDU session for user plane connection.

NOTE: The UE may establish the PDU session if there is no available PDU session or if there is existing PDU session for UE regular traffic but the PDU session is not used for UE side data reporting to OTT server for AI training.

- The selected UE receives the request and performs the data collection process. The UE may check and determine when to start data transfer. The UE may also collect data with the assistance of the RAN.

- The UE sends UE side data for AI/ML training through the user plane to NWDAF/DCCF or other NFs.

- The NWDAF/DCCF or other NFs uses the notification to send the UE training data to the OTT-Server.

6.X.3	Impacts on services, entities and interface

AMF

- The AMF transfer the user plane connection message received from NWDAF/DCCF or other NFs to UE via NAS connection.

NWDAF/DCCF or other NFs

- The  NWDAF/DCCF/other NFs determine which UE to be selected to perform data collection from other NFs in core network.

UE

- Recevie the report information from AMF for uplink data transmission.

- Support to collect the AI training data and send the collected data to the OTT server via user plane through the core network.

Data Collection Function:

-	Receives requests from consumer (UE-side server) to collect and transfer standardised data from the UE.

-	Determines configuration information for the UE to report standardised data.

-	Sends configuration information via an established session between the UE and the Data Collection Function.

---------- End of changes ----------