3GPP TSG-WG SA2 Meeting #169	S2- 2505024

# Fukuoka, Japan, May 19 – May 23, 2025

Source: 	vivo, Lenovo

Title: 	KI#1: discussion paper for the energy consumption calculation

Document for: 	Approval

Agenda Item: 	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes some observations and proposals for KI#1 in release 20 FS_EnergySys_Ph2.

1. Discussion

In the release 20 FS_EnergySys_Ph2 SID proposal, the following NOTE 3 was proposed in WT#1:

NOTE 3: WT#2 and WT#3 solutions may require one or more specific energy attributions. When applicable, the models for energy related parameters attributes at UE or NF level in WT#2 and WT#3 should be robust and accurate enough to be fit for purpose.

In TR 23.700-67, this NOTE was converted into the architecture requirements.

In release 19, the calculation formulas for energy consumption at UE level in EIF were proposed. However, considering the following issues, the energy consumption information provided by the EIF is not applicable for policy control in release 19. These issues should be further considered and enhanced in release 20 5GA EnergySys study or 6G related work.

- The current energy consumption calculation in EIF doesn’t consider the impact of the energy consumed for the basic operation of a node, (e.g., that enables an NF to be in idle/inactive state or in notEnergySaving state without handling any traffic and ready to be used for traffic handling,), which may not be directly related with the data processing and transmission tasks. This part of energy expenditure, which is not closely related with the UE data transmission task, is shared as the per bit energy consumption in release 19. This results in a fact in where the energy consumption information at a UE level not to be proportional to the data volume information at the related granularity.

The above aspect could be resolved by enhancing the energy related information obtained from OAM or the calculation formulas of energy consumption information in release 20 EnergySys study.

Observation 1: It is proposed to consider the impact of the energy consumed for the basic operation of a network node, e.g., when the node is in idle/inactive state or in notEnergySaving state without handling any traffic, to improve the accuracy and robustness of energy consumption at UE level in release 20 5GA EnergySys work.

- The current energy consumption calculation in EIF doesn’t consider the radio energy consumption level related information (which is subject to the relative location of the UE to the RAN). For example, the UE located at the cell edge consumes more energy compared to the one located at the cell center for the same amount of data volume transmission. The information reflecting the relative location of the UE within the cell helps to improve the accuracy of energy consumption information determination/estimation for the UE. Similarly, considering the interference level, i.e., when the interference is high a UE introduces a higher energy cost, e.g., due to retransmission.

In release 20 FS_EnergySys_Ph2 SID proposal, it captures the NOTE related with this aspect and its dependency on the RAN WGs. Considering the related situation, the following observation is proposed.

Observation 2: Radio energy consumption level related information helps to improve the energy consumption calculation formulas based on data volume in EIF. This aspect could be further studied after receiving feedback from the RAN WGs and shall be investigated in 6G related work.

- The energy related analytics information could be based on the historic energy consumption information provided by the EIF. Predictions related to energy related information could provide a trend for the near future and can assist policy decisions to improve the energy efficiency and energy saving.

In clause 6.1, the percentile of the consumption of the UE at the granularity of the estimate is proposed as the Event ID of the EIF. The percentile of the consumption of the UE is not only related with the data volume information of the UE at the related granularity, but also related with other information which may impact the energy consumption information at UE level, e.g., the radio energy consumption related information. So it is beneficial for the EIF to collect the information which may impact the energy consumption information at UE level and provide the estimates related with the energy consumption information.

Observation 3: The energy related analytics help to make the policy control for the UE. Some information related with this aspect could be studied in release 20 5GA EnergySys work or 6G related work.

2. Proposal

Based on the above observations in Key Issue #1 in release 20 FS_EnergySys_Ph2, this contribution proposes to consider the following three proposals:

- Proposal 1: It is proposed to consider the impact of the energy consumed for the basic operation of the node, e.g., when the node is in idle/inactive state or in notEnergySaving state without handling any traffic, to improve the accuracy of energy consumption at UE level in release 20 5GA EnergySys work.

- Proposal 2: Radio energy consumption level related information helps to improve the energy consumption calculation formulas based on data volume in EIF. This aspect could be further studied and investigated in 6G related work.

- Proposal 3: The energy related analytics help to make the policy control for the UE. Some information related with this aspect could be studied in release 20 5GA EnergySys work or 6G related work.