SA WG2 Meeting #169	S2-2505027

19 - 23 May, 2025, Fukuoka, Japan

Source: 	Xiaomi

Title: 	Discussion on AIML Positioning Case 3a

Document for: 	Discussion/Approval

Agenda Item: 	19.50

Work Item / Release:	NR_AIML_air-Core/Rel-19

Abstract of the contribution: Discusses the RAN3's proposal to retrieve positioning data at the NG-RAN and work plan in SA2 to fulfill the related requirements for AI/ML positioning Case 3a.

1	Introduction

In Rel-19 NR_AIML_air-Core, AI/ML positioning is being discussed including Case 1, Case 2b and Case 3b for direct AI/ML positioning, and Case 2a and Case 3a for AI/ML assisted positioning. And it is agreed have normative work on Case 2a and Case 2b in Rel-19.

In Rel-19 AIML_CN, the work on WT#1 For key issue #1 (Enhancements to LCS to support Direct AI/ML based Positioning) covered AI/ML positioning Case 3b.

In [1], RAN3 provided some potential solutions to retrieve positioning data at the NG-RAN as part of the work on Case 3a for AI/ML positioning in the WI NR_AIML_air-Core. These solutions include Solution 1: Positioning Information collection for UEs under positioning and Solution 2: Positioning Information collection for UEs served by the NG-RAN which are not being positioned. Before make the decision on the solutions, RAN3 needs feedback from SA2 on Solution 1, and the confirmation from SA2 regarding the feasibility on Solution 2.

In this paper, we aim to discuss the following:

- impact of Solution 1 provided by RAN3 from the SA2 perspective;

- feasibility of Solution 2 provided by RAN3 from the SA2 perspective;

- work plan in SA2 to fulfill the related requirements for AI/ML positioning Case 3a.

2	Discussion

## 2.1	Background Summary

In Rel-19 NR_AIML_air-Core, AI/ML positioning is being discussed with the following cases:

- Direct AI/ML positioning:

  - Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

  - Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

  - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

- AI/ML assisted positioning:

  - Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

  - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning.

And it is agreed have normative work on Case 2a and Case 2b in Rel-19.

In Rel-19 AIML_CN, the SA2 work on WT#1 For key issue #1 (Enhancements to LCS to support Direct AI/ML based Positioning) covered AI/ML positioning Case 3b supporting the following:

- The LMF is enhanced to perform location calculation based on ML model, and the trigger for data collection and for model training in LMF is up to implementation

- The MTLF and the LMF are enhanced to perform ML model training for AI/ML based positioning

- The related procedures for data collection

The following table give the summary of the AI/ML positioning within Rel-19.

| Rel-19 AI/ML positioning | RANs Impact | SA2 Impact |
| --- | --- | --- |
| Case 1 | Yes, NR_AIML_air-Core | TBD |
| Case 2a | Removed | NA |
| Case 2b | Removed | Removed |
| Case 3a | Yes, NR_AIML_air-Core | Yes, FFS |
| Case 3b | Yes, NR_AIML_air-Core | Yes, finished under AIML_CN WT#1 |

Observation 1:	AI/ML positioning Case 3a is out of scope of the SA2 Rel-19 WI AIML_CN, but required to be supported in align with RAN requirement within Rel-19.

For AI/ML positioning Case 3a, RAN3 agreed that LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. In [1], RAN3 provided some potential solutions to retrieve positioning data at the NG-RAN. These solutions include Solution 1: Positioning Information collection for UEs under positioning and Solution 2: Positioning Information collection for UEs served by the NG-RAN which are not being positioned. In [2], RAN1 agreed on the Part B label can include timing information and the LOS/NLOS indicator.

Agreement For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes timing information, support the following for providing label and quality indicator of label, - the corresponding label is provided by LMF to gNB in the format of timing information. “Timing Measurement Quality” in 38.455 can serve as quality indicator of the label. Note: It is assumed that user data privacy of non-PRU UE is preserved. Conclusion For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes the LOS/NLOS indicator, - the corresponding label is provided from LMF to the gNB by using the existing IE LOS-NLOS-Indicator-r17. There is no need of a separate field for quality indicator of LOS/NLOS indicator.

Observation 2:	The Part B label in AI/ML based positioning Case 3a include timing information and the LOS/NLOS indicator, not the UE location.

## 2.2	Impact analysis on RAN3 Solution 1: Positioning Information collection for UEs under positioning

1) Solution 1: Positioning Information collection for UEs under positioning provided by RAN3

The following solution 1 is agreed in RAN3 and captured in [3].

1. The LMF sends the NRPPa MEASUREMENT REQUEST message to one or more gNBs according to positioning procedures. 2. The gNB(s) determines that data collection is needed for the UE being positioned. 3. The gNB(s) sends the NRPPa MEASUREMENT RESPONSE message to the LMF and indicates that data collection is needed for the UE being positioned. Note: Steps 1 to 3 may occur while the LMF performs one or more of the positioning procedures described in clauses 6.11.1, 6.11.2 and 6.11.3 of TS 23.273 [35]. 4. The LMF sends the (new) NRPPa POSITIONING DATA COLLECTION REPORT message to the gNB(s) which indicated that positioning data collection is needed for the UE being positioned. The message includes information related to UE location and correlation information that enables the gNB to correlate the information related to UE location with information related to UL measurements (e.g., LMF Measurement ID and RAN Measurement ID).

2) Impact analysis

In general, solution 1 is based on existing UE positioning procedure using the existing LMF and NG-RAN interaction to collect measurement data from NG-RAN. So there are only limited UEs which are using NG-RAN to collect positioning measurement data, and the UEs under positioning with UE collecting measurement data are not involved. From SA2 perspective, detailed LMF enhancement is required from SA2, for example:

- To support step 3, the LMF enhancement is required to support the data collection indication.

- To support step 4, the LMF enhancement is required to retrieve the Part B label, and to send the related information to corresponding gNB(s).

| Solution 1 | SA2 impacts |
| --- | --- |
| General | LMF enhancement required to retrieve the Part B label, and to send the related information to corresponding gNB(s). Note: only limited UEs which are using NG-RAN to collect positioning measurement data can be involved. |
| Step 1 | No, existing positioning logic |
| Step 2 | No, not involved |
| Step 3 | LMF enhancement is required to retrieve the Part B label, including timing information and the LOS/NLOS indicator derived from UE location. NRPPa enhancement is within RAN3 scope. |
| Step 4 | LMF enhancement to send the Part B label and related correlation information to corresponding gNB. NRPPa enhancement is within RAN3 scope. |

Observation 3:	With regards to Solution 1 for Positioning Information collection for UEs under positioning and using NG-RAN to collect positioning measurement data, there are SA2 impacts requiring LMF enhancement.

Proposal 1:	SA2 reply to RAN3 to inform the SA2 impact for solution 1, and propose change request to implement to corresponding impact.

## 2.3	Feasibility analysis on RAN3 Solution 2: Positioning Information collection for UEs served by the NG-RAN which are not being positioned

1) Solution 2: Positioning Information collection for UEs served by the NG-RAN which are not being positioned provided by RAN3

- The gNB wishes to receive positioning information, consisting of labels (e.g. UE location information) to form a data set for training. Based on gNB implementation, the gNB decides when/if to trigger a (new) NG: POSITIONING INFORMATION REQUEST message to the serving AMF of the served UE(s). - The POSITIONING INFORMATION REQUEST message is an NGAP non-UE associated message containing e.g. the NGAP UE IDs of the UEs for which positioning information is required or other criteria. - The AMF receives the request message from the NG-RAN node. If the requested information is already available at AMF as part of UE positioning context, the AMF responds directly with the message in step 6. Otherwise, it selects an LMF to request positioning of each UE for Positioning Information collection purposes. The AMF triggers a Positioning Information Request towards the LMF (it could consist of the (legacy) Nlmf_Location_DetermineLocation service operation) to request the current location of the UE. - The LMF performs one or more of the positioning procedures described in clause 6.11.1, 6.11.2 and 6.11.3 of TS 23.273. - The LMF returns the requested information towards the AMF. The message Nlmf_Location_DetermineLocation Response might be reused. - The AMF returns the requested label(s) to the NG-RAN node in a POSITIONING INFORMATION RESPONSE message. The AMF may return instead a failure message with appropriate cause value if the procedure does not succeed. - The gNB combines the received label with the TRP measurements to form the data needed.

2) Feasibility analysis

In general, solution 2 is to support Positioning Information collection triggered by NG-RAN per node (or per cell), and the AMF determines the involved UEs, collects related data for each UE from LMF, and combines the related data for the UEs.

However, in Rel-19 AIML_CN work, the procedures for data collection for AI/ML positioning Case 3b is defined clause 6.22.3 in 3GPP TS 23.273, where, the LMF determines the involved UEs with user consent check with UDM. Furthermore, from system design point of view, decoupling various functions should be followed to facilitate network function deployment. It is a better choice not to deploy location-related functions in the AMF.

So, from SA2 perspective,

- the existing functionalities to have LMF determine the involved UEs with user consent check with UDM is preferred to be reused;

- the system design principle of decoupling various functions should be followed in order that the AMF is suggested not to deploy further location-related functions.

| Solution 2 | Feasibility | SA2 impacts |
| --- | --- | --- |
| General | Yes with some pros & cons. | Impacting AMF and LMF: AMF enhancement to UE determination and Positioning Information combination, and LMF enhancement to collect the new positioning information. Pros: - part of UE positioning interaction between AMF and LMF can be reused, but enhancement still needed to request the new positioning information Cons: - AMF functionality extension to support determination the involved UEs with user consent check with UDM, however similar functionality is already defined in LMF - AMF functionality extension to location-related handling to combine the related data for the UEs, and positioning information collection failure handling. Questions for clarification: - Whether the corresponding part A (measurement data) collection is triggered by LMF in the same procedure |
| Step 1 | Yes. | No, SA2 not involved, RAN3 scope |
| Step 2-3 | Yes with some pros & cons. | Impacting AMF and LMF: AMF enhancement to UE determination and Positioning Information combination, and LMF Location service enhancement to support requesting the new positioning information. Pros: - part of UE positioning interaction between AMF and LMF can be reused, but enhancement still needed to request the new positioning information Cons: - AMF functionality extension to support determination the involved UEs with user consent check with UDM, however similar functionality is already defined in LMF |
| Step 4 | Yes with LMF enhancement | Impacting LMF: enhancement on LMF implementation to retrieve the new type of positioning information. |
| Step 5 | Yes with LMF enhancement. | Impacting LMF: LMF Location service enhancement to send the Part B label and related correlation information to AMF |
| Step 6 | Yes with AMF enhancement and some cons. | Impacting AMF: AMF functionality extension to location-related handling to combine the related data for the UEs, and to support positioning information collection failure handling. Cons: AMF functionality extension to location-related handling is not aligned with the function decoupling principle for the AMF functionalities and LMF functionalities. |
| Step 7 | Yes. | No, SA2 not involved, RAN3 scope |

Observation 4:	Positioning Information collection triggered by NG-RAN per node (or per cell) is principally feasible, but the solution 2 have some cons for the AMF enhancement including the redundant functionality to determine involved UEs comparing with that defined in LMF, and the AMF enhancement with location-related handling is not aligned with the function decoupling principle for the AMF functionalities and LMF functionalities.

Proposal 2:	SA2 reply to RAN3 on solution 2 to confirm that it is feasible for Positioning Information collection triggered by NG-RAN per node (or per cell) with informing the pros and cons.

## 2.3	Work plan in SA2 to fulfill the related requirements for AI/ML positioning Case 3a

Based on the following observations gotten above:

Observation 1:	AI/ML positioning Case 3a is out of scope of the SA2 Rel-19 WI AIML_CN, but required to be supported in align with RAN requirement within Rel-19.

Observation 3:	With regards to Solution 1 for Positioning Information collection for UEs under positioning and using NG-RAN to collect positioning measurement data, there are SA2 impact requiring LMF enhancement.

Observation 4:	Positioning Information collection triggered by NG-RAN per node (or per cell) is principally feasible, but the solution 2 have some cons for the AMF enhancement including the redundant functionality to determine involved UEs comparing with that defined in LMF, and the AMF enhancement with location-related handling is not aligned with the function decoupling principle for the AMF functionalities and LMF functionalities.

It can be told that there are SA2 impacts on related procedures and AMF/LMF functionalities to meet the related requirements for AI/ML positioning Case 3a. To cover all relevant SA2 impacts, either one of the follow ways can be followed based on the group's decision:

- Using a new TEI19 work item to cover all SA2 impacts of Rel-19 NR_AIML_air-Core normative work including the AI/ML positioning Case 3a and potential any others.

- Using the WI codes of TEI19 and NR_AIML_air-Core to cover all SA2 impacts of Rel-19 NR_AIML_air-Core normative work including that for AI/ML positioning Case 3a and any potential others.

However, Considering the Rel-19 timeline, it is proposed to use the WI codes of TEI19 and NR_AIML_air-Core to cover all SA2 impacts of Rel-19 NR_AIML_air-Core normative work including that for AI/ML positioning Case 3a and any potential others.

Proposal 3:	It is proposed to use the WI codes of TEI19 and NR_AIML_air-Core to cover all SA2 impacts of Rel-19 NR_AIML_air-Core normative work including that for AI/ML positioning Case 3a and any potential others.

3	Conclusion

Observation 1:	AI/ML positioning Case 3a is out of scope of the SA2 Rel-19 WI AIML_CN, but required to be supported in align with RAN requirement within Rel-19.

Observation 2:	The Part B label in AI/ML based positioning Case 3a include timing information and the LOS/NLOS indicator, not the UE location.

Observation 3:	With regards to Solution 1 for Positioning Information collection for UEs under positioning and using NG-RAN to collect positioning measurement data, there are SA2 impact requiring LMF enhancement.

Observation 4:	Positioning Information collection triggered by NG-RAN per node (or per cell) is principally feasible, but the solution 2 has unexpected impacts on the AMF functionality including the redundant functionality to determine involved UEs comparing with that defined in LMF, and the location-related handling required for AMF that is not aligned with the function decoupling principle for the AMF functionalities and LMF functionalities.

Proposal 1:	SA2 reply to RAN3 to inform the SA2 impact for solution 1, and propose change request to implement to corresponding impact.

Proposal 2:	SA2 reply to RAN3 on solution 2 to confirm that it is feasible for Positioning Information collection triggered by NG-RAN per node (or per cell) with informing the cons on the AMF enhancement.

Proposal 3:	It is proposed to use the WI codes of TEI19 and NR_AIML_air-Core to cover all SA2 impacts of Rel-19 NR_AIML_air-Core normative work including that for AI/ML positioning Case 3a and any potential others.

The LS reply related to proposal 1 and proposal 2 is captured in S2-2505028, the change request to implement the corresponding impact for solution 1 is captured in S2-2505029 using use the WI codes of TEI19 and NR_AIML_air-Core as suggested in proposal 2 and proposal 3.

4	Reference

[1]  S2-2504525 (R3-252542), LS on AI/ML Positioning Case 3a, RAN3

[2]  Draft Report of 3GPP TSG RAN WG1 #120bis v0.1.0, https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_120b/Report/Draft_Minutes_report_RAN1%23120b_v010.zip, RAN1

[3]  R3-252486, (TP to TS 38.305) Model training at gNB for Case 3a, RAN3