3GPP TSG-SA WG2 Meeting #169	S2-2505033

# Fukuoka, Japan, May 19 - 23, 2025											 (revision of S2-2503580)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1544 | rev | 3 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding more event filters for ADC media type |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The event filters for IMS DC exposure events includes ADC media type. Application binding information is the only filter for event exposure for ADC media type. ADC can be established by local UE or remote UE, and three types of ADC can be established: P2P, P2A, P2A2P. Fine granularity for event exposure of ADC media type is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Refine the granularity of event filters for ADC media type. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Any type of ADC originating and no matter who originating an ADC will trigger the event exposure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AD.2.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change---

### AD.2.5.2	Event Monitoring

Table AD.2.5.3-1 enumerates the IMS DC exposure events and their detection criteria:

Table AD.2.5.3-1 List of events for monitoring capability

| Event | Description and Detection criteria | Which NF detects the event |
| --- | --- | --- |
| Bootstrap Data Channel established | Detected when a Bootstrap data channel is established. Network detects that a Bootstrap data channel is established when the SDP for an established IMS DC includes the media for a Bootstrap data channel with the corresponding Bootstrap DC stream ID according to TS 26.114 [76]. | IMS AS |
| Bootstrap Data Channel Release | Detected when an established Bootstrap data channel is released. Network detects that a Bootstrap data channel is released when an IMS DC that includes an SDP with the media for a Bootstrap data channel with the corresponding Bootstrap DC stream ID according to TS 26.114 [76] is released, or an IMS DC is updated to remove the media for a Bootstrap data channel with the corresponding Bootstrap DC stream ID according to TS 26.114 [76]. | IMS AS |
| Application Data Channel established | Detected when an Application Data Channel is established. Network detects that an Application data channel is established when the SDP for an established IMS DC includes the media for an Application data channel with the corresponding Application DC stream ID according to TS 26.114 [76]. The Application DC stream ID can be explicit or any stream ID. | IMS AS |
| Application Data Channel Release | Detected when an Application data channel is released. Network detects that an Application data channel is released when an IMS DC that includes an SDP with the media for an Application data channel with the corresponding Application DC stream ID according to TS 26.114 [76] is released, or an IMS DC is updated to remove the media for an Application data channel with the corresponding Application DC stream ID according to TS 26.114 [76]. | IMS AS |
| Application Data Channel requires interworking is established | Detected when an Application Data Channel requires interworking is in the process of establishment. Network detects that an P2A Application data channel is in process of establishment and the MDC2 resources are assigned for the Application data channel. | IMS AS |

Event Filters are used to specify the conditions to match for notifying the events (i.e. "List of Parameter values to match"). If there are no conditions to match for a specific Event ID, then the Event Filter is not provided. The following list provides an example how the conditions to match for event reporting can be specified for various Event IDs for IMS DC exposure events:

-	Calling ID: Identifies the public identity of calling subscriber in the targeted IMS session if present.

-	Called ID: Identifies the public identity of called subscriber in the targeted IMS session if present.

-	Media Information Set: If present, includes a set of media information indicating how the media used in the created targeted session for the selected event. Each media information contains:

-	Media Type: It indicates the media type used in the created session, e.g. Data Channel.

-	Media Parameter: It indicates the related media parameters. Different media, includes different parameters.

-	When the Media Type is Data Channel, it indicates the Data Channel Type (ADC or BDC).

-	When the Data Channel Type is BDC, it indicates local or remote, for local UE or for remote UE.

-	When the media type is ADC, it indicates the originator of the ADC (local or remote), ADC type (P2A, P2P, P2A2P) and the Data Channel application binding information.

-	Session ID: Indicates the targeted Session ID if present.

## ---End of the Change---